3GPP TSG RAN4 Meeting #81	R4-1610770

Reno, USA, 14 – 18 November, 2016

Title: 	MSD for 2+66 2UL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.6.2

Document for:	Approval

# 1	Introduction

This contribution proposes test points for 2+66 UL CA

# 2	Discussion

This CA combination has IMD3 falling on top of own B2 RX and IMD5 falling on top of own B66 RX. The test points are proposed:

| DL CA | UL CA | IMD |  | UL Fc (MHz) | DL Fc (MHz) | UL BW (MHz) | UL RB # | DL Band | DL Fc (MHz) | IMD center [MHz] | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1855 | 1935 | 5 | 25 | B2 | 1935.0 | 1935.0 | TBD |
|  | B66 |  |  | 1775 | 2175 | 5 | 25 |  |  |  | N/A |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1883.3 | 1963.3 | 5 | 25 | B66 | 2150.0 | 2149.9 | N/A |
|  | B66 |  |  | 1750 | 2150 | 5 | 25 |  |  |  | TBD |

# 3	Conclusion

The following test points shall be used for B2+B66 UL CA

| DL CA | UL CA | IMD |  | UL Fc (MHz) | DL Fc (MHz) | UL BW (MHz) | UL RB # | DL Band | DL Fc (MHz) | IMD center [MHz] | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B2+B66 | B2 | IMD3 | 2*fB2-fB66 | 1855 | 1935 | 5 | 25 | B2 | 1935.0 | 1935.0 | TBD |
|  | B66 |  |  | 1775 | 2175 | 5 | 25 |  |  |  | N/A |
| B2+B66 | B2 | IMD5 | 3*fB2-2*fB66 | 1883.3 | 1963.3 | 5 | 25 | B66 | 2150.0 | 2149.9 | N/A |
|  | B66 |  |  | 1750 | 2150 | 5 | 25 |  |  |  | TBD |

# 4	References