3GPP TSG-RAN WG4 Meeting #81	R4-1610835

# Reno, Nevada, USA, 14 - 18 November 2016

Agenda Item:	8.18.2

Title: 	RAN4#81 Evening Adhoc Meeting report

Document for:	Information

## 1	Approval of agenda

OK

## 2	UE TX requirements

### 2.1	Maximum output power and tolerance

23 dBm +2/-3 or 23 dBm +2/-3.5 dB for CA configurations (clause 6.2.2A)

Discussion:

Nokia: we propose to discuss the MPR first

Chair: only the lower tolerance affected?

Nokia: yes

Skyworks: is it interlaved waveformed only for MPR?

Nokia: some waveforms for which MPR is less than 2 dB then relevant

[discussion under agenda 2.2]

Chair: +2/-2 (single CC) since MPR is higher?

Skyworks: we support

Huawei: consistent with other CA

Decision:

23 dBm +2/-3 dB for the CA configurations

### 2.2	MPR with NS_01 (compliance with ACLR1 and general SEM)

Nokia:

| Modulation | MPR (dB) |
| --- | --- |
| QPSK | ≤ 2 |
| 16 QAM | ≤ 2.5 |
| 64 QAM | ≤ 2.5 |
| 256 QAM | TBD |

Ericsson:

2 dB for all modulation formats

Skyworks:

| Modulation | RB allocation |  |
| --- | --- | --- |
|  | 10, 20, 30, 40, 50, 60, 80, 90RB | 100RB |
| QPSK | ≤ 1dB | 0dB |
| 16QAM | ≤ 1dB | ≤ 1dB |
| 64QAM | ≤ 1dB | ≤ 2dB |
| 256QAM | ≤ 3.5dB | ≤ 3.5dB |

Mediatek:

Observation 2: 3 dB MPR could be defined for eLAA interlaced waveforms in Table I to meet 30 dB E-UTRA ACLR

Alignment of results

Discussion:

Qualcomm: the table above is an interpretation of the results in the contributions

Skyworks: is this only interlaced waveforms?

Chair: one value for all waveforms/modulation formats?

Skyworks: we agree if all partial allocations

Skyworks: one number for full allocation, and one for partial (per modulation)

Qualcomm: what is the practical value?

Skyworks: it was a simplification to group all the partial allocations

Nokia: let’s first discuss the number for partial allocation

Skyworks: QPSK and 16QAM the PAPR is quite high so not so much difference, we have also also made measurements on a wifi PA

Qualcomm: 5 GHz proptype LTE PA also a 3.5 Ghz PA

Nokia: 2.5 dB for QPSK

Qualcomm: our measurements showed 3 dB, is this acceptable?

Qualcomm: this will not preclude using a wifi PA

Nokia: 2.5 dB in brackets makes sense

Skyworks: one reason why we are not ready to accept high MPR since further relaxed by the tolerance

Qualcomm: if we are limited by 10 dBm/MHz?

Skyworks: only relevant for 10 RB allocation

Nokia: 3 dB for 16QAM in brackets?

Skyworks: why 1 dB difference between 16 and 64 from Qualcomm?

Qualcomm: 64 was dominated by EVM, EVM was not checked against

Skyworks: we are ready to admit we tested against 5% EVM

Qualcomm: EVM was dominating at 64QAM

Nokia: not sure we used EVM, we can accept higher value

Chair: [3.5 dB] for 64QAM

Skyworks: 0.5 dB could work

Qualcomm: 3, 3 and 4 dB for QPSK, 16QAM and 64QAM

Decision:

one number for full allocation, and one for partial allocation (per modulation)

Partial allocation: [2.5-3] for QPSK, [3] for 16QAM, [4] dB for 64QAM

### 2.3	NS values and associated requirements

Frequency ranges to be considered for the first version of the specification: 5150-5350 MHz and 5470-5725 MHz or larger?

Discussion:

Skyworks: how to separate the PSD requirements and channel edge requirements? Agree for the PSD limited cases

Chair: will be differentiated in the requirements and the A-MPR tables by the carrier frequency

Qualcomm: we do not like to see UNII-3 excluded

Decision:

Consider A-MPR and additional requirements for UNII-3 in addition to 5150-5350 MHz and 5470-5725 MHz

### 2.3.1	Europe (Region 1)

TX PSD 10 dBm/MHz, ETSI mask, -30 dBm/MHz immediately outside the ranges 5150-5350 MHz and 5470-5725 MHz

Discussion:

Chair: any requirements missing for Region 1?

Qualcomm: for Region 1, is the ETSI mask clear?

Skyworks: in our case we measured with IEEE mask, it’s more stringent than the ETSI mask

Ericsson: we considered the mask in accordance with EN 301 893

Qualcomm: same as that agreed in the way forward?

Decision:

--

### 2.3.2	Japan

TX PSD 10 dBm/MHz, ACLR2 = 40 dBc but which SEM to applies (occupied bandwidth)?

Table 2.2-1: 18 < occupied bandwidth < [19.7]

| Centre Frequency [MHz] | Protected range [MHz] | Frequency difference f between centre frequency – the edges of pretected frequency range(MHz) | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 5180 | 5135 ≤ f ≤ 5142 | ≥ 38 and ≤ 45 | -26 | 1 MHz |
|  | 5142 < f ≤ 5150 | ≥ 30 and < 38 | -18 |  |
| 5240 | 5250 ≤ f < 5251 | ≥ 10 and < 11 | 10(10 - f) |  |
|  | 5251 ≤ f < 5260 | ≥ 11 and < 20 | -10 – 8/9(f – 11) |  |
|  | 5260 ≤ f < 5266.7 | ≥ 20 and < 26.7 | -18 – 1.2(f – 20) |  |
|  | 5266.7 ≤ f ≤ 5365 | ≥ 26.7 and ≤ 125 | -26 |  |
| 5260 | 5135 ≤ f ≤ 5233.3 | ≥ 26.7 and ≤ 125 | -26 |  |
|  | 5233.3 < f ≤ 5240 | ≥ 20 and < 26.7 | -18 – 1.2(f – 20) |  |
|  | 5240 < f ≤ 5249 | ≥ 11 and < 20 | -10 – 8/9(f – 11) |  |
|  | 5249 < f ≤ 5250 | ≥ 10 and < 11 | 10(10 - f) |  |
| 5320 | 5350 ≤ f ≤ 5365 | ≥ 30 and ≤ 45 | -26 |  |

or

| Centre Frequency Fc [MHz] | Protected range [MHz] | Frequency difference f between centre frequency – 5240 (for Fc=5180, 5200, 5220, 5240) 5260 (for Fc=5260, 5280, 5300, 5320) (MHz) | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 5180, 5200, 5220, 5240 | 5135 ≤ f ≤ 5142 | - | -26 | 1 MHz |
|  | 5142 < f ≤ 5150 | - | -18 |  |
|  | 5250 ≤ f < 5251 | ≥ 10 and < 11 | 10(10 - f) |  |
|  | 5251 ≤ f < 5260 | ≥ 11 and < 20 | -10 – 8/9(f – 11) |  |
|  | 5260 ≤ f < 5266.7 | ≥ 20 and < 26.7 | -18 – 1.2(f – 20) |  |
|  | 5266.7 ≤ f ≤ 5365 | - | -26 |  |
| 5260, 5280, 5300, 5320 | 5135 ≤ f ≤ 5233.3 | - | -26 |  |
|  | 5233.3 < f ≤ 5240 | ≥ 20 and < 26.7 | -18 – 1.2(f – 20) |  |
|  | 5240 < f ≤ 5249 | ≥ 11 and < 20 | -10 – 8/9(f – 11) |  |
|  | 5249 < f ≤ 5250 | ≥ 10 and < 11 | 10(10 - f) |  |
|  | 5350 ≤ f ≤ 5365 | - | -26 |  |
| 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5700 | 5460 < f ≤ 5470 | - | -19 |  |
|  | 5725 < f ≤ 5740 | - | -19 |  |

Discussion:

DCM: the bigger table should be used according to Japanese regulation, we propose to use occupied bandwidth < 20 MHz

Skyworks: the ETSI mask will be more stringent?

Nokia: why can’t we use that table?

Nokia: we don’t see any issue

Skyworks: the masks are less than 20 MHz

DCM: this table defines only center frequency. We propose to use the table, on top of that we specify occupied BW less than 20 MHz

Nokia: other companies not ready to accept that

Decision:

Not use an occupied bandwidth less than 20 MHz for eLAA

### 2.3.3	US

TX PSD 11 dBm/MHz

Table 2.1.3-1: Additional requirements for E-UTRA channels assigned within 5150-5350 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 MHz |  |
| 4500 ≤ f ≤ 5150 | -41 | 1 MHz |
| 5350 < f ≤ 5460 | -41 |  |
| 5460 ≤ f < 5470 | -27 |  |

Table 2.1.3-2: Additional requirements for E-UTRA channels assigned within 5470-5725 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 MHz |  |
| 4500 ≤ f ≤ 5150 | -41 | 1 MHz |
| 5350 < f ≤ 5460 | -41 |  |
| 5460 ≤ f < 5470 | -27 |  |
| 5745 ≤ f | -27 |  |

Discussion:

Chair: add table for UNII-3

Decision:

Add UNII-3 emissions requirements

### 2.3.4	Resolution 229

TX PSD: 0.25 mW/25kHz in 5150-5250 MHz

Discussion:

Qualcomm: we did not receive any input from any region that this is applicable

Skyworks: we would like to understand which region this is applicable

Ericsson: this limit is in accordance with the global resolution 229 for 5150-5250 MHz and the specification should contain the means for meeting this requirement in case an administration choose to adopt it

Decision:

No decision

### 2.4	A-MPR for additonal spurious emissions

### 2.4.1	Europe (Region 1)

Nokia:

A-MPR for ETSI and Japanese requirements: 1.5 dB for QPSK and 1.0 dB for 16-QAM and 64-QAM.

Ericsson:

From figures 1-3 it can be seem that an A-MPR of

A-MPR = 2{1 – (L-1)/3} dB, for 1 ≤ L ≤ 4 and QPSK modulation

A-MPR = 2.5{1 – (L-1)/4} dB, for 1 ≤ L ≤ 5 and 16QAM/64QAM modulation

would be sufficient for compliance with the ETSI mask assuming that MPR = 2 dB. Hence no A-MPR is required for the larger allocations.

The -30 dBm/MHz unwanted emissions requirent requires larger A-MPR for the channels at the band edges of the sub-bands 5150-5350 MHz and 5470-5725 MHz (center frequencies 5160 MHz, 5340 MHz and 5480 MHz but not for 5700 MHz):

A-MPR = 10 – 0.1(L-1) – MPR dB, for 1 ≤ L ≤ 10 and all modulations

Skyworks

Proposal 4: following A-MPR table is proposed for Japan and region 1 assuming MPR from proposal 3

| modulation | 10-60RB contiguous interleaved waveforms | 80 and 90RB contiguous interleaved waveforms | 20, 40, 60 and 80RB non-contiguous interleaved waveforms | 100RB fully allocated waveform |
| --- | --- | --- | --- | --- |
| QPSK, 16QAM, 64QAM | ≤ 2dB | ≤ 1dB | ≤ 1dB | ≤ 0.5dB |
| 256QAM | 0dB | 0dB | 0dB | 0dB |

Qualcomm:

| Waveform | Modulation | Total backoff measured |
| --- | --- | --- |
| 1000000000 | QPSK | 4.8 |
| 0000000001 |  | 4.9 |
| 1111111111 |  | 3.7 |
| 1000000000 | 16QAM | 5.3 |
| 0000000001 |  | 5.5 |

Discussion:

Skyworks: the trend is related to PSD not mask, this is already accounting for the 1 dB MPR

Chair: should we differentiation between smaller and larger allocations?

Skyworks: we support that, but still to be discussed

Nokia: it makes sense to differentiate

Skyworks: depend in the modulation formats since higher MPR for 16/64QAM

Decision:

Differentiation between smaller and larger allocations, modulation dependent

### 2.4.2	Japan

Nokia:

A-MPR for ETSI and Japanese requirements: 1.5 dB for QPSK and 1.0 dB for 16-QAM and 64-QAM.

Qualcomm:

Skyworks:

Proposal 4: following A-MPR table is proposed for Japan and region 1 assuming MPR from proposal 3

| modulation | 10-60RB contiguous interleaved waveforms | 80 and 90RB contiguous interleaved waveforms | 20, 40, 60 and 80RB non-contiguous interleaved waveforms | 100RB fully allocated waveform |
| --- | --- | --- | --- | --- |
| QPSK, 16QAM, 64QAM | ≤ 2dB | ≤ 1dB | ≤ 1dB | ≤ 0.5dB |

Discussion:

Chair: same A-MPR for the Japanese and European requirements? (for the NS indicated in Japan)

Skyworks: we would like to capture that ACLR2 is not an issue and that the Japanese mask is not more stringent than the ETSI

DCM: we need occupied bandwidth less than 20 MHz, an additional requirement for the NS

Decision:

same A-MPR for the Japanese and European requirements? (for the NS indicated in Japan) recognising that ACLR2 is not an issue and that the Japanese mask is not more stringent than the ETSI

### 2.4.3	US

Nokia:

A-MPR for FCC (channels with fc between 5200 and 5300 MHz): 0.5 dB for QPSK

A-MPR for FCC (channels with fc = 5180 and 5320 MHz): 5.5 dB for QPSK , 16-QAM and 64-QAM.

Ericsson:

For the outermost channels in the sub-bands 5150-5350 MHz and 5470-5725 MHz with center frequencies 5160 MHz, 5340 MHz and 5480 MHz, the A-MPR needed for compliance with the -41 dBm/MHz unwanted emissions requirement is very large:

A-MPR =17 – 0.2(L-1) – MPR dB, for 1 ≤ L ≤ 10 and all modulation formats.

it is also likely that A-MPR would be needed the channels adjacent to these (not studied here). The above A-MPR is not feasible for the outermost channels. One alternative is to allow 10 MHz channel bandwidth (leaving a 10 MHz frequency separation) since these outermost channels are not used for Wi-Fi operations.

For the outermost channels in the sub-bands 5150-5350 MHz and 5470-5725 MHz, the A-MPR needed for compliance with the -27 dBm/MHz requirement is

A-MPR =7 – 0.2(L-1) – MPR dB, for 1 ≤ L ≤ 10 and all modulation formats.

Qualcomm:

Discussion:

Chair: specify A-MPR for the channels adjacent to the band edge where -41 dBm/MHz applies? [A-MPR of the order of 15-18 dB]

Nokia: what is the alternative?

Chair: channels not used

Nokia: A.MPR specified for all channels to meet the -27 dBm/MHz

Qualcomm: the value will depend on the channel number

Decision:

A-MPR specified for all channels in 5150-5350 and 5470-5725 MHz against the relevant requirements (-41 dBm/MHz, -27 dBm/MHz and the PSD requirements)

-- Specify A-MPR for the channels adjacent to the band edge where -41 dBm/MHz applies

### 2.5	A-MPR for TX PSD requirements (wanted channel)

### 2.5.1	Regional

+11 dBm/MHz in the US, +10 dBm/MHz in Europe and Japan

Discussion:

Skyworks: no further power reduction needed for US, but some power reduction needed for Europe

Qualcomm: we have not studied this yet

Decision:

No decision

### 2.5.2	Resolution 229 (WRC-12)

0.25mW/25kHz in 5150-5250 MHz

Ericsson:

A-MPR = 23 dBm - 10log10(NRB_alloc) - 2.5 dBm – MPR

Discussion:

Agenda item not treated

Decision:

### 2.6	Transmit signal quality

In-band emission in accordance with Ericsson (R4-1609586) or Qualcomm (R4-1610197) proposal?

Discussion:

Agenda item not treated

Decision:

### 2.7	Time masks

Proposal in R4-1609586

End of OFF power

20µs

20µs

Transient period

Transient period   tend

Start of OFF power

Start of ON power

requirement

Start Sub-frame

End sub-frame

End of ON power

requirement

* The OFF power requirements does not

apply for DTX and measurement gaps

tD

Figure 6.3.4.1-1A: General ON/OFF time mask for Frame Structure Type 3

Modify as follows not to extend idle periods by ramping time: shift the trailing transient period of UE1

Time templates: not relevant with CCA in between subsequent subframes for the same UE:

20µs

20µs

20µs

20µs

Transient period

Start of N

+1

power

requirement

End of N

+1

power

requirement

N

+1

Sub-frame

Slot

i

Slot

i+1

N

0

Sub-frame

N

+2

Sub-frame

Figure 6.3.4.3-1A: Transmission power template for Frame Structure Type 3

Discussion:

Huawei: we are reviewing this and will return, do you need to do this in between subframes?

Chair: no, only at the start and end of transmission for a given UE

Nokia:is  it possible to schedule CCA in between subsequent subframes from the same UE

Decision:

### 2.8	Band combinations to be included in the first version

Discussion:

Agenda item not treated

Decision:

## ------------------------------------------------------------------------------------------

## Background documents for agenda (not to be treated)

From RF Chairman’s report Monday evening.

#### 8.18.2	UE RF (36.101) [LTE_eLAA-Core]

R4-1609586	Introduction of UE transmitter requirements for LAA Scell operation in Band 46
					36.101	  CR-4032  rev  (Rel-14) v14.1.0
					Source: Ericsson

Abstract:

CR to introduce transmitter characteristics for uplink operations in Band 46 (clause 6 except the actual E-UTRA CA configurations addressed in basket CRs)

Discussion:

Media Tek: On section 6.5.2, wondering if the same wording can be reused for eLAA?

QC: On using band 1 as example, we had agreements on the example bands. There is no IMD and MSD requirements in  this CR. For supurious emission, there is another way to capture the agreements. For in-band emission, we have different proposals.

Ericsson: We can improve the wording on section 6.5.2. The intension is to use this CR as a staring point. We can further discuss the in-band emission requirements. Other example bands can be handled in basket CRs. Band 1 and Band 46 is used for example for tolerance requirements.

Decision: 		The document was Revised in R4-1610735

R4-1610735	Introduction of UE transmitter requirements for LAA Scell operation in Band 46
					36.101	  CR-4032  rev  (Rel-14) v14.1.0
					Source: Ericsson

Abstract:

CR to introduce transmitter characteristics for uplink operations in Band 46 (clause 6 except the actual E-UTRA CA configurations addressed in basket CRs)

Discussion:

Decision: 		The document was Return to.

[…]

##### 8.18.2.1	MPR [LTE_eLAA-Core]

R4-1610421	eLAA UE MPR
					36.101	  CR-  rev  (Rel-14) v14.1.0
					Source: MediaTek Inc.

Abstract:

In this contribution, we evaluate the MPR for eLAA interlaced waveforms to meet E-UTRA ACLR 30 dB requirement with a 5 GHz WiFi PA

Observation 1: The saturated power of 5 GHz WiFi PA needs to be improved for power class 3 eLAA application

Observation 2: 3 dB MPR could be defined for eLAA interlaced waveforms in Table I to meet 30 dB E-UTRA ACLR

Observation 3: For Japan 40 dB E-UTRA ACLR2 requirement, no A-MPR is required.

Discussion:

QC: for MPR requirements, EVM shall be considered. 1dB offset shall be considered on top of results.

Skyworks: On observation 1, it depends on implemantion of PA.

MTK: We confirm the results does not consider EVM. No contiuous allocation (No.2) is not valid one.

Decision: 		The document was Noted

R4-1609235	eLAA MPR and A-MPR requirements based on 5GHz WiFi PA measurements
						  CR-  rev  (Rel-14) v
					Source: Skyworks Solutions Inc.

Abstract:

This contribution proposes MPR and A-MPR values for eLAA based on RAN4#80bis way forwards R4-168820 [1] based on a set of LTE, eLAA and WiFi measurement taken from a standard 5GHz WiFi PA.

Proposal 1: 0dB MPR reference is set for fully allocated RB 10MHz and 20MHz channels (respectively 50RB and 100RB allocation)

Proposal 2: band 46 power class and tolerance reuses band 47 requirements (23dBm +/-2dB)

Proposal 3: the following MPR table is proposed for 20MHz eLAA channel

| Modulation | RB allocation |  |
| --- | --- | --- |
|  | 10, 20, 30, 40, 50, 60, 80, 90RB | 100RB |
| QPSK | ≤ 1dB | 0dB |
| 16QAM | ≤ 1dB | ≤ 1dB |
| 64QAM | ≤ 1dB | ≤ 2dB |
| 256QAM | ≤ 3.5dB | ≤ 3.5dB |

Proposal 4: following A-MPR table is proposed for Japan and region 1 assuming MPR from proposal 3

| modulation | 10-60RB contiguous interleaved waveforms | 80 and 90RB contiguous interleaved waveforms | 20, 40, 60 and 80RB non-contiguous interleaved waveforms | 100RB fully allocated waveform |
| --- | --- | --- | --- | --- |
| QPSK, 16QAM, 64QAM | ≤ 2dB | ≤ 1dB | ≤ 1dB | ≤ 0.5dB |
| 256QAM | 0dB | 0dB | 0dB | 0dB |

Proposal 5: following A-MPR table is proposed for region 2 assuming MPR from proposal 3

| modulation | 10-60RB contiguous interleaved waveforms | 80 and 90RB contiguous interleaved waveforms | 20, 40, 60 and 80RB non-contiguous interleaved waveforms | 100RB fully allocated waveform |
| --- | --- | --- | --- | --- |
| QPSK, 16QAM, 64QAM | ≤ 1dB | 0dB | 0dB | 0dB |
| 256QAM | 0dB | 0dB | 0dB | 0dB |

Discussion:

Huawei: On proposal 3, for other bands, we have, e.g., 1dB MPR for QPSK for full allocation, can you clarify?

Ericsson: On MPR requirements, we normal define MPR comparing with the normal maximum power, i.e., 23dBm. For A-MPR for Europea requirements, explain more about the results of 1RB allocation case.

QC: Reference shall be 1dB instead 0dB as indicated in this paper. The pass/fail criteria of test equipment is different from the spec. For 256QAM, different understand.

Skyworks: Our paper is to use the full allocation wave form as reference. We need to discuss further. For 1RB allocation, we assume 1dB power backoff.

Decision: 		The document was Noted

R4-1609588	MPR and A-MPR for eLAA UL
						  CR-  rev  (Rel-14) v
					Source: Ericsson

Abstract:

In this contribution we discuss the specification of MPR and A-MPR for eLAA

Conclusion:

Simulations based on the way forward agreed at RAN4#80bis indicate that an MPR = 2dB independent of modulation format and allocation would suffice for compliance with the LTE mask, ACLR = 30 dBc and the EVM requirement (not considering 256QAM).

For the outermost channels of the frequency ranges 5150-5350 MHz and 5470-5725 MHz the A-MPR required for compliance with the unwanted emissions requirement in Europe and the US is substantial. The A-MPR needed for compliance with the ETSI mask is feasible.

The A-MPR needed for operations above 5725 MHz could be added as part of maintenance.

Discussion:

QC: Which EVM requirements used in the results?

Ericsson: same EVM requirements as licensed bands.

MTK: on secion 6.1 A-MPR equations will give the negative value.

Ericsson: typo, it shall be positive.

NTT DoCoMo: For Japan regulatory, measurement bandwidth is less than 20MHz. Can we test the occupied bandwidth with less than 20MHz.

Ericsson: Not sure about the occupied bandwidth definition in Japan regulatory requirements, whether it is measured bandwidth or minimum requirements.

NTT DoCoMo: it is measured bandwidth.

Ericsson: so, we need to use 18~19MHz as measurement bandwidth to decide the requirements. We need to use NS_29 even no A-MPR required for Japan requirements.

QC: any changes needed for Japan regulatory requirements?

Ericsson: Change of Japan regulatory requiremetns will have impact to both RAN1 and RAN4. It takes time to update the Japan regulotary requirements. Requirements shall be defined based on the latest regulatory requirements.

Decision: 		The document was Noted

R4-1609838	eLAA MPR study
						  CR-  rev  () v
					Source: Nokia, Alcatel-Lucent Shanghai Bell

Abstract:

This document presented PA backoff simulation results for eLAA uplink, according to the agreed way forward [1]. Based on the simulation results, the following MPR and A-MPR are proposed:

Table 1: eLAA MPR for 23 dBm Power Class

| Modulation | MPR (dB) |
| --- | --- |
| QPSK | ≤ 2 |
| 16 QAM | ≤ 2.5 |
| 64 QAM | ≤ 2.5 |
| 256 QAM | TBD |

A-MPR for ETSI and Japanese requirements: 1.5 dB for QPSK and 1.0 dB for 16-QAM and 64-QAM.

A-MPR for FCC (channels with fc between 5200 and 5300 MHz): 0.5 dB for QPSK

A-MPR for FCC (channels with fc = 5180 and 5320 MHz): 5.5 dB for QPSK , 16-QAM and 64-QAM.

Discussion:

Decision: 		The document was Noted

##### 8.18.2.2	Additional spurious emissions  [LTE_eLAA-Core]

R4-1610196	A-MPR for eLAA in Band 46
						  CR-  rev  () v
					Source: Qualcomm Incorporated

Abstract:

Simulation results for US, Europe, and Japan

Discussion:

Decision: 		The document was Noted

##### 8.18.2.3	REFSENS [LTE_eLAA-Core]

R4-1609920	Licensed band de-sense in eLAA CA combinations
						  CR-  rev  (Rel-14) v
					Source: Skyworks Solutions Inc.

Abstract:

Propasal to include licensed band spurious harmonic response and IMD2 related de-sense in eLAA CA combination studies.

Proposal 1: Spurious harmonic receiver responses shall be studied for eLAA and potential MSD specified.

Proposal 2: B42 MSD due to blocking related to B46 eLAA transmitter shall be studied.

Proposal 3: Potential licensed band MSD issues related to IMD of B46 and licensed band TX shall be studied.

Discussion:

QC: not sure how to handle these de-sense given the WI is supposed to be completed in this week

Ericsson: We recognise the problems. We can discuss these requirements in basket CA WIs.

MTK: Network can take care to avoid the de-sense of license bands.

Ericsson: prefer not to define any restriction on the scheduling

Skyworks: We cannot use the similar approach as LAA case since the issue is related to licensed bands.

Decision: 		The document was Noted

##### 8.18.2.4	Other requirements [LTE_eLAA-Core]

R4-1609589	Time masks for eLAA UL
						  CR-  rev  (Rel-14) v
					Source: Ericsson

Abstract:

In this contribution we propose time masks and power templates for eLAA

Discussion:

QC: Transient between PUCCH and PUCCH is not only for the frequency hopping. Not sure if the transient period between PUCCH can be removed.

Ericsson: we can futher discuss. We can futher discuss with RAN1 on the frequency hopping for inter-lace changes. We agree the changes on the time mask in last RAN4 meeting.

Decision: 		The document was Return to.

R4-1610198	Maximum output power for eLAA in Band 46
						  CR-  rev  () v
					Source: Qualcomm Incorporated

Abstract:

For approval.  This contribution proposes the maximum output power tolerance be defined as +2/-2.5 dB.

Discussion:

Skyworks: +2/-2 is proposed based on Band 47. Band 46 has larger BW comparing with Band 47. It is also related to the MPR reference.

Ericsson: Fine with the proposals. For MPR reference, we shall use the same approach as license bands.

Nokia: Since all the waveform have MPR, it does not matter to define -2.5dB tolerance.

Decision: 		The document was Noted

R4-1609988	B46 MOP tolerance
						  CR-  rev  () v
					Source: Huawei, Hisilicon

Abstract:

This contribution analyses B46 MOP (Maximum Output Power) tolerance according to WF [1] and proposes which value should be used.

PROPOSAL: B46 Maximum output power lowers tolerance TL,c=2.5dB shall be used and thus the tolerance is +2dB/-2.5dB

(Late contributions)

Discussion:

Decision: 		The document was Noted

R4-1610197	In-band emission requirement for eLAA
						  CR-  rev  () v
					Source: Qualcomm Incorporated

Abstract:

For approval.  This contribution provides a proposal for the IBE requirement extending the existing requirement to the interlaced waveform

Discussion:

Skyworks: we also need to capture the in-band emissison requiremetns for 256QAM.

Ericsson: the proposal does not consider the IMD between the non-continous allocated blocks.

Nokia: the requirement of the mask is not finalized.

QC: Not considered for 256QAM. No agreement for in-band emission requirement, we only agree with the tested waveform. We need to consider the requirement of mask.

Ericsson: we do not only consider the 1RB allocation. We did not see any results showing the results are tightened.

QC: we understand Ericsson proposal of mask is based on 1RB allocation. Ercisson proposal has impact to MPR requirements.

Ercisson:

Decision: 		The document was Return to.