3GPP TSG-RAN WG4 Meeting #82	R4-1700766

Athens, Greece, 13 – 17 February, 2017

Source:	Ericsson

Title:	TP to TR 36.714-04-01: CA_39C-46C and CA_39A-46D

Agenda item:	7.4.2

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.714-04-01 for adding REFSENS values for CA_39C-46C and CA_39A-46C

# 2. Text Proposal for TR 36.714-04-01

##### ---Start of changes---

## 6.116	CA_39C-46C_BCS0

### 6.116.1	Operating bands for CA

# Table 6.116.1-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_39-46 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | 46 | N/A |  |  | 5150 MHz | – | 5925 MHz | TDD |

### 6.116.2	Channel bandwidths per operating band for CA

# Table 6.116.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_39C-46C | 39 | See the CA_39C Bandwidth combination set 0 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  | 75 | 0 |
|  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

### 6.116.3	Co-existence studies

Table 6.116.3-1 summarizes frequency ranges where harmonics occur due to Band 39 or Band 46 for UE.

# Table 6.116.3-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 3760 | 3840 | 5640 | 5760 |
| 46 | N/A |  | 5150 | 5925 | N/A |  | N/A |  | 10300 | 11850 | 15450 | 17775 |

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 39 and Band 46 DL carriers can be calculated as shown in table 6.116.3-2 below:

# Table 6.116.3-2: Band 39 and Band 46 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1880 | 1920 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 3760 | 3840 | 10300 | 11850 |
| 3rd order harmonics frequency range (MHz) | 5640 | 5760 | 15450 | 17775 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency limits (MHz) | 3230 | 4045 | 7030 | 7845 |
| 3rd order IMD products | ¦f2_low - 2*f1_high¦ | ¦f2_high -2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 1310 | 2165 | 8380 | 9970 |
| 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 8910 | 9765 | 12180 | 13770 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 1105 | 2695 | 5110 | 5965 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1_low – f2_BWmax) | (f1_high + f2_BWmax) | (f2_low – f1_BWmax) | (f2_high + f1_BWmax) |
| IMD frequency limits (MHz) | 1840 | 1960 | 5115 | 5960 |

If any of the harmonic frequency range or the IMD frequency range in Table 6.116.3-1 and 6.116.3-2 are overlapping with the BS uplink operating frequency range of those band listed in Table 5.5-1 in 36.101, it is recommended that Bands 39 and 46 BS transmitters do not share the same antenna with those Bands to avoid antenna PIM issue.

### 6.116.4	ΔTIB,c and ΔRIB,c values

For the UE which supports CA_39C-46C the ΔTIB,c and ΔRIB,c  is defined for applicable bands in Table 6.116.4-1 and 6.116.4-2 respectively assuming separate antenna architecture.

# Table 6.116.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_39C-46C | 39 | 0 |

# Table 6.116.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_39C-46C | 39 | 0 |

### 6.116.5	REFSENS requirements

The REFSENS is specified in the following table:

### Table 6.116.5-1: Reference sensitivity QPSK PREFSENS

| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_39C-46C | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |

## 6.117	CA_39A-46D_BCS0

### 6.117.1	Operating bands for CA

# Table 6.117.1-1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_39-46 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | 46 | N/A |  |  | 5150 MHz | – | 5925 MHz | TDD |

### 6.117.2	Channel bandwidths per operating band for CA

# Table 6.117.2-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_39A-46D | 39 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

### 6.117.3	Co-existence studies

Table 6.117.3-1 summarizes frequency ranges where harmonics occur due to Band 39 or Band 46 for UE.

# Table 6.117.3-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 3760 | 3840 | 5640 | 5760 |
| 46 | N/A |  | 5150 | 5925 | N/A |  | N/A |  | 10300 | 11850 | 15450 | 17775 |

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 39 and Band 46 DL carriers can be calculated as shown in table 6.117.3-2 below:

# Table 6.117.3-2: Band 39 and Band 46 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1880 | 1920 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 3760 | 3840 | 10300 | 11850 |
| 3rd order harmonics frequency range (MHz) | 5640 | 5760 | 15450 | 17775 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency limits (MHz) | 3230 | 4045 | 7030 | 7845 |
| 3rd order IMD products | ¦f2_low - 2*f1_high¦ | ¦f2_high -2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 1310 | 2165 | 8380 | 9970 |
| 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 8910 | 9765 | 12180 | 13770 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 1105 | 2695 | 5110 | 5965 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1_low – f2_BWmax) | (f1_high + f2_BWmax) | (f2_low – f1_BWmax) | (f2_high + f1_BWmax) |
| IMD frequency limits (MHz) | 1820 | 1980 | 5130 | 5945 |

If any of the harmonic frequency range or the IMD frequency range in Table 6.117.3-1 and 6.117.3-2 are overlapping with the BS uplink operating frequency range of those band listed in Table 5.5-1 in 36.101, it is recommended that Bands 39 and 46 BS transmitters do not share the same antenna with those Bands to avoid antenna PIM issue.

### 6.117.4	 ΔTIB,c and ΔRIB,c values

For the UE which supports CA_39A-46D the ΔTIB,c and ΔRIB,c  is defined for applicable bands in Table 6.117.4-1 and 6.117.4-2 respectively assuming separate antenna architecture.

# Table 6.117.4-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_39A-46D | 39 | 0 |

# Table 6.117.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_39A-46D | 39 | 0 |

### 6.117.5	REFSENS requirements

The REFSENS is specified in the following table:

Table 6.117.5-1: Reference sensitivity QPSK PREFSENS

| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_39A-46D | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |

##### ---End of changes---