3GPP TSG-RAN WG4 Meeting #82                                                                R4-1700980

# Athens, Greece, 13 - 17 February, 2017

Title:	Regulatory framework in 3.5GHz frequency range for 5G NR RF requirements

Source:	CMCC

Agenda Item:	10.2

Document for:	Approval

- Introduction

In RAN4 NR ad-hoc meeting, the way forward [1] was discussed and companies are encouraged to study the possibility of harmonized band for the proposal bands. The 3.5GHz frequency range proposed in [2] and [3] is recommended to be the promising 5G NR bands.

The purpose of this contribution is to summarize the requirements relevant for RAN4 that are specified by ITU and ECC for RF equipment to be utilized for use of 900 MHz of spectrum in the 3300-4200 MHz (3.5 GHz) band. Considering that the IMT is recommended to be shared use in USA and there is no operators from Region 2 in last meeting that has interested in using this band in 5G NR. So the regulations in this paper are more focused on that in Region 1 and Region 3.

- Discussion

ECC DEC (11) 06 has been recommended by the ECC PT1 as the baseline when considering 3400-3800MHz used for 5G NR. In September 2016, ECC PT1 sent an LS to 3GPP to seek feedback on the technical parameters foreseen for introducing 5G technologies in the 3400-3800 MHz band. The TP [4] was approved in last meeting which is the corresponding study of suitability of technical conditions of ECC DEC (11) 06 for 5G. Considering the 3400-3800MHz operating band may be different from 3300-4200MHz band, the ECC DEC (11) 06 is also suggested as the baseline for the RF requirements in 3300-4200GHz band in 5G NR WI.

Proposal#1: ECC DEC (11) 06 is suggested as the baseline for the RF requirements in 3.5GHz band in 5G NR WI

In addition to CEPT, there may be no other regional regulations on this frequency band. It is suggested to consider other regulations from some country that does have adjacent services listed in ITU radio regulation (edition of 2016) when RAN4 defines the RF requirements, especially adjacent RF characters, such as SEM of base station, or output RF spectrum emissions of UE, etc.

To better understand the services in 3300-4200MHz from ITU Radio Regulation, the service information in Region 1, 2 and 3 is listed in Table 1 and Table 2. For example, there may be radiolocation services operating in 3100-3300MHz in Region 3, which may need additional power limit from the adjacent band 3300-4200MHz. So companies are encouraged to propose related regulations that will impact the requirements of RF characters.

Proposal#2: Companies are encouraged to propose related regulations that will impact the requirements of RF characters.

Table 1 2 700-3 600 MHz

| Allocation to services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |  |
| 2 700-2 900 AERONAUTICAL RADIONAVIGATION 5.337 Radiolocation 5.423 5.424 |  |  |  |  |
| 2 900-3 100 RADIOLOCATION 5.424A RADIONAVIGATION 5.426 5.425 5.427 |  |  |  |  |
| 3 100-3 300 RADIOLOCATION Earth exploration-satellite (active) Space research (active) 5.149 5.428 |  |  |  |  |
| 3 300-3 400 RADIOLOCATION |  | 3 300-3 400 RADIOLOCATION Amateur Fixed Mobile |  | 3 300-3 400 RADIOLOCATION Amateur |  |
| 5.149 5.429 5.429A 5.429B 5.430 |  | 5.149 5.429C 5.429D |  | 5.149 5.429 5.429E 5.429F |  |
| 3 400-3 600 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.430A Radiolocation |  | 3 400-3 500 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.431A 5.431B Amateur Radiolocation 5.433 5.282 |  | 3 400-3 500 FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile 5.432 5.432B Radiolocation 5.433 5.282 5.432A |  |
|  |  | 3 500-3 600 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.431B Radiolocation 5.433 |  | 3 500-3 600 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.433A Radiolocation 5.433 |  |
| 5.431 |  |  |  |  |  |

| Table 2 3 600-4 800 MHz |  |  |
| --- | --- | --- |
| Allocation to services |  |  |
| Region 1 | Region 2 | Region 3 |
| 3 600-4 200 FIXED FIXED-SATELLITE (space-to-Earth) Mobile | 3 600-3 700 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.434 Radiolocation 5.433 | 3 600-3 700 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile Radiolocation 5.435 |
|  | 3 700-4 200 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile |  |
| 4 200-4 400 AERONAUTICAL MOBILE (R) 5.436 AERONAUTICAL RADIONAVIGATION 5.438 5.437 5.439 5.440 |  |  |

- Conclusion

Proposal#1: ECC DEC (11) 06 is suggested as the baseline for the RF requirements in 3.5GHz band in 5G NR WI

Proposal#2: Companies are encouraged to propose related regulations that will impact the requirements of RF characters.

- Reference

R4-1700304, WF on NR spectra related work

R4-1700162, Dicussion on the proposal on harmonized TDD band plan in the 3.5GHz frequency range for 5G NR

R4-1700139, Potential harmonized bands for NR

R4-1700122, TP for 38.803: ECC requirements in 3400-3800MHz

ECC DEC (11) 06