3GPP TSG-RAN WG4 #82	R4-1702272

Athens, Greece, 13th – 17th February 2017

Source: 	Ericsson

Title:  	TP to 37.843: Background information on conducted RX blocking requirement

Agenda Item:	7.13.3.5

Document for:	Approval

# Introduction

This TP is a revised version of [1] following offline discussions.

[1] Ericsson, “TP to 37.843: Background information on conducted RX blocking requirement”, R4-1609937, RAN4#81

# Text Proposal

--------------------------- First section of text proposal --------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-160940	“Work Item Description for Enhancements of Base Station (BS) RF and EMC requirements for Active Antenna System (AAS)”

[3]	3GPP TS 37.105	“Active Antenna System (AAS) Base Station (BS) transmission and reception”

[4]	3GPP TR 37.842	“Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”

[x.1]	3GPP TR 36.942, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”

[x.2]	3GPP TR 37.840, “Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station”

[x.3]	3GPP TS 37.104, “E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception”

[x.4]	3GPP TR 25.951 “FDD Base Station (BS) classification”

[x.5]	3GPP TR 37.809 “Evolved Universal Terrestrial Radio Access (E-UTRA) medium range and Multi-Standard Radio (MSR) medium range / local area Base Station (BS) class requirements”

[x.6]	3GPP TR 36.931 “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) requirements for LTE Pico Node B”

--------------------------- Second section of text proposal --------------------------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

RMC		Reference measurement channel

--------------------------- Third section of text proposal --------------------------------

# 5	Radiated requirements

## 5.1	General

## 5.2	Transmitter

## 5.3	Receiver

### 5.3.x In band receiver blocking

#### 5.3.x.1 Background information on the conducted requirement

The in band receiver blocking requirement captures minimum requirements on several aspects of receiver behaviour. The conducted in band blocking requirement is based upon placing a high power interferer within the band on another carrier to a wanted signal. The Several types of interferer and several frequency relationships are defined in different blocking tests.

Figure 5.3.x-1 Example of an RX blocking requirement

The basic principle of the blocking test is that when the blocker is applied, a throughput target on the wanted signal should continue to be achieved. The wanted signal is set at 6dB above reference sensitivity with the same RMC as for reference sensitivity. Thus in principle distortion arising from the blocker is allowed to desensitize a receiver operating at exactly reference sensitivity by around 6dB.

It should be noted that the wanted signal level is set with respect to reference sensitivity, which is a minimum requirement and not the actual sensitivity level of the receiver, which may be lower.

Two signal levels are required for the blocking requirement; the blocker interferer level and the wanted signal level. To derive the blocker interferer level for wide area BS, uplink system level simulations were performed as documented in the TR 36.942 [x.1]. The blocker interferer level was decided partally by considering the 99.99th percentile of RX power observed from an a non co-located aggressor LTE system on a victim system but mainly based on the UE transmit power and MCL. Later on, the AAS SI in 37.840 [x.2] investigated the dependency of the conducted blocker interferer level on antenna architecture and concluded that for the investigated array types, the conducted blocker interferer level was similar both for a receiver attached to a passive array and one attached to an individual antenna element. Based on this, in 37.105 [3] a per TAB connector conducted blocking interferer  level is defined that is the same as the blocker interferer level defined in 37.104 [x.3].

For UTRA single RAT, blocker interferer levels were derived using system level simulations for the medium range and local area BS classes [x.4]. Higher blocker interferer levels are defined that for the wide area scenario. The wanted signal level is kept at 6dB above the reference sensitivity.

For E-UTRA single RAT, the blocker interferer level for the local area BS class was derived from system level simulations [x.6], and the wanted signal set at 6dB above reference sensitivity. For the medium range BS class, the wanted signal was set at 6dB above reference sensitivity, whilst the blocker interferer level was adjusted such that the difference between the blocker interferer level and the wanted signal level is equalized between E-UTRA wide area and medium range (The adjustment is needed because the reference sensitivity level differs between the two classes) [x.5].

Comparing UTRA and E-UTRA single RAT blocking requirements, there are differences in the blocker interferer levels (UTRA being more larger). For MSR, there is a need for a single set of in band blocking requirements. To achieve this, the UTRA blocker interferer levels, which are more stringent than the E-UTRA were used for the MSR blocking requirement. However the difference between the blocker interferer level and the wanted signal level needed for the wide area BS class to pass blocking is also maintained for the other BS classes. To achieve this, the offset of the wanted signal from the reference sensitivity is adjusted and different to 6dB for E-UTRA. Also the UTRA wanted signal levels are adjusted to keep a common difference between the blocker interferer level and the wanted signal level.