3GPP TSG-RAN WG4 Meeting #83 	R4-1704627

Hangzhou, China, 15 – 19 May, 2017

Agenda item:	7.17.6

Source: 	Intel Corporation

Title: 	NPDSCH simulation results for Rel.14 eNB-IoT

Document for:	Discussion

- Introduction

In RAN4#82bis meeting, the Release 14 eNB-IoT Work Plan [1] and initial simulation assumptions for NPDSCH [2] were agreed. According to the eNB-IoT Work Plan [1], it indicates that:

## RAN4#83 (May 2017)

- Discussion on possible open issues left in RAN4#82Bis

- Initial ideal simulation results alignment as per the simulation assumptions agreed in RAN4#82Bis

- Update the simulation assumptions as per the further discussion and agreements reached during the meeting if necessary

In this contribution, we provide the initial ideal simulation results based on the simulation assumptions in [2].

- NPDSCH simulation assumptions and results

Table 1 and 2 list the parameters for Release 14 Category NB2 UE NPDSCH test cases as detailed in [2].

Test Case 1: Category NB2 UE supporting 1 HARQ process only.

Table 1: Simulation assumptions for NPDSCH with 1 HARQ process configured

| Test number | Deployment mode | Carrier Type | (ITBS, ISF) | Coding rate | Repetition number | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | Stand-alone | Non-Anchor | ([9], [5]) (TBS=936 bits) | 0.5 | 1 | EPA5 | 1 | 70% | TBD |

Test Case 2: Category NB2 UE supporting 2 HARQ processes.

Table 2: Simulation assumptions for NPDSCH with 2 HARQ processes configured

| Test number | Deployment mode | Carrier Type | (ITBS, ISF) | Coding rate | Repetition number | Propagation Condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | Stand-alone | Non-Anchor | ([9], [5]) (TBS=936 bits) | 0.5 | 1 | EPA5 | 1 | 70% | TBD |

Table 3 below lists the TBS table for reference.

Table 3: Transport block size (TBS) table

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 | 968 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1352 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |

As specified in [2], Release 14 eNB-IoT Cat. NB2 UE can select one out of the two above test cases in Table 1 and 2 as per its capability. If Cat. NB2 UE only supports 1 HARQ process, the NB-IoT UE only needs to conduct Test Case 1; otherwise, if Cat. NB2 UE supports 2 HARQ processes, the NB-IoT UE only needs to conduct Test Case 2.

Figure 1 and 2 below plot the NPDSCH simulation results for Normalized Throughput vs. SNR for Cat. NB2 UE supporting 1and 2 HARQ processes, respectively.

Fig. 1 NPDSCH simulation results for Normalized Throughput vs. SNR for Cat. NB2 UE supporting 1 HARQ process

Fig. 2 NPDSCH simulation results for Normalized Throughput vs. SNR for Cat. NB2 UE supporting 2 HARQ processes

Observation 1: The performances for 1 HARQ and 2 HARQ processes are very close. It may result from the simulation assumptions of continuous HARQ re-transmission, and that only the valid downlink NPDSCH subframes are taken into account of the metric of throughput calculation.

Observation 2: 70% of maximum throughput is achieved around 8dB with repetition level of 1. The SNR of 8dB is rather far from the normal coverage target SNR level at -6dB.

Proposal: To increase the repetition level to [32] in order to align the simulation results and demodulation requirements at least around -6dB.

- Conclusion

In this contribution, we provide the initial ideal simulation results based on the simulation assumptions in [2].

Observation 1: The performances for 1 HARQ and 2 HARQ processes are very close. It may result from the simulation assumptions of continuous HARQ re-transmission, and that only the valid downlink NPDSCH subframes are taken into account of the metric of throughput calculation.

Observation 2: 70% of maximum throughput is achieved around 8dB with repetition level of 1. The SNR of 8dB is rather far from the normal coverage target SNR level at -6dB.

Proposal: To increase the repetition level to [32] in order to align the simulation results and demodulation requirements at least around -6dB.

- Reference

[1] R4-1703753, “Work Plan for NB-IoT UE enhancements”, Huawei, Hisilicon, April 2017.

[2] R4-1704147, “Simulation assumptions for NB-IoT enhancements UE demodulation”, Huawei, Hisilicon, Ericsson, Intel, Qualcomm, April 2017.