3GPP TSG-RAN WG4 Meeting #84	R4-1707195

# Berlin, Germany, 21 – 25 August, 2017

Source:	SoftBank Corp.

Title:	TP for TR36.715-04-01: the support of CA_1A-8A-11A-28A BCS0

Agenda item:	8.4.2

Document for:	Approval

1.  Introduction

1A-8A-11A-28A BCS0 was approved in 4DL/1UL basket WID [1]. This paper is to evaluate IM/harmonics and relaxation required, including the relevant TP to complete this combination.

2. Basic Configurations

As written in the WID, the following configurations are assumed for B1+B3+B11+B28.

Table 1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-8-11-28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |

Table 2: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-8A-11A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  | 8 |  |  | Yes | Yes |  |  |  |  |
|  | 11 |  |  | Yes | Yes |  |  |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

3.  Co-existence

On BS side, there is no need to analyse harmonic/IMD in REL-15. On UE side, there are two harmonic issues within this 4DL CA, B28 Tx to B11 Rx and toward B1 Rx (HTF assumed for both in constituent low-order CAs). The UE issues are discussed in the following sections.

4.  ΔTIB and ΔRIB

In terms of ΔTIB and ΔRIB, we can reuse the results of constituent 3DL CAs. Note that, in relation to CA with Band 8, Band 28 uplink is restricted as in Notes.

Table 3: ΔTIB of constituent 3DL/1UL

| ΔTIB | 1+8+11 | 1+8+2811 | 1+11+28 | 8+11+2817 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.3 | 0.3 |  | 0.3 |
| B8 | 0.3 | 0.6 |  | 0.6 | 0.6 |
| B11 | 0.4 |  | 0.4 | 0.4 | 0.4 |
| B28 |  | 0.6 | 0.6 | 0.6 | 0.6 |
| NOTE 11: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 8 NOTE 17: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 11. |  |  |  |  |  |

Table 4: ΔRIB of constituent 3DL/1UL

| ΔRIB | 1+8+11 | 1+8+2810 | 1+11+28 | 8+11+2816 | Proposed Value |
| --- | --- | --- | --- | --- | --- |
| B1 | 0 | 0 | 0 |  | 0 |
| B8 | 0 | 0.2 |  | 0.2 | 0.2 |
| B11 | 0 |  | 0 | 0 | 0 |
| B28 |  | 0.2 | 0.2 | 0.2 | 0.2 |
| NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 8 NOTE 16: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 11. |  |  |  |  |  |

Then the following values are proposed. The uplink restriction is also applied.

Table 5: Proposed ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-8A-11A-28AY | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | 28 | 0.6 |
| NOTE Y: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 8 or Band 11 |  |  |

Table 6: Proposed ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-8A-11A-28AY | 1 | 0 |
|  | 8 | 0.2 |
|  | 11 | 0 |
|  | 28 | 0.2 |
| NOTE Y: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 8 or Band 11 |  |  |

5.  REFSENS

As noted above, two harmonic relations are included in the 4DL CA. B28 Tx would generate the 2nd harmonic to B11 Rx and the 3rd to B1 Rx. However, in relation to CA with Band 8, Band 28 uplink is restricted as mentioned in the Notes in ΔTIB and ΔRIB tables.

With this limitation, there is no need to define REFSENS exception for this 4DL CA.

6.  Summary

This paper investigates all the necessary elements to complete CA_1A-8A-11A-28A. The relevant TP is shown in Annex.

7.  Reference

[1]      RP-170963    New WID: LTE Advanced inter-band CA Rel-15 for 4DL/1UL, Ericsson (Rapporteur).

Annex: TP for TR36.715-04-01 (v0.0.1)

[Unchanged portions skipped]

5.X	CA_1A-8A-11A-28A_BCS0

5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: 4DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-8-11-28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |

Table 5.X.1-2: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-8A-11A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  | 8 |  |  | Yes | Yes |  |  |  |  |
|  | 11 |  |  | Yes | Yes |  |  |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

5.X.2	UE co-existence studies

On UE side, there are two harmonic issues within this 4DL CA, B28 Tx to B11 Rx and toward B1 Rx (HTF assumed for both in constituent low-order CAs). The UE issues are discussed in the following sections.

5.X.3	∆TIB and ∆RIB values

<Text will be added.>

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c and ΔRIB,c is defined for applicable bands in Table 5.X.3-1 and 5.X.3-2 respectively. Note that, in relation to CA with Band 8, Band 28 uplink is restricted in this CA.

Table 5.X.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-8A-11A-28AY | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | 28 | 0.6 |
| NOTE Y: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 8 or Band 11 |  |  |

Table 5.X.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-8A-11A-28AY | 1 | 0 |
|  | 8 | 0.2 |
|  | 11 | 0 |
|  | 28 | 0.2 |
| NOTE Y: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 8 or Band 11 |  |  |

5.X.4	REFSENS requirements

As noted above, two harmonic relations are included in the 4DL CA. B28 Tx would generate the 2nd harmonic to B11 Rx and the 3rd to B1 Rx. However, in relation to CA with Band 8, Band 28 uplink is restricted. With this limitation, there is no need to define REFSENS exception for this 4DL CA.

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"