3GPP TSG-RAN WG4 RAN#84	R4-1707338

Berlin, Germany, August 21st-25th, 2017

Agenda item:	 9.6.6.2.1

Source: 	Qualcomm Incorporated

Title: 	PSS/SSS detection in NR: initial link level simulation results

Document for:	Discussion

# 1. 	Introduction

Following the agreements in [1] for NR link level simulation assumptions of PSS/SSS detection, in this paper we present preliminary simulation results for mmWave bands.

# 2. 	Assumptions and Results

PSS/SSS detection simulations have been done according to the assumption in [1] and with these particular cases:

- mmWave band with synchronous deployment (relative delay of CP/2)

- 1 TX antenna with cross-polarization and 2 RX antennas with ideal receiver beamforming (Rx beam is pointing towards the Rx signal and no beam sweep is simulated).

- Prior knowledge of serving cell – cell ID = 337 (NID1, NID2 = {112,1})

- Velocity of 3 km/h and simulated SS burst periodicity is 20ms

The selected parameters for subcarrier spacing, propagation conditions, Eor/Ioc levels and cell IDs are listed directly in Table 1 with the results, whereby the 90 percentiles of the detection time are reported. The SINR value is measured at the baseband (detector) and no implementation margin nor RF impairments have been simulated.

It can be observed that for 240 kHz subcarrier spacing one burst suffice for all AWGN cases apart colliding NID2, while for more challenging propagation conditions of CDL-A two burst are required. In case of 120 kHz subcarrier spacing the cell can be detected in one burst for almost all cases also with CDL-A propagation conditions.

Table 1. PSS/SSS detection time for Cell identification, in number of SS burst set periods

| Case# | SINR | Cell ID | NID1, NID2 | AWGN |  | CDL-A |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 120 kHz | 240 kHz | 120 kHz | 240 kHz |
| 1 | -6 dB | 674 | {224,2} | 1 | 1 | 1 | 1 |
| 2 |  | 673 | {224,1} | 1 | 1 | 1 | 1 |
| 3 | -7 dB | 674 | {224,2} | 1 | 1 | 1 | 2 |
| 4 |  | 673 | {224,1} | 1 | 1 | 2 | 2 |
| 5 | -8 dB | 674 | {224,2} | 1 | 1 | 2 | 2 |
| 6 |  | 673 | {224,1} | 1 | 2 | 2 | 2 |

# 3. 	Conclusion

In this paper, we provided initial results for PSS/SS detection in NR mmWave band, for a number of parameter combinations according to the simulation assumptions agreed in [1]. The results should be taken into account when RAN4 discusses the requirements.

# 4. 	References

- R4-1706903, “Link level simulation of PSS/SSS detection in NR”, Huawei, HiSilicon