3GPP TSG-RAN WG4 Meeting #84 	R4-1707406

Berlin, Germany, 21 – 25 August, 2017

Agenda item:	7.9.8.2

Source: 	Intel Corporation

Title: 	Discussion on demodulation requirements for Rel-14 non-BL/CE UE

Document for:	Discussion

- Introduction

In RAN4#83 meeting, the demodulation performance of Rel-13 non-BL/CE UE with 2Rx and 4Rx was discussed. A WF [1] was agreed and companies were encouraged to provide simulation results to define new demodulation requirements for Rel-13 non-BL/CE UE supporting coverage enhancement with 2Rx/4Rx. However, there is no agreement on the simulation assumptions and demodulation requirements for Rel-14 non-BL/CE UE supporting 96 PRBs. In this contribution, we provide our view and simulation assumptions for Rel-14 non-BL/CE UE supporting 96 PRBs.

- Discussion and Simulation assumptions for Rel-14 non-BL/CE UE

In [2], it lists the PDSCH simulation assumptions for Rel-14 FeMTC Cat.M2 UE demodulation requirements. One major difference between Rel-14 Cat.M2 UE and Rel-14 non-BL/CE UE is the maximum number of supporting PRBs in PDSCH transmission, which is 24 PRBs for Rel-14 Cat.M2 UE and 96 PRBs for non-BL/CE UE. Although ideal simulation results for Cat.M2 were provided among interested companies, it is still not clear how the non-BL/CE UE would perform with 96 PRBs allocated and with repetition number larger than one. Therefore, we recommend to re-use the parameters of Cat.M2 simulation assumptions in CE Mode A and Mode B in [2] for non-BL/CE UE demodulation performance tests, except for disabling the Frequency Hopping and increasing the PRB number from 24 to 96.

  - Rel-14 Non-BL/CE UE with CE Mode A

Table 1, 2 and 3 list the test parameters, test case and FRC for Rel-14 non-BL/CE UE with CE Mode A.

Table 1. Test parameters for PDSCH TM2 CE Mode A demodulation requirements

| Parameters |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
|  | δ | dB | 3 |
|  | MPDCCH_RA | dB | + δ + 3 |
|  | MPDCCH_RB | dB | + δ + 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Coverage enhancement mode |  |  | CE Mode A |
| PDSCH transmission mode |  |  | 2 |
| OFDM starting symbol (startSymbolBR) |  |  | 2 |
| Maximum number of repetitions |  |  | [4] for 2Rx; [2] for 4Rx |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Disabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | [12] |
| Frequency hopping interval (interval-FDD) |  | ms | [8] |
| MPDCCH transmission duration (mpdcch-NumRepetition) |  | ms | TBD |
| Number of widebands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | [0] |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | TBD |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | [0] |
| MPDCCH aggregation level |  |  | TBD |
| Note 1: . Note 2: For each test,DC subcarrier puncturing shall be considered. |  |  |  |

Table 2 Test case for PDSCH TM2 CE Mode A

| Test number | Bandwidth and MCS | PDSCH max bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) | DL Category |
| 1 | 20MHz QPSK 1/3 | 20MHz | R.yy FDD | OP.2 FDD | EPA5 | 2x2 Low | 70 | TBD | >=1 |
| 2 | 20MHz QPSK 1/3 | 20MHz | R.yy FDD | OP.2 FDD | EPA5 | 2x4 Low | 70 | TBD | >=1 |

Table 3 FRC for PDSCH TM2 CE Mode A

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.yy FDD |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 96 (Note 4) |
| Allocated subframes per Y subframes (Note 3) |  | TBD |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | [8504] |
| For Sub-Frames 0,5 | Bits | [0] |
| Number of Code Blocks |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Code blocks | 1 |
| Binary Channel Bits |  |  |
| For Sub-Frames 1,2,3,4, 6,7,8,9 | Bits | [25344] |
| For Sub-Frames 0,5 | Bits | [0] |
| Max. Throughput | Mbps | TBD |
| UE DL Category |  | >=1 |
| Note 1: 2 symbols allocated to PDCCH Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4] Note 3: The downlink subframes are scheduled at the 0th to X-th subframes every Y ms. Information bit payload is availabled from the Z-th to X-th subframes with repetition. Note 4: Allocated PRB positions are {0, 1, …, 23} within the assigned wideband. Note 5: MPDCCH are scheduled at the 0th to N-th subframes with repetition. The allocated PRB positions are {0, 1, 2, 3} within the assigned narrowband. |  |  |

  - Rel-14 Non-BL/CE UE with CE Mode B

Table 4, 5 and 6 list the test parameters, test case and FRC for Rel-14 non-BL/CE UE with CE Mode B.

Table 1. Test parameters for PDSCH TM2 CE Mode B demodulation requirements

|  | Parameters | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
|  | δ | dB | 3 |
|  | MPDCCH_RA | dB | + δ + 3 |
|  | MPDCCH_RB | dB | + δ + 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Coverage enhancement mode |  |  | CE Mode B |
| PDSCH transmission mode |  |  | 2 |
| OFDM starting symbol (startSymbolBR) |  |  | 2 |
| Maximum number of repetitions |  |  | [32] for 2Rx; [16] for 4Rx |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Disabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | [12] |
| Frequency hopping interval (interval-FDD) |  | ms | [16] |
| MPDCCH transmission duration (mpdcch-NumRepetition) |  | ms | TBD |
| Number of widebands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | [0] |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | TBD |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | [0] |
| MPDCCH aggregation level |  |  | TBD |
| Note 1: . Note 2: For each test,DC subcarrier puncturing shall be considered. |  |  |  |

Table 2 Test case for PDSCH TM2 CE Mode B

| Test number | Bandwidth and MCS | PDSCH max bandwidth | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) | DL Category |
| 1 | 20MHz QPSK 1/10 | 20MHz | R.bb FDD | OP.2 FDD | ETU1 | 2x2 Low | 70 | TBD | >=1 |
| 2 | 20MHz QPSK 1/10 | 20MHz | R.bb FDD | OP.2 FDD | ETU1 | 2x4 Low | 70 | TBD | >=1 |

Table 3 FRC for PDSCH TM2 CE Mode B

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.bb FDD |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 96 (Note 4) |
| Allocated subframes per Y subframes (Note 3) |  | TBD |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/10 |
| Information Bit Payload |  |  |
| For Sub-Frames 1,2,3,4,6,7,8,9 | Bits | [2664] |
| For Sub-Frames 0,5 | Bits | [0] |
| Number of Code Blocks |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Code blocks | 1 |
| Binary Channel Bits |  |  |
| For Sub-Frames 1,2,3,4, 6,7,8,9 | Bits | [25344] |
| For Sub-Frames 0,5 | Bits | [0] |
| Max. Throughput | Mbps | TBD |
| UE DL Category |  | >=1 |
| Note 1: 2 symbols allocated to PDCCH Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4] Note 3: The downlink subframes are scheduled at the 0th to X-th subframes every Y ms. Information bit payload is availabled from the Z-th to X-th subframes with repetition. Note 4: Allocated PRB positions are {0, 1, …, 23} within the assigned wideband. Note 5: MPDCCH are scheduled at the 0th to N-th subframes with repetition. The allocated PRB positions are {0, 1, 2, 3} within the assigned narrowband. |  |  |

Proposal: to use the test parameters, test case and FRC in Table 1 to Table 6 for Rel-14 non-BL/CE UE with CE Mode A and Mode B.

- Conclusion

In this contribution, we provide our view and simulation assumptions for Rel-14 non-BL/CE UE supporting 96 PRBs.

Proposal: to use the test parameters, test case and FRC in Table 1 to Table 6 for Rel-14 non-BL/CE UE with CE Mode A and Mode B.

- Reference

[1] R4-1706196, “WF on requirements for Rel-13 Non-BL UE”, Huawei, HiSilicon, Ericsson, Intel Corporation, Qualcomm, May 2017.

[2] R4-1704179, “Simulation assumption of FeMTC for UE demodulation requirements”, Ericsson, Huawei, HiSilicon, Intel, Qualcomm, April 2017.