3GPP TSG-RAN WG4 Meeting #84	R4-1707471

Berlin, Germany, 21 - 25 Aug, 2017

Agenda item:	8.4.2

Source:	Dish Network

Title:	TP for TR 36.715-04-01: MSD, TIB and RIB for CA_66A-66A-70C, CA_66C-70C, CA_29A-66A-66A-70A, CA_29A-66C-70A, and CA_29A-66A-70C

Document for:	Approval

# 1	Introduction

This contribution provides a TP to capture MSD, TIB and RIB for CA_66A-66A-70C, CA_66C-70C, CA_29A-66A-66A-70A, CA_29A-66C-70A, and CA_29A-66A-70C

# 2	Discussion

The only remaining issue for these CA combinations is MSD for B66+B70 CA. It has been addressed in 2DL/1UL contribution [1].

## 5.X CA_66A-66A-70C_BCS0

## 5.x.1 Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_66-70 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_66A-66A-70C

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_66A-66A-70C | 66 | See the CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 65 | 0 |
|  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

Table 5.x.1-3: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_70C | 5 | 20 |  |  | 25 | 0 |
|  | 10 | 15 |  |  |  |  |
|  | 15 | 10 |  |  |  |  |

Table 5.x.1-4: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

| E-UTRACA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66A-66A | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |

## 5.x.2 UE co-existence studies

UE harmonics are shown in table below.

Table 5.x.2-1 UE harmonics

| UE UL frequencies | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| Frequency [MHz] | 1710 | 1780 | 1695 | 1710 |
| 2nd harmonic [MHz] | 3420 | 3560 | 3390 | 3420 |
| 3rd harmonic [MHz] | 5130 | 5340 | 5085 | 5130 |
| 4th harmonic [MHz] | 6840 | 7120 | 6780 | 6840 |
| 5th harmonic [MHz] | 8550 | 8900 | 8475 | 8550 |

There are no harmonic issues for UE.

## 5.x.3 ΔTIB,c and ΔRIB,c values

The TIB,c values are shown in table 5.x.3-1, accounting values for CA_66A-70A.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_66A-66A-70C | 66 | 0.5 |
|  | 70 | 0.5 |

The RIB,c values are shown in table 5.x.3-2, accounting values for CA_66A-70A.

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_66A-66A-70C | 66 | 0 |
|  | 70 | 0 |

## 5.x.4 REFSENS requirements

As studied in CA_66A-70A, MSD requirement is not needed for CA_66A-70A. CA_66A-66A-70C is similar in this sense, i.e. no MSD requirements are needed for CA_66A-66A-70C.

## 5.X CA_66C-70C_BCS0

## 5.x.1 Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_66C-70C | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_66C-70C

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_66C-70C | 66 | See the CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 65 | 0 |
|  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

Table 5.x.1-3: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66C | 5 | 20 |  |  | 40 | 0 |
|  | 10 | 15, 20 |  |  |  |  |
|  | 15 | 10, 15, 20 |  |  |  |  |
|  | 20 | 5, 15, 10, 20 |  |  |  |  |
| CA_70C | 5 | 20 |  |  | 25 | 0 |
|  | 10 | 15 |  |  |  |  |
|  | 15 | 10 |  |  |  |  |

## 5.x.2 UE co-existence studies

As studied in CA_66A-70A, these are no harmonic issues for UE for CA_66A-70C.

## 5.x.3 ΔTIB,c and ΔRIB,c values

The TIB,c values are shown in table 5.x.3-1, accounting values for CA_66A-70A.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_66C-70C | 66 | 0.5 |
|  | 70 | 0.5 |

The RIB,c values are shown in table 5.x.3-2, accounting values for CA_66A-70A.

Table 6.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_66C-70C | 66 | 0 |
|  | 70 | 0 |

## 5.x.4 REFSENS requirements

As studied in CA_66A-70A, MSD requirement is not needed for CA_66A-70A. CA_66C-70C is similar in this sense, i.e. no MSD requirements are needed for CA_66C-70C.

## 5.X CA_29A-66A-66A-70A_BCS0

## 5.x.1 Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_29-66-70 | 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_29A-66A-66A-70A

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_29A-66A-66A-70A | 29 |  |  | Yes | Yes |  |  | 65 | 0 |
|  | 66 | See the CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
|  | 70 |  |  | Yes | Yes | Yes |  |  |  |

Table 5.x.1-3: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

| E-UTRACA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66A-66A | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |

## 5.x.2 UE co-existence studies

As studied in CA_29A-66A-70A, these are no harmonic issues for UE for CA_29A-66A-66A-70A.

## 5.x.3 ΔTIB,c and ΔRIB,c values

The TIB,c values are shown in table 5.x.3-1, accounting values for CA_66A-70A.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_29A-66A-66A-70A | 29 | N/A |
|  | 66 | 0.5 |
|  | 70 | 0.5 |

The RIB,c values are shown in table 5.x.3-2, accounting values for CA_66A-70A.

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_29A-66A-66A-70A | 29 | 0 |
|  | 66 | 0 |
|  | 70 | 0 |

## 5.x.4 REFSENS requirements

As studied in CA_66A-70A, MSD requirement is not needed for CA_66A-70A. CA_29A-66A-66A-70A is similar in this sense, i.e. no MSD requirements are needed for CA_29A-66A-66A-70A.

## 5.X CA_29A-66C-70A_BCS0

## 5.x.1 Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_29-66-70 | 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_29A-66C-70A

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_29A-66C-70A | 29 |  |  | Yes | Yes |  |  | 65 | 0 |
|  | 66 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | 70 |  |  | Yes | Yes | Yes |  |  |  |

Table 5.x.1-3: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66C | 5 | 20 |  |  | 40 | 0 |
|  | 10 | 15, 20 |  |  |  |  |
|  | 15 | 10, 15, 20 |  |  |  |  |
|  | 20 | 5, 15, 10, 20 |  |  |  |  |

## 5.x.2 UE co-existence studies

As studied in CA_29A-66A-70A, these are no harmonic issues for UE for CA_29A-66C-70A.

## 5.x.3 ΔTIB,c and ΔRIB,c values

The TIB,c values are shown in table 5.x.3-1, accounting values for CA_66A-70A.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_29A-66C-70A | 29 | N/A |
|  | 66 | 0.5 |
|  | 70 | 0.5 |

The RIB,c values are shown in table 5.x.3-2, accounting values for CA_66A-70A.

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_29A-66C-70A | 29 | 0 |
|  | 66 | 0 |
|  | 70 | 0 |

## 5.x.4 REFSENS requirements

As studied in CA_66A-70A, MSD requirement is not needed for CA_66A-70A. CA_29A-66C-70A is similar in this sense, i.e. no MSD requirements are needed for CA_29A-66C-70A.

## 5.X CA_29A-66A-70C_BCS0

## 5.x.1 Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_29-66-70 | 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_29A-66A-70C

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_29A-66A-70C | 29 |  |  | Yes | Yes |  |  | 55 | 0 |
|  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

Table 5.x.1-3: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_70C | 5 | 20 |  |  | 25 | 0 |
|  | 10 | 15 |  |  |  |  |
|  | 15 | 10 |  |  |  |  |

## 5.x.2 UE co-existence studies

As studied in CA_29A-66A-70A, these are no harmonic issues for UE for CA_29A-66A-70C.

## 5.x.3 ΔTIB,c and ΔRIB,c values

The TIB,c values are shown in table 5.x.3-1, accounting values for CA_66A-70A.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_29A-66A-70C | 29 | N/A |
|  | 66 | 0.5 |
|  | 70 | 0.5 |

The RIB,c values are shown in table 5.x.3-2, accounting values for CA_66A-70A.

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_29A-66A-70C | 29 | 0 |
|  | 66 | 0 |
|  | 70 | 0 |

## 5.x.4 REFSENS requirements

As studied in CA_66A-70A, MSD requirement is not needed for CA_66A-70A. CA_29A-66A-70C is similar in this sense, i.e. no MSD requirements are needed for CA_29A-66A-70C.

# 3	TP

## ----- Start of Text proposal -----

## 5.X CA_66A-66A-70C_BCS0

## 5.x.1 Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_66-70 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_66A-66A-70C

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_66A-66A-70C | 66 | See the CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 65 | 0 |
|  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

Table 5.x.1-3: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_70C | 5 | 20 |  |  | 25 | 0 |
|  | 10 | 15 |  |  |  |  |
|  | 15 | 10 |  |  |  |  |

Table 5.x.1-4: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

| E-UTRACA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66A-66A | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |

## 5.x.2 UE co-existence studies

UE harmonics are shown in table below.

Table 5.x.2-1 UE harmonics

| UE UL frequencies | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| Frequency [MHz] | 1710 | 1780 | 1695 | 1710 |
| 2nd harmonic [MHz] | 3420 | 3560 | 3390 | 3420 |
| 3rd harmonic [MHz] | 5130 | 5340 | 5085 | 5130 |
| 4th harmonic [MHz] | 6840 | 7120 | 6780 | 6840 |
| 5th harmonic [MHz] | 8550 | 8900 | 8475 | 8550 |

There are no harmonic issues for UE.

## 5.x.3 ΔTIB,c and ΔRIB,c values

The TIB,c values are shown in table 5.x.3-1, accounting values for CA_66A-70A.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_66A-66A-70C | 66 | 0.5 |
|  | 70 | 0.5 |

The RIB,c values are shown in table 5.x.3-2, accounting values for CA_66A-70A.

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_66A-66A-70C | 66 | 0 |
|  | 70 | 0 |

## 5.x.4 REFSENS requirements

As studied in CA_66A-70A, MSD requirement is not needed for CA_66A-70A. CA_66A-66A-70C is similar in this sense, i.e. no MSD requirements are needed for CA_66A-66A-70C.

## 5.X CA_66C-70C_BCS0

## 5.x.1 Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_66C-70C | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_66C-70C

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_66C-70C | 66 | See the CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 65 | 0 |
|  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

Table 5.x.1-3: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66C | 5 | 20 |  |  | 40 | 0 |
|  | 10 | 15, 20 |  |  |  |  |
|  | 15 | 10, 15, 20 |  |  |  |  |
|  | 20 | 5, 15, 10, 20 |  |  |  |  |
| CA_70C | 5 | 20 |  |  | 25 | 0 |
|  | 10 | 15 |  |  |  |  |
|  | 15 | 10 |  |  |  |  |

## 5.x.2 UE co-existence studies

As studied in CA_66A-70A, these are no harmonic issues for UE for CA_66A-70C.

## 5.x.3 ΔTIB,c and ΔRIB,c values

The TIB,c values are shown in table 5.x.3-1, accounting values for CA_66A-70A.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_66C-70C | 66 | 0.5 |
|  | 70 | 0.5 |

The RIB,c values are shown in table 5.x.3-2, accounting values for CA_66A-70A.

Table 6.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_66C-70C | 66 | 0 |
|  | 70 | 0 |

## 5.x.4 REFSENS requirements

As studied in CA_66A-70A, MSD requirement is not needed for CA_66A-70A. CA_66C-70C is similar in this sense, i.e. no MSD requirements are needed for CA_66C-70C.

## 5.X CA_29A-66A-66A-70A_BCS0

## 5.x.1 Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_29-66-70 | 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_29A-66A-66A-70A

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_29A-66A-66A-70A | 29 |  |  | Yes | Yes |  |  | 65 | 0 |
|  | 66 | See the CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
|  | 70 |  |  | Yes | Yes | Yes |  |  |  |

Table 5.x.1-3: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

| E-UTRACA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66A-66A | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 40 | 0 |

## 5.x.2 UE co-existence studies

As studied in CA_29A-66A-70A, these are no harmonic issues for UE for CA_29A-66A-66A-70A.

## 5.x.3 ΔTIB,c and ΔRIB,c values

The TIB,c values are shown in table 5.x.3-1, accounting values for CA_66A-70A.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_29A-66A-66A-70A | 29 | N/A |
|  | 66 | 0.5 |
|  | 70 | 0.5 |

The RIB,c values are shown in table 5.x.3-2, accounting values for CA_66A-70A.

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_29A-66A-66A-70A | 29 | 0 |
|  | 66 | 0 |
|  | 70 | 0 |

## 5.x.4 REFSENS requirements

As studied in CA_66A-70A, MSD requirement is not needed for CA_66A-70A. CA_29A-66A-66A-70A is similar in this sense, i.e. no MSD requirements are needed for CA_29A-66A-66A-70A.

## 5.X CA_29A-66C-70A_BCS0

## 5.x.1 Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_29-66-70 | 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_29A-66C-70A

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_29A-66C-70A | 29 |  |  | Yes | Yes |  |  | 65 | 0 |
|  | 66 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | 70 |  |  | Yes | Yes | Yes |  |  |  |

Table 5.x.1-3: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_66C | 5 | 20 |  |  | 40 | 0 |
|  | 10 | 15, 20 |  |  |  |  |
|  | 15 | 10, 15, 20 |  |  |  |  |
|  | 20 | 5, 15, 10, 20 |  |  |  |  |

## 5.x.2 UE co-existence studies

As studied in CA_29A-66A-70A, these are no harmonic issues for UE for CA_29A-66C-70A.

## 5.x.3 ΔTIB,c and ΔRIB,c values

The TIB,c values are shown in table 5.x.3-1, accounting values for CA_66A-70A.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_29A-66C-70A | 29 | N/A |
|  | 66 | 0.5 |
|  | 70 | 0.5 |

The RIB,c values are shown in table 5.x.3-2, accounting values for CA_66A-70A.

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_29A-66C-70A | 29 | 0 |
|  | 66 | 0 |
|  | 70 | 0 |

## 5.x.4 REFSENS requirements

As studied in CA_66A-70A, MSD requirement is not needed for CA_66A-70A. CA_29A-66C-70A is similar in this sense, i.e. no MSD requirements are needed for CA_29A-66C-70A.

## 5.X CA_29A-66A-70C_BCS0

## 5.x.1 Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_29-66-70 | 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for CA_29A-66A-70C

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_29A-66A-70C | 29 |  |  | Yes | Yes |  |  | 55 | 0 |
|  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

Table 5.x.1-3: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_70C | 5 | 20 |  |  | 25 | 0 |
|  | 10 | 15 |  |  |  |  |
|  | 15 | 10 |  |  |  |  |

## 5.x.2 UE co-existence studies

As studied in CA_29A-66A-70A, these are no harmonic issues for UE for CA_29A-66A-70C.

## 5.x.3 ΔTIB,c and ΔRIB,c values

The TIB,c values are shown in table 5.x.3-1, accounting values for CA_66A-70A.

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_29A-66A-70C | 29 | N/A |
|  | 66 | 0.5 |
|  | 70 | 0.5 |

The RIB,c values are shown in table 5.x.3-2, accounting values for CA_66A-70A.

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_29A-66A-70C | 29 | 0 |
|  | 66 | 0 |
|  | 70 | 0 |

## 5.x.4 REFSENS requirements

As studied in CA_66A-70A, MSD requirement is not needed for CA_66A-70A. CA_29A-66A-70C is similar in this sense, i.e. no MSD requirements are needed for CA_29A-66A-70C.

----- End of Text proposal -----

# References

[1] R4-1707469, “TP for TR 36.715-02-01: MSD for CA_66A-70A”, Dish network, RAN4#84