3GPP TSG-RAN4 Meeting #84	R4-1708202

# Berlin, Germany, 21st Aug 2017 - 25th Aug 2017

Source:	Rohde & Schwarz, Azimuth Systems Incorporated, Anritsu, Keysight Technologies, CATR, Fraunhofer HHI

Title:	NR UE test interface functions

Agenda Item:		9.7.2

Document for:	Approval

# Introduction

According to [1] a Test Interface (TI) is needed for certain NR UE control and measurement functions. However, detailed functions and implementation of the TI are TBD still. While [2] proposed that RAN 4 should collect high level test interface functions, [3] to [7] previously discussed a number of potential TI functions and deployment approaches already.

In response to [2], this document provides a minimum set of mandatory high level functions for approval. Optional high level functions for the sake of test time and complexity reduction will be discussed separately, if required by RAN4. Functional and implementation details shall be part of upcoming RAN5 test case specification efforts.

# Discussion

Contributions [3] to [7] discussed the need of a NR UE TI to support OTA UE RF testing in particular. Therefore, [1] clearly states the need for such a TI, however, left the detailed TI functions specification open. This contribution provides a minimum list of mandatory high level TI functions for approval. As it is very likely, that this initial list turns out to be incomplete with the evolving NR standards, it shall be revised continuously to add further functions in future releases. Implementation details, such as command and data formats are not regarded as the responsibility of RAN4, but rather RAN5.

RF baseline test systems according to [1] are designed to support centre of beam and off centre of beam OTA measurements for frequency range 2. As NR UEs will implement adaptive antenna systems, the beam steering implementation is up to the UE vendor and will be difficult, if not impossible to control via default NR procedures. However, to allow off centre of the beam measurements without a dedicated link antenna, the beam steering mechanism should be disabled during the measurement for the sake of accuracy, traceability and repeatability.

Thus, a high level function for a NR UE TI would be a beam locking function for both TX and RX beams, to support off centre of beam RF UE OTA measurements. As RF UE OTA measurements are mandatory tests, the required beam locking function shall be mandatory, too.

Proposal 1: A test function is needed to enable/disable beam locking to support off centre of beam OTA UE RF measurements.

To verify radiated multi-antenna spatial diversity or spatial multiplexing reception performance, knowledge of actual channel conditions during the test are required, i.e., the actual complex downlink channel matrix is needed, which can only be provided by the UE under test. For example, sustained downlink data rate testing require ideal downlink spatial RF conditions. To setup such ideal spatial downlink RF conditions there is a need to eliminate as much as possible the impact of the actual downlink channel matrix. Thus, a function is required to retrieve the complex downlink channel matrix data from the UE while keeping the UE downlink (receive) antenna settings unchanged during the performance measurement.

Proposal 2: In order to verify radiated multi-antenna diversity reception performance, a function is needed to fix the UE DL antenna settings and to retrieve the complex (magnitude and phase) receive signal strength information per receiver. This will allow to set up ideal downlink conditions for various test purposes, e.g. sustained downlink data rate testing.

For some test cases, e.g., upper OSI layer signalling test cases, no explicit measurement antennas are required beside the link antenna. In those cases, the RF conditions are of minor interest, however, they are assumed to be good enough to avoid any influence on the test result. For such kind of tests, quasi-omnidirectional transmission and reception characteristics would create RF test conditions independent of the relative position of the DUT with respect to the link antenna. This will allow test setups w/o any positioning system requirements for all such tests.

Proposal 3: In order to setup reliable OTA test conditions, e.g., for protocol tests, a function is needed to lock the beams to quasi-omnidirectional characteristics. This might be combined with proposal 1 function.

# Proposals

The following mandatory TI functions to allow NR OTA UE testing are proposed for approval:

Proposal 1: A test function is needed to enable/disable beam locking to support off centre of beam OTA UE RF measurements.

Proposal 2: In order to verify radiated multi-antenna diversity reception performance, a function is needed to fix the UE DL antenna settings and to retrieve the complex (magnitude and phase) receive signal strength information per receiver. This will allow to set up ideal downlink conditions for various test purposes, e.g. sustained downlink data rate testing.

Proposal 3: In order to setup reliable OTA test conditions, e.g., for protocol tests, a function is needed to lock the beams to quasi-omnidirectional characteristics. This might be combined with proposal 1 function.

# References

[1]	3GPP TR 38.803 Version 14.1.0

[2]	3GPP RAN4 NR-AH#2, R4-1706510, „On test interface parameters“, Intel, June 2017

[3]	3GPP RAN4#81, R4-1609160 „On test interface for NR UE“ Intel Corporation, CATR

[4]	3GPP RAN4#81, R4-1609809 „Control commands for NR UE testability“, Anritsu

[5]	3GPP RAN4#80bis, R4-167277 „UE testing interface for NR RF“, Intel Corporation, CATR

[6]	3GPP RAN4#80bis, R4-168558, „Special conformance test functions for 5G testability“, Keysight

[7]	3GPP RAN4 NR AH#1, R4-1700078, „NR UE test interface considerations“, Rohde & Schwarz, January 2017

[8]	3GPP TR 37.977, „Verification of radiated multi-antenna reception performance of User Equipment (UE)“