3GPP TS G-RAN WG4 Meeting #84		R4-1708652

# Berlin, Germany, 21 - 25 August, 2017

Source:	CATT

Agenda item:	7.15.2.1

Title:	Demodulation performance for eFD-MIMO Semi-Open-Loop transmission

Document for:	Discussion

Introduction

From last RAN4 meeting, RAN4 started the WI work of defining performance requirements of certain new features introduced in Rel-14 enhanced FD-MIMO [1].  And agreements were achieved for the demodulation evaluation on semi-open-loop transmission in the WF [2].

- Test parameters for Rank1 test case

  - 2*2 EVA[70Hz] with Medium correlation, QPSK 1/3

- Test parameters for Rank2 test case

  - 16QAM , EVA [70Hz] , ULA low correlation

    - Option 1:  4*2 MIMO

    - Option 2: 2*2 MIMO

- Precoding matrix

  - Using precoding matrix following CSI calculation assumption as defined in RAN1 spec with single-stage codebook without PMI feedback during test

  - 2Tx: 1/sqrt(2) [1 0;0 1];

  - 4Tx:

In this contribution, we provide simulation results based on the agreed simulation assumptions provided in [3]. The detailed simulation parameters are attached in the Annex.

Discussion

Two PDSCH demodulation test cases agreed for semi-open-loop transmission as summarized below:

| Test number | Bandwidth | MCS | Propagation condition | Correlation matrix and antenna config. |
| --- | --- | --- | --- | --- |
| 1 (Rank1） | 10 MHz | QPSK 1/3 | EVA [70Hz] | 2x2 Medium |
| 2 (Rank2) | 10 MHz | 16QAM 1/2 | EVA [70Hz] | Option1:4x2 Low Option2: 2x2 Low |

Precoding matrix:

- For 2 antennas, the precoding matrix corresponding to codebook index 0 in Table 6.3.4.2.3-1 of 36.101 with

- For 4 antennas, the precoding matrix for REs of jth PRB-pair according to , where  is the precoder index given by and denote precoder matrices corresponding to precoder indices 12,13,14 and 15, respectively, in Table 6.3.4.2.3-2 of  36.101 with

Simulation assumptions

The simulation assumptions are given in table 1&2 below. Considering the design purpose of semi-open-loop MIMO transmission is to achieve robust performance for medium and high mobility UEs, so a much higher Doppler scenario should be evaluated as a more generic case for the transmission scheme. Here EVA200Hz is suggested as the typical RAN1 evaluation scenario of this transmission scheme with UE speed at about 120km/h. And the accuracy of PMI can be implicitly verified in a demodulation test case.

Table1 Main parameters for simulations

| parameter |  | Unit | Test 1 (rank1) | Test (rank2) |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | TBD | TBD |
| Correlation and antenna configuration |  |  | 2*2 Medium | Option1: 4*2 Low Option 2: 2*2 Low |
| Propagation channel |  |  | EVA [70Hz]/[200Hz] | EVA [70Hz] /[200Hz] |
| semiolEnabled |  |  | Enable | Enable |
| Cell-specific reference signals |  |  | 0, 1 | 0, 1 |
| CSI reference signals |  |  | Antenna ports 15, …, 18 | Antenna ports 15, …, 18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | 5 / 2 |
| CSI reference signal configuration |  |  | 0 | 0 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG | OCNG |
| FRC |  |  | QPSK 1/3, Rank1 | 16QAM 1/2 Rank2 |
| Number of allocated resource blocks |  | PRB | 50 | 50 |
| Simultaneous transmission |  |  | No | No |
| PDSCH transmission mode |  |  | 9 | 9 |
| Number of MBSFN subframes |  | Subframes | NA | NA |

Table2. FRC table for the single & multiple PMI test cases

| Paramter | Unit | Semi-OL transmission |  |
| --- | --- | --- | --- |
| Reference channel |  | 4 CSI-RS ports 2CRS ports QPSK 1/3 Rank1 | 4 CSI-RS ports 2CRS ports 16QAM Rank2 |
| Duplex Mode |  | FDD | FDD |
| Channel bandwidth | MHz | 10 | 10 |
| Allocated resource blocks (Note 1) |  | 50 | 50 |
| Allocated subframes per Radio Frame |  | 9 | 9 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 1/2 |
| Information Bit Payload |  |  |  |
| For Sub-Frames (Non CSI-RS subframe) | Bits | 3624 (MCS 4) | 11448 (MCS 13) |
| For Sub-Frames (CSI-RS subframe) | Bits | 3624 (MCS 4) | 11448 (MCS 13) |
| For Sub-Frames (ZeroPowerCSI-RS subframe) | Bits | 3624 (MCS 4) | 11448 (MCS 13) |
| For Sub-Frame 5 | Bits | N/A | N/A |
| For Sub-Frame 0 | Bits | 2984 (MCS 4) | 9528 (MCS 13) |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frames (Non CSI-RS subframe) |  | 1 | 2 |
| For Sub-Frames (CSI-RS subframe) |  | 1 | 2 |
| For Sub-Frames (ZeroPowerCSI-RS subframe) | Bits | 1 | 2 |
| For Sub-Frame 5 |  | N/A | N/A |
| For Sub-Frame 0 |  | 1 | 2 |
| Binary Channel Bits Per Sub-Frame |  |  |  |
| For Sub-Frames (Non CSI-RS subframe) | Bits | 12000 | 24000 |
| For Sub-Frames (CSI-RS subframe) | Bits | 11600 | 23200 |
| For Sub-Frames (ZeroPowerCSI-RS subframe) | Bits | 11600 | N/A |
| For Sub-Frame 5 | Bits | N/A | N/A |
| For Sub-Frame 0 | Bits | 9840 | 19680 |
| Max. Throughput averaged over 1 frame | Mbps | 3.1976 | 10.1112 |
| UE Category |  | ≥ 2 | ≥ 2 |
| Note1:50 resource blocks are allocated in sub-frames 1, 2, 3, 4, 6, 7, 8, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0; |  |  |  |

Simulation results

Here we give our simulation results of the demodulation performance of medium (40km/h) and high (120km/h) mobility UEs.

RANK1 2*2  EVA medium

RANK2 EVA

- RANK2 EVA 2*2 Low

- RANK2 EVA 4*2 Low

Table 3.  SNR points  @ 70%  of  the maximum throughput (FDD)

| Test Case |  | EVA70Hz--SNR(dB) | EVA200Hz--SNR(dB) |
| --- | --- | --- | --- |
| RANK1 | EVA 2*2 medium | 0.71 | 1.02 |
| RANK2 | EVA 4*2 Low | 12.71 | 13.62 |
|  | EVA 2*2 Low | 12.81 | 13.82 |

Observation:  From our simulation results, the performance degradations for high Doppler test cases are less than 1dB.

Proposal 1:  Use EVA200Hz to specify the requirements for the semi-open-loop MIMO transmission to better align with the purpose of RAN1 design.

Proposal 2: Specify the RANK2 test case with 4Tx MIMO Considering the different implementation and test coverage.

Conclusion

Observation:  From our simulation results, the performance degradation for high Doppler test cases is less than 1dB.

Proposal 1:  Use EVA200Hz to specify the requirements for the semi-open-loop MIMO transmission to better align with the purpose of RAN1 design.

Proposal 2: Specify the RANK2 test case with 4Tx MIMO Considering the different implementation and test coverage.

- References

[1] R1-1704021, “Agreements for Rel.14 eFD-MIMO up to RAN1#88”, Samsung, Nokia, CATT, RAN1 #88, Feb 2017

[2] R1-1706194, “Way forward on eFD-MIMO performance requirements”, Samsung, Qualcomm, Huawei, Ericsson, RAN4 #83, May 2017

[3] R4-1706268, “Simulation assumption for eFD-MIMO performance requirements”, Samsung, RAN4#83,May 2017