3GPP TSG-RAN WG4 Meeting #84bis	R4-1710399

Dubrovnik, Croatia, 09 - 13 October 2017

Agenda item:	10.1.1

Title: 	Discussion on the antenna correlation impact on 8Rx scenarios

Source: 	Intel Corporation

Document for:	Discussion

- Introduction

The Release 15 Study Item “Study on LTE DL 8Rx antenna ports” has been approved and introduced in RAN#76 and the Study Item Description is given in [1] with an objective to study the performance gain with 8 Rx antenna configuration compared to a smaller number of receive antennas, such as 4 Rx.

In RAN4#84 Meeting, some initial simulation results have shown in [2] that the PDSCH throughput vs. SNR performance is very sensitive to the correlation level between 8Rx antennas. And, the antenna correlation issue was captured and extensively discussed in the RAN4#84 8Rx SI. Interested companies would provide more studies and evaluation on the antenna correlation impact for the 8Rx antenna configuration scenarios.

Therefore, in this contribution, we first provide more simulation results for 8Rx with various combinations of Rx antenna numbers, antenna correlation levels and MIMO layers. Then, we share our view on the benefit and gain of 8Rx antenna configurations compared with those with a smaller amount of antenna numbers.

# Antenna Correlation Discussion and Simulation Results

As a starting point, there are three antenna correlation models addressed in the Way Forward [3] from RAN4#84 Meeting. The antenna correlation parameters α and β are specified and shown in Table 1 below.

Table 1 The  and  parameters for ULA MIMO correlation matrices

| Correlation Model |  |  |
| --- | --- | --- |
| Low Correlation | 0 | 0 |
| Medium Correlation A | 0.3 | 0.3874 |
| Medium Correlation B | 0.3 | 0.005154 |

The justification to define β = 0.005154 for 8Rx in Medium Correlation B is to maintain the same correlation level for 4Rx in Medium Correlation A for two adjacent antennas. Generally speaking, this also implies that 8Rx in Medium Correlation B would double the size/form factor compared to 4Rx in Medium Correlation A. Based on such justification, if the size/form factor has to be the same while increasing the Rx antenna number, the antenna correlation parameters of 8Rx would be similar to that of 4Rx (e.g. apply Medium Correlation A to both 4Rx and 8Rx configurations).

Next, in this section, we provide three scenarios to demonstrate the correlation impact on the 8Rx antenna configuration.

Scenario 1: Single-layer Spatial Multiplexing

In Scenario 1, we consider an existing 4Rx testcase for TM9 Single-layer Spatial Multiplexing, which is an MU-MIMO testcase and specified in TS 36.101 [4] Section 8.10.1.1.5A, and is also summarized and listed below:

Table 8.10.1.1.5A-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with multiple CSI-RS configurations

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Beamforming model |  |  | Annex B.4.1 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | Yes (Note 3, 5) |
| PDSCH transmission mode |  |  | 9 |
| Number of MBSFN subframes |  | Subframes | NA |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: Modulation symbols of an interference signal is mapped onto the antenna port (7 or 8) not used for the input signal under test. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 5: The two UEs’ scrambling identities are set to 0 for CDM-multiplexed DM RS with interfering simultaneous transmission test cases. |  |  |  |

Table 8.10.1.1.5A-2: Minimum performance for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.50 FDD | OP.1 FDD | EPA5 | 2x4 Low | 70 | 15.8 | ≥2 |
| Note 1: The reference channel applies to both the input signal under test and the interfering signal. |  |  |  |  |  |  |  |  |

We then try to re-use the above 4Rx testcase for 8Rx configuration as a measure of comparison between 4Rx and 8Rx antenna configurations. Moreover, besides the “ULA low correlation” case, we also consider the “ULA medium correlation A” and “ULA medium correlation B” cases as specified in Table 1 above.

Fig.1 Normalized throughput performance for TM9, single-layer spatial multiplexing, EPA5

Observation 1: For the MU-MIMO Low Correlation (ULA-LowCorr) case, 2×8 antenna configuration achieves about 1.5dB gain against 2×4 antenna configuration at 70% maximum throughput.

Observation 2: For the MU-MIMO 2×4 and 2×8 both with Medium Correlation A (ULA-MedCorrA) cases, which suggests the same size/form factor for 4Rx and 8Rx, performance of 8Rx is even worse than 4Rx.

Observation 3: For the MU-MIMO 2×8 with Medium Correlation B (ULA-MedCorrB) case, we see about 4dB gain, compared with 2×4 ULA-MedCorrA case.

Proposal 1: To include both noise and interference limited scenarios for further study in the 8Rx SI and to capture the conclusion in the TR.

Scenario 2: Dual-layer Spatial Multiplexing

In Scenario 2, we consider cases for TM9 Dual-layer Spatial Multiplexing, and the simulation assumptions are listed in Table 2 below.

Table 2 Simulation Assumptions for Scenario 2 of TM 9 Dual-layer Spatial Multiplexing

| Duplex Mode | FDD |
| --- | --- |
| Transmission Mode (TM) | 9 |
| System Bandwidth (MHz) | 10 |
| Propagation Condition | EPA5 |
| MCS | 18 (64QAM) |
| Antenna Configuration | 4×4 / 4×8 |

Fig.2 Normalized throughput performance for TM9, dual-layer spatial multiplexing, EPA5

Observation 4: For TM9 dual-layer MCS18, 4×4 with Medium Correlation A cannot achieve the maximum throughput; while, by enlarging the size/form factor, 4×8 with Medium Correlation B can achieve maximum if SNR is sufficiently high.

Observation 5: For TM9 dual-layer MCS18, when SNR is greater than 30dB, 4×4 with Medium Correlation A obtains higher throughput than 4×8 with Medium Correlation A.

Scenario 3: TM9 8×8 8-layer Spatial Multiplexing

In Scenario 3, we consider cases for TM9 8×8 8-layer Spatial Multiplexing, and the simulation assumptions are listed in Table 3 below.

Table 3 Simulation Assumptions for Scenario 3 of TM 9 8×8 8-layer Spatial Multiplexing

| Duplex Mode | FDD |
| --- | --- |
| Transmission Mode (TM) | 9 |
| System Bandwidth (MHz) | 10 |
| Propagation Condition | EPA5 |
| MCS | 17 (64QAM) |
| Antenna Configuration | 8×8 |

Fig.3 Normalized throughput performance for TM9 8×8 8-layer Spatial Multiplexing

Observation 6: TM9 8×8 8-layer Spatial Multiplexing is very sensitive to the antenna correlation factor. In Fig.3, it shows that the maximum throughput for 64QAM MCS17 can be achieved when both correlation parameters α and β are zero. However, a very tiny increase in correlation value, such as β = 10-6 would bring dramatic degradation in throughput vs. SNR performance.

Proposal 2: To further study the antenna correlation issue in the 8Rx SI for different correlation models in different MIMO layer scenarios, and to capture the conclusion in the TR.

# Conclusions

In this contribution, we first provided simulation results for 8Rx with various combinations of Rx antenna numbers, antenna correlation levels and MIMO layers. Then, we share our view on the 8Rx antenna configuration compared with those with a smaller amount of antenna numbers.

We made the following observations and proposals:

Observation 1: For the MU-MIMO Low Correlation (ULA-LowCorr) case, 2×8 antenna configuration achieves about 1.5dB gain against 2×4 antenna configuration at 70% maximum throughput.

Observation 2: For the MU-MIMO 2×4 and 2×8 both with Medium Correlation A (ULA-MedCorrA) cases, which suggests the same size/form factor for 4Rx and 8Rx, performance of 8Rx is even worse than 4Rx.

Observation 3: For the MU-MIMO 2×8 with Medium Correlation B (ULA-MedCorrB) case, we see about 4dB gain, compared with 2×4 ULA-MedCorrA case.

Observation 4: For TM9 dual-layer MCS18, 4×4 with Medium Correlation A cannot achieve the maximum throughput; while, by enlarging the size/form factor, 4×8 with Medium Correlation B can achieve maximum if SNR is sufficiently high.

Observation 5: For TM9 dual-layer MCS18, when SNR is greater than 30dB, 4×4 with Medium Correlation A obtains higher throughput than 4×8 with Medium Correlation A.

Observation 6: TM9 8×8 8-layer Spatial Multiplexing is very sensitive to the antenna correlation factor. In Fig.3, it shows that the maximum throughput for 64QAM MCS17 can be achieved when both correlation parameters α and β are zero. However, a very tiny increase in correlation value, such as β = 10-6 would bring dramatic degradation in throughput vs. SNR performance.

Proposal 1: To include both noise and interference limited scenarios for further study in the 8Rx SI and to capture the conclusion in the TR.

Proposal 2: To further study the antenna correlation issue in the 8Rx SI for different correlation models in different MIMO layer scenarios, and to capture the conclusion in the TR.

# References

[1] RP-171491, “New SI proposal: Study on LTE DL 8Rx antenna ports”, Huawei, HiSilicon, 3GPP RAN #76, June 2017.

[2] R4-1707409, “Initial PDSCH simulation results for 8Rx with rank lower than or equal to 4”, Intel Corporation, August 2017.

[3] R4-1708729, “Way forward on 8Rx simulation assumptions”, Huawei, HiSilicon, Sprint, August 2017.

[4] TS 36.101 User Equipment (UE) radio transmission and reception v13.6.1 (2017-01).