3GPP TSG-RAN WG4 Meeting  #84bis                                                                        R4-1710469

Dubrovnik, Croatia, 9 - 13 Oct., 2017

Source:	CATT

Title:	Discussion  on radiated interface boundary(RIB)

Agenda Item:	8.27.1

Document for:	  Approval

Introduction

In TR 37.843[1], The OTA AAS BS radiated requirements are defined at the radiated interface boundary (RIB), which is specified to be in the far field, i.e. at least  meters distance away from the AAS BS. Thus, confusions may rise about whether the PL between the AAS BS and the RIB will be included in the specified requirement or not. Hence we think it is necessary to clarify that the radiated requirements don’t include any path-loss between the AAS BS and RIB.

A text proposal is attached to add the following note in TR 37.843:

Note 9: the specified radiated requirements don’t include any path-loss between the AAS BS and RIB.

Conclusion

This contribution discussed RIB. It is proposed to agree on the corresponding text proposal in the annex.

Reference

- 3GPP TR 37.843 V0.5.0 (2017-10), Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) radiated requirements (Release 15).

Annex

<Text Proposal>

<Unchanged texts omitted>

- 4.4	General AAS BS RF architecture

An AAS BS may have 2 different types of RF interface at which the RF and performance requirements are defined:

- Conducted RF interface at the transceiver array boundary (TAB) and a radiated interface boundary (RIB) in the far field for a hybrid AAS BS.

- Radiated interface boundary (RIB) only for an OTA AAS BS.

For both types of AAS BS the radio architecture is represented by three main functional blocks, the transceiver unit array (TRXUA), the radio distribution network (RDN), and the antenna array (AA). The transceiver units (TRXU) interface with the base band processing within the AAS BS.

The TRXUA consists of multiple transmitter units (TXU) and receiver units (RXUs). The TXU takes the baseband input from the AAS BS and provides the RF TX outputs. The OTA AAS BS is restricted to architectures which have ≥8 TRXUs for E_UTRA and ≥4 TRXUs for UTRA, restricting the minimum number of TRXUs in the OTA AAS BS architecture allows the emissions scaling factor to be fixed and avoids defining an OTA definition of an active transceiver unit which is difficult without the ability to count TAB connectors. AAS BS with small numbers of TRXUsshould retain a conducted interface and are classified as hybrid AAS BS.

The RF TX outputs are distributed to the AA via a RDN. The RXU performs the reverse of the TXU operations. The RDN performs the distribution of the TX outputs into the corresponding antenna paths and antenna elements, and a distribution of RX inputs from antenna paths in the reverse direction. The TXU and RXU can be separated and can have different mapping through the RDN towards the AA.

For the hybrid AAS BS the transceiver array boundary (TAB) is the point(s) at which the TRXUA is connected to the RDN. The point where a TXU or RXU connects with the RDN is equivalent to an "antenna connector" of a non-AAS BS and is called TAB connector. The TAB connector is defined as conducted requirement reference point. The transmitted signal per carrier from one Transmitter Unit appear at one or more TAB connector(s) and the received signals per carrier from one or more TAB connector(s) appear at a single RXU.

Figure 4.4-1 shows a general architecture of a hybrid AAS BS radio architecture, where M is the total number of transceiver units and K is the total number of TAB connectors at the transceiver array boundary.

The OTA AAS BS has no transceiver array boundary or TAB connectors defined as it has no conducted requirements. The OTA AAS BS radiated requirements are defined at the radiated interface boundary (RIB).

Figure 4.4-2 shows a general architecture of an OTA AAS BS radio architecture, where P is the total number of transceiver units and P ≥ 8 (E-UTRA) or P ≥ 4 (UTRA) for OTA AAS BS.

NOTE 1:	The RDN may consist of a simple one to one mapping between the TXU(s)/RXU(s) and the passive antenna array. In this case, the RDN would be a logical entity but not necessarily a physical entity.

NOTE 2:	The antenna array includes various implementations and configurations e.g. polarization, array geometry (including element factor and element separation), etc.

NOTE 3:	The physical location of the TRXUA, the RDN, and the AA may differ from this logical representation and is implementation dependent.

NOTE 4:	No specific mapping in the RDN between TAB connectors and antenna elements is assumed. Further the number of separate receiver and transmitter units as well as the mapping in the RDN between TAB connectors and  radiating elements can differ between the transmit and receive directions. Both AAS BS reference architectures allow for full asymmetry between receiver path and transmit path.

NOTE 5:	For AAS BS capable of supporting applications employing beamforming, all or subgroups of TAB connectors can be configured with designated amplitude and phase weights such that one or more beams are radiated from the antenna array.

NOTE 6:	If text of this TR and figures 4.4-1, 4.4-2 contradict each other, then the TR text applies.

NOTE 7: 	In the case of the OTA AAS BS, the fixed scaling factor (FSF) equals  8 for E-UTRAand 4 for UTRA and is based on the maximum number of layers/streams specified in Rel-12. The FSF value may be further reconsidered for future releases if the maximum number of layers/streams supported by RAT(s) supported by the AAS BS is changed.

NOTE 8: 	If an OTA AAS BS is declared to support more than 1 cell (Ncells>1) the total number of transceiver units must be greater than FSF* Ncells , i.e. for 8* Ncells for E-UTRA or 4* Ncells for UTRA.

NOTE 9:  The specified radiated requirements don’t include any path-loss between the AAS BS and RIB.

Antenna array   (AA)

Radio distribution network (RDN)

#1

#2

#K

Transceiver array boundary (TAB)

Transceiver array boundary connector

Transceiver unit array

(TRXUA)

1 to M

Far field region

Composite antenna

Radiated interface boundary (RIB)

Figure 4.4-1: General architecture of hybrid AAS BS and the reference points for the conducted and radiated requirements

Antenna array   (AA)

Radio distribution network (RDN)

Transceiver unit array

(TRXUA)

1 to P

Far field region

Composite antenna

Radiated interface boundary (RIB)

Figure 4.4-2: General architecture of OTA AAS BS and the reference point for the radiated requirements

<Unchanged texts omitted>

<End of Text Proposal>