3GPP TSG-RAN WG4 Meeting #84bis                                                                              R4-1710480

# Dubrovnik , Croatia, 9 - 13 Oct, 2017

Title:                        SS block SCS of n41 and n78

Source: 	CMCC

Agenda Item:	9.3.1.1

Document for:	Approval

1   Introduction

RAN4 discussed the SS block SCS for a few meetings. The current agreements for SS block subcarrier spacing of different band is as follows:

| Sub 6 NR Bands | Minimum Channel bandwidth | SS block sub-carrier spacing |
| --- | --- | --- |
| 1 | 5 MHz | 15 kHz |
| 3 | 5 MHz | 15 kHz |
| 5 | 10MHz / 5MHz | [30 kHz/15kHz] |
| 7 | 5 MHz | 15 kHz |
| 8 | 5 MHz | 15 kHz |
| 20 | 5 MHz | 15 kHz |
| 28 | 5 MHz | 15 kHz |
| 41 | 10MHz | [15KHz/30 kHz] |
| 66 | 5 MHz / 10MHz | [15 kHz/30kHz] |
| 70 | 5 MHz | 15 kHz |
| 71 | 5 MHz | 15 kHz |
| 1.427-1.518 GHz | 5 MHz | 15 kHz |
| 3.3 – 3.8 GHz | 10 MHz | [15 kHz/30kHz] |
| 3.3 - 4.2 GHz | 10 MHz | [15 kHz/30kHz] |
| 4.4 - 4.99 GHz | [40 MHz] | 30 kHz |

For some bands, two values of SS block SCS are proposed according to operators’ request. In this contribution, we further discuss the SS block SCS for n41 and 3.5GHz.

2. Discussion

For n41 and n78, both 15KHz and 30KHz is feasible to support 10MHz minimum bandwidth. In order to reduce the UE initial search time and complexity, it is preferable to have only one default SS block SCS in each band from both UE and network perspective. Note that for NSA, the SS block SCS can be configured to either 15KHz or 30KHz.

RAN1 discussed the numerology sets from beginning of NR WI. The agreed numerologies for 15KHz/30KHz/60KHz are provided in Table 1.

Table 1: Numerologies

| Subcarrier spacing | 15kHz | 30kHz | 60kHz |
| --- | --- | --- | --- |
| Symbol duration(us) | 66.67 | 33.33 | 16.67 |
| CP length(us) (CP0/CP1) | 5.21/4.69 | 2.60/2.34 | 1.30/1.17 |
| Num of OS in 1ms duration | 14 | 28 | 56 |
| Overhead of CP | 6.67% | 6.67% | 6.67% |

In OFDM system, Doppler shift could disrupt the orthogonally and cause severe inter-carrier interference (ICI), especially when the subcarrier spacing is not large enough. According to our analysis in [1], 30KHz subcarrier spacing is needed to support high speed scenario in NR (i.e. 500km/h above 2GHz).

Observation 1: 30KHz SS block SCS is preferable for high speed scenario.

According to RAN1 agreement, the maximum number of subcarriers per NR carrier is 3300 or 6600 in Rel-15. Since RAN4 assumes 4096 FFT size, the maximum subcarrier is 3300 in Rel-15. So 15KHz can support NR maximum bandwidth of 50MHz and 30kHz can support NR maximum bandwidth of 100MHz. n41 and n78 are relative wide bands for below 6GHz. For operators who have wider spectrum in there two bands, most likely 30KHz will be used for data. Also, RAN4 agreed that support of simultaneous reception of data/SS with mixed numerologies is optional from UE implementation perspective. So 30KHz SS block SCS is preferable for support of wider band operation.

Observation 2: 30KHz SS block SCS is preferable for support of wider band operation.

Beam sweeping of SS blocks is supported in NR in order to improve the common channel coverage, especially for higher frequency, e.g. 3.5GHz. According to RAN1 design, the maximum number of SS blocks (beams) is 8 for frequency range between 3GHz and 6GHz. For 15KHz, transmitting 8 SS blocks will consume 4ms (see Figure 1), which will lead to larger overhead compared to 30KHz. Hence, 30KHz SS block SCS is preferable for improving cell coverage.

Observation 3: 30KHz SS block SCS is preferable for improving cell coverage in 3.5GHz.

Figure 1: SS block mapping pattern

In RAN4 NR#3, some company raised concern on the inter-symbol interference of 30KHz especially for LTE re-farming band, since the CP length of 30KHz is half compared to 15KHz [2]. Figure 2 shows the simulation results of 15KHz/30KHz/60KHz under different delay spread. It can be observed that even under delay spread 1000ns, 30KHz can achieve similar performance as 15KHz. To our understanding, 1000ns delay spread is very large value and can cover most deployment scenarios. Hence we believe there is no ISI issue for 30KHz.

Figure 2: Performance for different subcarrier spacing for 2GHz 3km/h

Based on our above observations, we propose to apply 30KHz SS block SCS for n41 and n78.

Proposal 1: It is proposed that SS block subcarrier spacing of n41 is 30KHz.

Proposal 2: It is proposed that SS block subcarrier spacing of n78 is 30KHz.

2   Conclusions

In this contribution, we discuss the SS block SCS for n41 and n78. Our observations and proposals are provided as follows:

Observation 1: 30KHz SS block SCS is preferable for high speed scenario.

Observation 2: 30KHz SS block SCS is preferable for support of wider band operation.

Observation 3: 30KHz SS block SCS is preferable for improving cell coverage in 3.5GHz.

Proposal 1: It is proposed that SS block subcarrier spacing of n41 is 30KHz

Proposal 2: It is proposed that SS block subcarrier spacing of n78 is 30KHz

3   References

[1] R1- 167107, Discussion and Evaluation on Numerology Design for High Speed Train Scenario, CMCC. [2] R4-1709414, SCS of sync channel for n41, Softbank