3GPP TSG-RAN WG4 Meeting #84bis                                                                              R4-1710481

# Dubrovnik , Croatia, 9 - 13 Oct, 2017

Title:                        Channel raster for n41

Source: 	CMCC

Agenda Item:	9.3.2.1

Document for:	Approval

- Introduction

In RAN4#84 meeting, a way forward on channel raster for NR was agreed in [1] as follows.

- Channel raster for LTE re-farming bands up to 2.4GHz (frequency range below Band 41) is based on 100kHz(same as LTE)

- For Band 41, FFS 100kHz channel raster or subcarrier based raster should be used.

- Channel raster for Bands above 2.6GHz (above Band 41) is tentatively agreed to be a subcarrier based raster (i.e 15kHz for range 1 and 60kHz for range 2), pending further check at RAN4 NR AH#3 and RAN1 decision

Channel raster for band n41 is still not decided yet. In this contribution, we will discuss the channel raster for n41.

2. Discussion

Different from other LTE re-farming band, n41 is a relative large band with 190MHz spectrum. For operators who have wide spectrum in this band, it is preferable to have flexibility of achieving 0 guardband CA. This is also one important pros of subcarrier based raster compared to 100KHz channel raster.

Observation 1: Subcarrier based channel raster is preferable for achieving 0 guardband CA.

One concern on subcarrier based raster is the number of sync entries. For 100KHz, the number of sync entries is 1900. For subcarrier based raster (15KHz), the sync entries is over 12000. However, as discussed in [2], such frequent entries are most likely not needed. Sync raster based around 100kHz multiple with an integer multiple of 15KHz can be defined in order to reduce the sync entries. Hence, if no further down selection of sync raster can be achieved (e.g. reducing PBCH), at least subcarrier based raster can have the same sync entries as 100KHz.

Observation 2: Subcarrier based channel raster will not cause additional sync entries compared to 100KHz.

Hence, according to our above observations, we propose to apply subcarrier based channel raster for n41.

Proposal: It is proposed that channel raster for n41 is a subcarrier based raster.

2   Conclusions

In this contribution, we discuss the channel raster for n41. The observations and proposals are provided as follows:

Observation 1: Subcarrier based channel raster is preferable for achieving 0 guardband CA.

Observation 2: Subcarrier based channel raster will not cause additional sync entries compared to 100KHz.

Proposal: It is proposed that channel raster for n41 is a subcarrier based raster.

3   References

[1] R4-1708850, WF on channel raster for NR, Qualcomm. [2] R4-1709313, Synchronization Raster for NR, Qualcomm.