3GPP TSG-RAN WG4#84bis	R4-1710498

# Dubrovnik, Croatia, 9 - 13 October 2017

# Title:						mmWave antenna arrangement for UE OOB blocking tests

Source:			Anritsu

Agenda Item:			9.4.4.5.2

Document for:			Approval

# 1.	Introduction

In RAN4 NR AH3 meeting, the UE-blocking-test proposal below has been approved. [1]

Proposal: For all mmWave UE blocking tests, the wanted signal and blockers are applied in the peak gain direction (lowest % point in EIS CDF) regardless of the blocker frequency offset.

In other words, the incident directions of all the blockers and the wanted signal shall be specified to be common across wide frequency range.  This document shares a testability issue in UE OOB blocking tests.

# 2. Discussion

# 2.1 OOB blocking requirements

As written in the introduction above, it has been approved that the wanted signal and the blockers are applied in a common direction regardless of the blocker frequency offset. [1]

The exact frequency range of the out-of-band blocker has not yet been decided so far, and the document of [2] encourages companies to provide further information on actual-field blocker frequencies in mmWave and lower frequency ranges.  But referring to contribution documents on out-of-band blocking [3][4], the frequency range of the blockers for UE out-of-band blocking tests seems likely to be defined as very wide range, such as from 1MHz to 90GHz.

2.2 Consideration of Measurement Antenna Arrangement

Since the incidence direction of the wanted signal and the blockers are to be common, all of those signals and blockers should be radiated from a common antenna in a test environment.  But considering actual available antenna specs, there exists no such a single antenna that covers the whole wide frequency range for outputting the out-of-band blockers.

Observation1:  Multiple antennas shall be necessary to cover the whole wide frequency range of out-of-band blockers.

Observation2: Using multiple antennas to radiate the wanted signal and the out-of-band blockers across wide frequency ranges, it seems difficult to exactly align all the incident directions of the wanted signal and the blockers.

Proposal1:  The requirements and test tolerances of OOB blocking shall be created taking account of the points described in Observation1 and Observation2 above.

DUT

Mid-freq antenna

Low-freq antenna

High-freq antenna

The peak gain direction

Figure 1.   Incident direction difference caused by multiple antennas

# 3.	Conclusion

This document has pointed out the testability issue in UE OOB blocking tests as below.

Observation1:  Multiple antennas shall be necessary to cover the whole wide frequency range of out-of-band blockers.

Observation2: Using multiple antennas to radiate the wanted signal and the out-of-band blockers across wide frequency ranges, it seems difficult to exactly align all the incident directions of the wanted signal and the blockers.

Proposal1:  The requirements and test tolerances of OOB blocking shall be created taking account of the points described in Observation1 and Observation2 above.

# 4. References

[1] R4-1709449, “mmWave UE blocking tests”, MediaTek Inc., RAN4 NR AH#3

[2] R4-1706801, “Out-of-band blocking around 28 GHz”, NTT DOCOMO, INC., RAN4 NR AH#2

[3] R4-1703001, “UE above-6 GHz OOB blocking considerations & proposals”, Intel Corporation, RAN4#82bis

[4] R4-1702043, “Out of band blocking for UE receivers”, Intel Corporation, RAN4#82