3GPP TSG-RAN WG4 Meeting #84-bis	R4-1710657

Dubrovnik, HR, Date 9th – 13th Oct, 2017

Source:	MediaTek Inc

Title:	Correction TP for TR36.715-05-01: the support of CA_1A-3C-7A-8A_1UL_BCS0

Agenda item:	8.5.2

Document for:	Approval

1.  Introduction

CA_1A-3C-7A-8A_1UL_BCS0 was approved in 5DL/1UL basket WID in the RAN-P#76 meeting [1]. TP for completion of the CA has been provided[2]. This paper is to correct error in tables in [2] for 5.x.4 REFSENS requirements.

2.  Reference

[1]      RP-171466 LTE Advanced inter-band CA Rel-15 for 5DL/1UL, RAN #76.

[2]      R4-1707249  TP_1A-3C-7A-8A_1UL_BCS0, RAN4#84

Annex: TP for TR36.715-05-01 (va.b.c)

[Unchanged portions skipped]

CA_1A-3C-7A-8A_1UL_BCS0

[Unchanged portions skipped]

### 5.x.4	REFSENS requirements

Additions in Table 7.3.1A-0a of TS 36.101: The REFSENS for carrier aggregation QPSK PREFSENS,CA (exceptions due to harmonic issue) for 1A-3C-7A-8A need to be included as defined in below Table 5.x.4-1. Values derived from 1A-3A-7A-8A.

Table 5.x.4-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration |  | EUTRA band |  | 1.4 MHz (dBm) |  | 3 MHz (dBm) |  | 5 MHz (dBm) |  | 10 MHz (dBm) |  | 15 MHz (dBm) |  | 20 MHz (dBm) |  | Duplex mode |  |  |
| CA_1A-3C-7A-8A4 | 1 |  |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 3 |  |  |  | N/A | N/A | N/A | N/A |  |
|  | 7 |  |  |  | N/A | N/A | N/A | N/A |  |
|  | 8 |  |  |  | N/A | N/A |  |  |  |
| CA_1A-3C-7A-8A4,5,6 | 1 |  |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 733 |  |  |  | -88 | -87.4 | -87 | -86.7 |  |
|  | 8 |  |  |  | -96.8 | -93.8 |  |  |  |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Additions in TS36.101 Table 7.3.1A-0b for 1A-3C-7A-8A need to be included as defined below:

Table 5.x.4-2: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3C-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |

Additions in TS36.101 Table 7.3.1A-0bD1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel) for 1A-3C-7A-8A need to be included as defined below. Values derived from 1A-3C-7A with Band 8 added:

Table 5.x.4-3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration |  | EUTRA band |  | 1.4 MHz (dBm) |  | 3 MHz (dBm) |  | 5 MHz (dBm) |  | 10 MHz (dBm) |  | 15 MHz (dBm) |  | 20 MHz (dBm) |  | Duplex mode |  |  |
| CA_1A-3C-7A-8A |  | 1 |  |  |  |  |  | -100 |  | -97 |  | -95.2 |  | -94 |  | FDD |  |  |
|  |  | 34,9 |  |  |  |  |  | -94 |  | -91.5 |  | -90 |  | -89 |  |  |  |  |
|  |  | 35 |  |  |  |  |  | -97 |  | -94 |  | -92.2 |  | -91 |  |  |  |  |
|  |  | 7 |  |  |  |  |  | -98 |  | -95 |  | -93.2 |  | -92 |  |  |  |  |
|  |  | 8 |  |  |  |  |  | -96.8 |  | -93.8 |  |  |  |  |  |  |  |  |
| NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Additions in TS36.101 Table 7.3.1A-0bD2 for 1A-3C-7A-8A need to be included as defined below.

Table 5.x.4-4: Uplink configuration for the low band (exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3C-7A-8A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"