3GPP TSG-RAN WG4 Meeting #84bis	R4-1710711

Dubrovnik, Croatia, 9 – 13, October 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM measurement performance for feNB-IoT in-band deployment

Agenda item:	8.20.4.1

Document for:	Discussion

# 1 Introduction

In RAN4#84 meeting in Berlin RAN4 had the initial discussions related to feNB-IoT work. One outcome from the meeting was agreed simulation assumptions for evaluating NRSRP measurement performance with NRSRP based on either NB-SSS alone or combined with NRS. In this paper, we provide our simulation results.

# 2 Discussion

Firstly, we take a look at the existing NRSRP measurement accuracy requirements as agreed for NB-IoT Rel-13. These requirements were developed based on NRS and inband deployment. In Rel-13 RAN4 agreed to have accuracy requirement for UEs under normal and enhanced coverage as follows:

### 8.14.2	NB-IoT intra frequency measurements under normal coverage

#### 8.14.2.1	NB-IoT intra frequency measurements when no DRX is used

In the RRC_CONNECTED state the measurement period for intra frequency measurements is 800ms. The NRSRP measurement accuracy shall be as specified in the sub-clauses 9.1.22.1.

### 8.14.3	NB-IoT intra frequency measurements under enhanced coverage

#### 8.14.3.1	NB-IoT intra frequency measurements when no DRX is used

In the RRC_CONNECTED state the measurement period for intra frequency measurements is 1600ms. The NRSRP measurement accuracy shall be as specified in the sub-clauses 9.1.22.1.

Besides the non-DRX requirements also requirements for UE in DRX were defined. The actual measurement accuracy requirements are listed in table 9.1.22.1-1 below.

Table 9.1.22.1-1: NRSRP Intra frequency absolute accuracy for UE Category NB1 for HD-FDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -6 dB | NFDD_G | [-118] | N/A | [-70] |
| 8 | 11 | -6 dB | NFDD_G | N/A | [-70] | [-50] |
| 10.3 | 13.3 | -15 Ês/Iot -6 dB | NFDD_G | [-118] | N/A | [-70] |
| 12.3 | 15.3 | -15 Ês/Iot -6 dB | NFDD_G | N/A | [-70] | [-50] |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

As discussed in RAN4#84 RAN1 has decided to inform the UE about any power offset between the NRS and the NB-SSS signal. This means that it is possible to utilize the NB-SSS signal also for NRSRP measurements – either alone or combined with available NRS signals.

Using the NB-SSS signal for RRM measurements provides a significant increase in the number of reference signals available for the UE for measurements which will enable better measurement accuracy on UE side. Next, we look at the achievable NRSRP measurement accuracy using the NB-SSS signal.

# 3 Simulation Results

## 3.1 Simulation setup

In the simulations, we use measurement BW of 1 RB which is also the same as the NB-IoT system BW. We have used 4 different L1 measurement periods. Power imbalance between NB-SSS and NRS has not been applied and we have only used the NB-SSS signal for measurements.

We have used in-band deployment scenario, with puncturing of the NB-SSS by 2Tx CRS. We have used 40ms sampling rate and average all samples over the L1 measurement period.

In the following sections present the results for NRSRP. All results are without impairment margin (except of course the reference legacy requirements numbers).

## 3.2 Simulation Results

### 3.2.1 NRSRP -6 dB

In Table 1 the results for SNR of -6dB is shown for different L1 measurement periods and different channel models.

Table 1: Results for SNR of -6dB

| SNR [dB] | Meas period [ms] | Channel model | 5% | 50% | 95% | Absolute accuracy | Relative Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| -6 | 200 | AWGN | -1.57 | -0.13 | 0.90 | 1.57 | 2.47 |
|  |  | EPA1 | -2.29 | -0.03 | 3.68 | 3.68 | 5.97 |
|  |  | ETU1 | -2.67 | -0.14 | 2.17 | 2.67 | 4.84 |
| -6 | 400 | AWGN | -1.08 | -0.11 | 0.70 | 1.08 | 1.78 |
|  |  | EPA1 | -1.90 | -0.23 | 1.45 | 1.90 | 3.35 |
|  |  | ETU1 | -1.99 | -0.25 | 1.14 | 1.99 | 3.13 |
| -6 | 800 | AWGN | -0.99 | -0.36 | 0.27 | 0.99 | 1.26 |
|  |  | EPA1 | -1.23 | -0.23 | 0.48 | 1.23 | 1.71 |
|  |  | ETU1 | -1.05 | -0.24 | 0.48 | 1.05 | 1.53 |
| -6 | 1600 | AWGN | -0.66 | -0.16 | 0.21 | 0.66 | 0.87 |
|  |  | EPA1 | -1.02 | -0.26 | 0.26 | 1.02 | 1.28 |
|  |  | ETU1 | -0.87 | -0.27 | 0.34 | 0.87 | 1.21 |

Currently in Rel-13, the L1 measurement period when under normal coverage is 800ms. If we look at L1 measurement period of 800ms and AWGN channel model, and compare the results with the current requirements, we see a significant improvement in the measurement accuracy (note: NB-SSS results are without impairments):

| SNR [dB] | Signal | Meas period [ms] | Channel model | 5% | 50% | 95% | Absolute accuracy | Relative Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -6 | NB-SSS | 800 | AWGN | -0.99 | -0.36 | 0.27 | 0.99 | 1.26 |
| -6 | NRS | 800 | AWGN |  |  |  | 6 |  |

### 3.2.2 NRSRP -12.6 dB

In Table 2, the results for SNR of -12,6dB is shown for different L1 measurement periods and different channel models.

Table 2: Results for SNR of -12,6dB

| SNR [dB] | Meas period [ms] | Channel model | 5% | 50% | 95% | Absolute accuracy | Relative Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| -12.6 | 200 | AWGN | -4.21 | 0.44 | 3.05 | 4.21 | 7.26 |
|  |  | EPA1 | -3.65 | 1.10 | 8.71 | 8.71 | 12.36 |
|  |  | ETU1 | -3.33 | 1.01 | 7.15 | 7.15 | 10.48 |
| -6 | 400 | AWGN | -4.28 | 0.03 | 2.08 | 4.28 | 6.36 |
|  |  | EPA1 | -3.23 | 0.37 | 5.47 | 5.47 | 8.70 |
|  |  | ETU1 | -3.32 | 0.33 | 4.68 | 4.68 | 8.00 |
| -6 | 800 | AWGN | -2.70 | -0.21 | 1.68 | 2.70 | 4.38 |
|  |  | EPA1 | -3.38 | 0.06 | 2.88 | 3.38 | 6.26 |
|  |  | ETU1 | -2.78 | 0.06 | 2.78 | 2.78 | 5.56 |
| -6 | 1600 | AWGN | -2.39 | -0.39 | 0.96 | 2.39 | 3.35 |
|  |  | EPA1 | -2.08 | -0.10 | 1.64 | 2.08 | 3.72 |
|  |  | ETU1 | -2.04 | -0.07 | 1.52 | 2.04 | 3.56 |

### 3.2.3 NRSRP -15 dB

In Table 3, the results for SNR of -15dB is shown for different L1 measurement periods and different channel models.

Table 3: Results for SNR of -15dB

| SNR [dB] | Meas period [ms] | Channel model | 5% | 50% | 95% | Absolute accuracy | Relative Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| -15 | 200 | AWGN | -4.35 | 1.50 | 4.43 | 4.43 | 8.78 |
|  |  | EPA1 | -3.13 | 2.72 | 11.35 | 11.35 | 14.48 |
|  |  | ETU1 | -3.05 | 2.35 | 9.42 | 9.42 | 12.47 |
| -6 | 400 | AWGN | -4.59 | 0.59 | 3.26 | 4.59 | 7.85 |
|  |  | EPA1 | -3.52 | 1.11 | 7.73 | 7.73 | 11.25 |
|  |  | ETU1 | -3.83 | 1.28 | 6.92 | 6.92 | 10.75 |
| -6 | 800 | AWGN | -4.52 | 0.17 | 2.53 | 4.52 | 7.05 |
|  |  | EPA1 | -3.70 | 0.66 | 5.12 | 5.12 | 8.82 |
|  |  | ETU1 | -3.55 | 0.72 | 4.64 | 4.64 | 8.19 |
| -6 | 1600 | AWGN | -3.47 | 0.09 | 2.06 | 3.47 | 5.53 |
|  |  | EPA1 | -3.56 | -0.12 | 2.84 | 3.56 | 6.40 |
|  |  | ETU1 | -3.53 | 0 | 2.59 | 3.53 | 6.12 |

Currently in Rel-13, the L1 measurement period when under normal coverage is 1600ms. If we look at L1 measurement period of 1600ms and AWGN channel model, and compare the results with the current requirements, we see a significant improvement in the measurement accuracy (note: NB-SSS results are without impairments):

| SNR [dB] | Signal | Meas period [ms] | Channel model | 5% | 50% | 95% | Absolute accuracy | Relative Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -15 | NB-SSS | 1600 | AWGN | -3.47 | 0.09 | 2.06 | 3.47 | 5.53 |
| -15 | NRS | 1600 | AWGN |  |  |  | 10.3 |  |

## 3.2 Simulation Results Discussion

Based on these initial simulation results it seems clear that the UE measurement accuracy using the NB-SSS can be significantly improved compared to legacy when using only the NRS.	For the normal coverage mode, the improvement could be in the order of around 2-3dB when accounting the UE impairments of ~2dB. For enhanced coverage mode, the accuracy improvement potential seems ever bigger – around 4-5dB when accounting the UE impairments of ~2dB.

In addition to discussing the NRSRP accuracy RAN4 will of course also have to discuss the NRSRQ accuracy requirements.

Finally, it would also necessary for RAN4 to discuss whether UE support of NB-SSS signal for measurements could be allowed also for pre-Rel-15 UE as early implementable solution?

# 4 Conclusion

In this paper, we provide our simulation results evaluating NRSRP measurement performance with NRSRP based on NB-SSS alone. Based on these initial simulation results it seems clear that the measurement accuracy using the NB-SSS can be significantly improved compared to legacy when using only the NRS. We accuracy improvement potential in the order of 2-5dB depending on the coverage use case.

# References

# [1] R4-1709045, Simulation assumptions for feNB-IoT in-band deployment,

# Annex

### Simulation assumptions for feNB-IoT in-band deployment [R4-1709045]

| Parameters | Value | Comments |
| --- | --- | --- |
| Measurement bandwidth | 1 resource blocks | Both RSRP and RSRQ measured over 1 RB |
| System bandwidth | 1 resource blocks |  |
| L1 measurement period | 200ms, 400ms, 800 ms, 1600ms |  |
| Measurement sampling rate | 1 sample/40ms | Implementation dependent (NOTE 1) |
| L3 filtering | Disabled |  |
| Antenna configuration | 2TX, 1RX | Puncturing of NB-PSS and NB-SSS in in-band operation when colliding with 2TX CRS pattern Diversity scheme should be indicated in the simulation. |
| Power imbalance between NSSS and NRS | -3dB, 0 dB, 3 dB | Only applicable for measurement based on combination of NSSS and NRS |
| Mobility | Stationary UEs, |  |
| Channel model | AWGN, ETU 1Hz, EPA 1Hz | Models stationary device |
| Measurement type | Synchronization signals only based (NSSS), Combination of NRS and synchronization signals (NSSS) | Companies are encouraged to provide simulation results for measurements based on follows signals. |
| CP length | Normal |  |
| Carrier frequency | 2 GHz |  |
| Ec/Iot | -15dB, -12.6dB, -6dB | AWGN noise |
| Frequency error modelling | +/-50 Hz | With respect to reference cell |
| NOTE 1: Companies are requested to provide the details of the measurement sampling rate for interpretation and comparison of the results NOTE 2: Companies are requested to provide the details of the RS averaging techniques for interpretation and comparison of the results. |  |  |