3GPP TSG-RAN WG4 Meeting #84bis	R4-1710796

Dubrovnik, Croatia, 9 – 13 August 2017

Agenda Item:	6.2.3.2

Source:	Ericsson

Title:	SI acquisition time for non-BL CE UE

Document for:	Discussion

# 1	Introduction

RAN#83 agreed with the way forward on SI acquisition time for non-BL UE as follows [1]:

- RAN4 will consider shortening the SI acquisition time testing requirement for non-BL CE Mode B UE assuming 2Rx in Rel-14.

- RAN4 will consider how to clarify the SI acquisition time requirement for non-BL CE Mode B UE assuming 2Rx in Rel-14 in the core requirement.

- Companies are encouraged to propose the SI acquisition time for non-BL CE Mode B UE based on the practical simulation results in RAN4#84.

Since RAN4#84 had no progress on this issue, we resubmit the same contribution which proposes to reduce the SI acquisition time for non-BL UE based on the practical simulation results.

# 2	Discussion

According to TS36.133, TSI-EUTRA-M1-CEModeB is the time required for receiving all the relevant system information, and it is set to 6,400ms. In general, TSI is the time just after UE complete the cell selection to the time UE transmit PRACH preamble corresponding to the RRC message such as RRC connection request or RRC connection re-establishment request. Since PRACH preamble scheduling information is signaled on SIB2, TSI should include the acquisition time for MIB, SIB1-BR and SIB2-BR.

Table 1 shows the simulation parameters for MIB and SIB1-BR acquisition time. The way forward [1] specifies the TBS for SIB1-BR is 152bits. However, we found it is not possible to set according to TS36.213 Table 7.1.7.2.7-1, and it should be set to at least 208bits (ITBS=1).

Table 2 is the simulation parameters for SIB2-BR acquisition time. Unlike SIB1-BR, the periodicity (si-Periodicity, si-RepetitionPattern) and the repetition number (si-WindowLength-BR) is configurable in SIB1-BR message as well as TBS (si-TBS). RAN4 has not well discussed the simulation assumption for SIB2-BR, but we set the values in Table 2. We set si-WindowLength-BR=40ms, si-RepetitionPattern=everyRF, and si-Periodicity=160ms. This means SIB2 is transmitted on PDSCH every 160ms where it is repeated 40 times continuously.

Table 1	Simulation assumption for MIB/SIB1-BR acquisition time.

| Parameters | CE Mode B |
| --- | --- |
| System bandwidth | 10MHz |
| Transmit antennas | 2 |
| Propagation channel | EPA1, ETU1 |
| TBS for SIB1-BR | 208 bits |
| SIB1-BR repetition number | 16 |
| PBCH repetition | Enabled |
| UE receive antenna | 2 |
| UE frequency offset | 100 Hz |
| Evaluation criteria | 99% MIB decoding success rate at SNR=-15dB 99% SIB1-BR decoding success rate at SNR=-15dB |

Table 2	Simulation assumption for SIB2-BR acquisition time.

| Parameters | CE Mode B |
| --- | --- |
| System bandwidth | 10MHz |
| Transmit antennas | 2 |
| Propagation channel | EPA1, ETU1 |
| si-TBS | 208 bits |
| si-WindowLength-BR | 40ms |
| si-RepetitionPattern | everyRF |
| si-Periodicity | 160ms (rf16) |
| UE receive antenna | 2 |
| UE frequency offset | 100 Hz |
| Evaluation criteria | 99% SIB2-BR decoding success rate at SNR=-15dB |

Table 3	Required window size for MIB/SIB1-BR/SIB2-BR acquisition

|  | MIB | SIB1-BR (TBS152bits) | SIB1-BR (TBS=208bits) | SIB2-BR |
| --- | --- | --- | --- | --- |
| EPA1, SNR=-15dB | 20 | 9 | 15 | 9 |
| ETU1, SNR=-15dB | 19 | 12 | 19 | 11 |
| Window length per MIB window size: 40ms Window length per SIB1-BR window size: 80ms Window length per SIB2-BR window size: 160ms |  |  |  |  |

Table 3 is our simulation result assuming impairments. In the table, we compare the required window sizes for TBS=152bits and TBS=208bits. It is observed that this change results in more acquisition time to achieve 99% decoding success rate. If we consider 20 windows for MIB, 19 windows for SIB1-BR, and 11 windows for SIB2-BR, the total acquisition time becomes 40ms*(20+1) + 80ms*(19+1) + 160ms*(11+1) = 4,360ms. Considering the margin, we propose to set TSI-EUTRA-NonBL-CEModeB = 4,800ms for non-BL CE Mode B UE.

Proposal: RAN4 set TSI-EUTRA-NonBL-CEModeB = 4,800ms for non-BL CE Mode B UE.

Note that the RRM requirement usually assumes 90% success rate. Since this proposal is derived from 99% success rate, we think there is enough margin to achieve the RRM requirements.

# 3	Conclusion

Proposal: RAN4 set TSI-EUTRA-NonBL-CEModeB = 4,800ms for non-BL CE Mode B UE.

# 4	Reference

R4-1706183, “Way forward on SI acquisition time requirements for non-BL/CE UE”, Ericsson.

R4-1707675, “SI acquisition time for non-BL CE UE in Rel-14”, Ericsson.