3GPP TSG-RAN WG4 Meeting #84bis	R4-1710960

Dubrovnik, Croatia, 9 – 13 October 2017

Source: 	Nokia, Nokia, Nokia Shanghai Bell

Title: 					TP to TR 38.817-01: Applicability of bandwithd combination sets to NR

Agenda Item: 			9.4.1

Document for:	Approval

# 1	Introduction

In this contribution we capture the agreements [1] on bandwidth combinations sets into the TR 38.817-01.

# 2	References

[1] R4-1710083, WF on BCS and CA bandwidth class for LTE-NR DC, Nokia, Nokia Shanghai Bell, RAN4#NR AH3

******************************* Start of the TP ******************************************

## 4.2	Channel bandwidth

Detailed structure of the subclause is TBD.

### 4.2.1	Bandwidth combination sets

Following agreements have beed made for NR Bandwidth combination sets:

Condering NR bands in LTE-NR DC:

- Bandwidth combinations sets are not specified for NR bands part of LTE-NR DC configuration

- Where IMD is an issue only for a subset of channel bandwidths within a band/band combination, the bandwidths for which MSD is applicable shall be specified in receiver charateristics section.

Considering LTE bands LTE-NR DC:

- The need for BCS for LTE band in LTE-NR DC is discussed separately from NR BCS

- LTE-NR DC UE shall signal BCS information to eNodeB (to reduce changes to LTE UE and eNodeB desing)

- LTE-NR DC UE shall signal support for all specified LTE bandwidth combinations sets that belong to the LTE CA configuration part of LTE – NR DC (to reduce UE fragmentation and make RAN4 specification effort smaller as no LTE BCS information is needed in NR specs)

- When operating in DC band combinations of LTE 1DL/1UL + NR band(s) UE will support all LTE channel bandwidths that are specified to the LTE band (otherwise LTE bandwidth capablility is unknow as single CC operation does not have BCS)

******************************* End of the TP ******************************************