3GPP TSG-RAN WG4 Meeting #84bis	R4-1711373

Dubrovnik, Croatia, 9th – 13th October 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	On aligning the boundary of FR1 NR spectrum emission mask with AAS BS for E-UTRA operating bands ≥ 100 MHz

Agenda item:	9.5.3.2.1

Document for:	Approval

# 1	Introduction

During the last 3GPP RAN4 NR#3 meeting, a way forward document [1] on aligning the boundary of FR1 NR spectrum emission mask with AAS BS for operating bands ≥ 100 MHz was approved. The way forward lists two options for consideration.

In this document, we provide the motivation for our selection.

# 2	Discussion

From [1], the following options are for further studies (other options are not precluded):

Option 1: apply ΔfUEM = 40 MHz to AAS BS Tx requirements (TS 37.105, 37.145-1, 37.145-2) for NR and E-UTRA operation. Consider if ΔfUEM = 40 MHz can be applied to non-AAS BS Tx requirements (TS 36.104, 36.141, 37.104, 37.141)

Option 2: apply ΔfUEM = 40 MHz to AAS BS Tx requirements (TS 37.105, 37.145-1, 37.145-2) for NR and E-UTRA operation and consider to revisit the ΔfUEM = 40 agreement for FR1 E-UTRA refarming bands for NR BS

Table 1 highlights the differences between Option 1 and Option 2.

| Impact on technical specifications | Option 1 | Option 2 |
| --- | --- | --- |
| AAS specifications: TS 37.105, 37.145-1, 37.145-2 | Yes | Yes |
| Non-AAS specifications: TS 36.104, 36.141, 37.104, 37.141 | Yes (this is conditional if it considered necessary according to the second part of the option) | No (this is conditional if it considered necessary according to the second part of the option) |

Table 1: Option 1 versus Option 2

The main selection criterion is that the impact of applying the ΔfUEM = 40MHz extension on technical specifications should be minimized as far as possible.

To recapitulate, our motivation for applying the same ΔfUEM = 40MHz extension of NR to AAS BS is due to a potentially large number of transmitters (i.e., massive MIMO) in the E-UTRA AAS BS. The benefits of such an extension are as follows [2]:

- re-purposing of E-UTRA AAS to NR

- supporting dual-mode E-UTRA AAS and NR radios on the same piece of hardware

- filter design is not constrained by limited space

According to [3], the extension satisfies regulatory requirements.

# 3	Conclusions

Based on the discussion, we propose the following:

Proposal: Option 2 for agreement with the following clarification:

- For AAS type BS, apply ΔfUEM = 40MHz for E-UTRA and NR bands with bandwidth equal to or wider than 100MHz.

- For non-AAS type BS, apply ΔfUEM = 40MHz for new NR bands with bandwidth equal to or wider than 100 MHz wide.

- For non-AAS type BS, apply ΔfUEM = 10MHz for re-farming NR bands with bandwidth equal to or wider than 100 MHz wide.

It should be noted ΔfUEM numbers above are proposed for transmitter requirements only. Document [4] provides our proposal for receiver requirements.

# References

- R4-1710040, WF on aligning the boundary of FR1 NR spectrum emission mask with AAS BS for E-UTRA operating bands ≥ 100 MHz, Nokia, Nokia Shanghai Bell, Ericsson.

- R4-1709821, Discussing Range 1 NR and AAS BS spectrum emission mask requirements for bands ≥ 100 MHz, Nokia, Nokia Shanghai Bell.

- R4-1703094, NR BS boundary between out-of-band and spurious domain below 6 GHz. Ericsson.

- R4-1711420, On blocking requirements for FR1 for bands ≥ 100 MHz, Nokia, Nokia Shanghai Bell.