3GPP TSG-WG RAN4 Meeting NR ad-hoc #3	R4-1711378

# Nagoya, Japan, 18nd – 21st September, 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	Intra-band CA and wideband operation RF requirement

Agenda item:		9.3.1.2

Document for:		Approval

# 1	Introduction

In this paper, we discuss the difference of intra-band CA and wideband operation and propose what the baseline RF requirement should be for each operation.

# 2	Discussion

There has been a long discussion what wideband operation is specified in RAN4 RF requirement but no consensus has been reached so far. The confusions are partly due to insufficient RAN1 agreement until the last meeting so that it was difficult for RAN4 to clearly define the wideband operation. Further, there has been no common understanding on the difference between wideband operation and intra-band carrier aggregation. In this contribution, we propose the RAN4 way-forward aligned with the latest discussion from RAN1 and RAN1 perspective.

First, we consider a similar approach as LTE intra-band contiguous carrier aggregation illustrated in Figure 1. Each component carrier for BS and UE are identical. In LTE, we assume a nominal channel spacing to derive the RF requirement for intra-band contiguous CA and there is a guard band among component carriers.

Figure 1 An example of carrier aggregation scenario

Second, we consider an approach indicated in RAN1 LS [1] where intra-band contiguous CCs can be used within a wideband CC without guard band.

Figure 2 An example of Wide operation scenario

Several contributions pointed out both options are needed and should be specified, although the priority of the second operation was not the same among companies. It is also noted that there is no clear understanding yet what RF requirement is specified for each case and if it is the same or not.

We understand that a CA based RF requirement needs to be developed for the scenario depicted in Figure 1. For this case, the framework to specify RF requirement can be based on the intra-band contiguous CA from E-UTRA UE specifications, in particular for the frequency range 1(FR1).

On the other hand, for the wideband operation depicted in Figure 2, the coexistence requirement (such as ACLR and ACS) should be still based on the wideband CC as the splitting UE component carrier was just to ease the UE implementation but not to change the RF requirement for coexistence. The emission and blocking requirement should be based on the wideband CC requirement. The UE should conform to the single wideband CC requirement even if the aggregations of CCs within the BS carrier is made.

The following table shows the baseline assumptions for wideband operation and carrier aggregation.

Table 1 Overview of wideband operation and carrier aggregation

|  | Wideband operation | Carrier aggregation (intra-band contiguous) |
| --- | --- | --- |
| multiple CCs for BS | No | Yes |
| multiple CCs for UE | Yes | Yes |
| PRB grid | contiguous | non-contiguous based on nominal channel spacing. Nominal channel spacing is FFS. |
| Guard band | zero GB | non-zero GB based on nominal channel spacing. Nominal channel spacing is FFS. |
| channel raster for BS | one raster point | multiple raster points based on nominal channel spacing. Nominal channel spacing is FFS. |
| channel raster for UE | One raster point (or possibly multiple raster points for CCs) | multiple raster points based on nominal channel spacing. Nominal channel spacing is FFS. |
| Support of 100kHz channel raster | No (or FFS if there is interested companies) | Yes |
| Support of subcarrier based raster | Yes | Yes |
| BS Transceiver requirement | single carrier | CA based (LTE intra-band contiguous CA as reference) |
| UE Transceiver requirement | aligned with single carrier RF requirement of the wideband CC | CA based (LTE intra-band contiguous CA as reference) |
| UE Spectrum Utilization | The same as wideband CC (only up to maximum channel bandwidth in Rel-15.) | Per CC based |

# 3	Conclusion

We have discussed the RF requirements for wideband operation and carrier aggregation. The following proposal are derived.

Proposal 1: RF requirements for wideband operation and carrier aggregation are distinguished.

Proposal 2: UE RF requirement for wideband operation shall be specified according to the single (wideband) carrier requirement.

Proposal 3 UE RF requirement for carrier aggregation shall be specified according to E-UTRA CA requirement as a starting point.

# 4	References

- R1-1706615		LS on Wideband Operating Options

- R4-1708849	WF on Further clarification on wideband operation,  LGE, MediaTek, Huawei, Nokia, ZTE

- R4-1708851	WF on spectrum utilization in wideband operation, ZTE

- R4-1709579	Further clarification for Wideband operation of NR UE, LGE

- R4-1709346	Further discussion on wideband operation, Samsung

- R4-1709744	UE spectrum utilization in wideband operation,	Nokia

- R4-1709385	Further consideration on spectrum utilization in wideband operation, ZTE

- R4-1709722	Wideband and CA Operation for NR, Qualcomm

- R4-1709407	On UE bandwidth support in NR, Intel

- R4-1709933	WF on wideband operation for both gNB/UE aspects

- R4-1709075	Way forward on spectrum utilization, Samsung, Qualcomm, Intel, Ericsson, Nokia

- R4-1708850	WF on Channel Raster for NR, Qualcomm, Ericsson, Vodafone, AT&T, Sprint, Verizon, ZTE