3GPP TSG-RAN WG4 #85 Meeting                                     R4-1712158

Reno, Nevada, 27 November to 1 December, 2017

Agenda item:	9.3.3

Source: 	ZTE

Title: 	On how to specify NR spectrum utilization and PRB arrangement for single numerology case

Document for:	Approval

Introduction

In October RAN4#84bis meeting R4-1711736 WF on NR spectrum utilization was approved and here are the open issues have been identified for NR spectrum utilization for single carrier operation and single numerology case [1]:

In this contribution, we give our views on the open issue 3 for how to specify NR spectrum utilization and PRB arrangement for single numerology case.

How to specify NR spectrum utilization and PRB arrangement

In our contribution [2] for this meeting we calculated the guard band for each SCS and channel BW combination according to the latest WF on NR spectrum utilization [1], and had the following proposal for the RB values for the single numerology case in FR1:

Proposal 2 in [2]: For the NR spectrum utilization of single carrier operation and single numerology case for FR1, the tentative agreed RB values in R4-1709075 and R4-1711732 of the following SCS and channel BW combinations (highlighted with yellow color) should be reduced to the values shown in the table below in order to meet the minimum guard band requirement.

| CBW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 30(BS) | 40 | 50 | 60 | 70(BS) | 80 | 90(BS) | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N.A | N.A | N.A | N.A | N.A |
|  | 30 | 10 | 24 | 37 | 51 | 64 | 78 | 106 | 133 | 162 | 189 | 217 | [245] | 273 |
|  | 60 | N.A | 11 | 18 | 23 | 30 | 38 | 51 | 64 | 79 | [92] | 106 | [120] | 134 |

Note 1: Assume the reference SCS is 15 kHz for channel BWs ≤ 50 MHz, and 30 kHz for channel BWs > 50 MHz.

Note 2: Assume the center of all RBs of the reference SCS will be left shifted ∆ ≤ ½ reference SCS away from the center of the channel BW.

Note 3: Assume RAN1 subcarrier 0 alignment agreement across different SCSs in a channel BW. No constraints on aligning PRB #0 across different SCSs.

The open issue 3 in [1] is about where and how to capture the PRB arrangement in NR specification(s). In order to understand the exact RB positions of each SCS in a channel BW, from NR specification(s) a BS or UE needs to know more information than just the RB values in the table above. Our view is that in the NR spectrum utilization table, the following information could be added to capture the PRB arrangement if reference SCS term is assumed to be used in NR specifications:

- Which SCS is the reference SCS, and RB position indication of the reference SCS;

RB position indication of the other SCSs, e.g. the RB#0 of a SCS is aligned to which RB of the reference SCS.

Proposal 1: RAN4 NR specifications (e.g. TS 38.104 and TS 38. 101) will capture PRB arrangement for all SCSs of each channel BW for the single numerology case.

According to the RB values in the table above, we arranged all RBs of all SCSs in such as way that the difference between the left guard band and right guard band is minimized as suggested by [1] and the results are summarized in the Appendix. Based on the results we have the following proposal for how to specify PRB arrangement and NR spectrum utilization together:

Proposal 2: If reference SCS term is assumed to be used in NR specifications, NR spectrum utilization and PRB arrangement could be specified as the table below for the single numerology case for FR1.

| CBW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 30(BS) | 40 | 50 | 60 | 70(BS) | 80 | 90(BS) | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N.A | N.A | N.A | N.A | N.A |
|  | 30 | 10 RB3 | 24 RB2 | 37 RB3 | 51 RB2 | 64 RB3 | 78 RB2 | 106 RB2 | 133 RB2 | 162 | 189 | 217 | [245] | 273 |
|  | 60 | N.A | 11 RB4 | 18 RB5 | 23 RB8 | 30 RB7 | 38 RB4 | 51 RB6 | 64 RB8 | 79 RB2 | [92] [RB3] | 106 RB3 | [120] [RB3] | 134 RB3 |

Note 1: It’s assumed that the reference SCS is 15 kHz for channel BWs ≤ 50MHz, and 30 kHz for channel BWs > 50MHz.

Note 2: It’s assumed that the center of all RBs of the reference SCS will be left shifted ∆ ≤ ½ reference SCS away from the center of the channel BW. The exact shift will be defined after the channel raster related designs are finalized.

Note 3: RB# highlighted in yellow colour indicates that the RB#0 of the SCS is aligned to which RB of the reference SCS. Here it’s assumed that the RBs of the reference SCS are numbered from 0, 1, 2, etc.

Conclusions

Based on the discussions in this contribution, we have the following Observation and Proposal:

Proposal 1: RAN4 NR specifications (e.g. TS 38.104 and TS 38. 101) will capture PRB arrangement for all SCSs of each channel BW for the single numerology case.

Proposal 2: If reference SCS term is assumed to be used in NR specifications, NR spectrum utilization and PRB arrangement could be specified as the table below for the single numerology case for FR1.

| CBW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 30(BS) | 40 | 50 | 60 | 70(BS) | 80 | 90(BS) | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N.A | N.A | N.A | N.A | N.A |
|  | 30 | 10 RB3 | 24 RB2 | 37 RB3 | 51 RB2 | 64 RB3 | 78 RB2 | 106 RB2 | 133 RB2 | 162 | 189 | 217 | [245] | 273 |
|  | 60 | N.A | 11 RB4 | 18 RB5 | 23 RB8 | 30 RB7 | 38 RB4 | 51 RB6 | 64 RB8 | 79 RB2 | [92] [RB3] | 106 RB3 | [120] [RB3] | 134 RB3 |

Note 1: It’s assumed that the reference SCS is 15 kHz for channel BWs ≤ 50MHz, and 30 kHz for channel BWs > 50MHz.

Note 2: It’s assumed that the center of all RBs of the reference SCS will be left shifted ∆ ≤ ½ reference SCS away from the center of the channel BW. The exact shift will be defined after the channel raster related designs are finalized.

Note 3: RB# highlighted in yellow colour indicates that the RB#0 of the SCS is aligned to which RB of the reference SCS. Here it’s assumed that the RBs of the reference SCS are numbered from 0, 1, 2, etc.

# References

- R4-1711736 WF on NR spectrum utilization, ZTE, October 2017.

- R4-1712157 Guard band calculations for NR spectrum utilization in single numerology case, ZTE, November 2017.

# Appendix

#### A1.1 Subcarrier 0 and RB alignment among all SCSs of a channel BW for FR1