3GPP TSG RAN WG4 Meeting #85	R4-1712299

Reno, US, 27 November - 1 December 2017

Agenda item:	9.7.10.1

Title: 	Simulation results for NR-PBCH Decoding

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4 AH NR#3 the simulation assumptions for PBCH decoding were agreed up on to derive the basic SI reading time [1]. Based on the agreements, companies were encouraged to provide simulation results for PBCH acquisition for 2 cases.

The proposed simulation assumptions are used to derive the basic SI reading (PBCH acquisition) time. Case1: PBCH reading Basic SI reading time = the number of SS-blocks required for 99% to successfully decode the PBCH. Case2: PBCH-DMRS time index reading (Without decode PBCH data) DMRS time index reading time = the number of SS-blocks required for 99% to successfully reading DMRS time index.

In this contribution we present results for PBCH acquisition and discuss our views.

# Simulation Results

As agreed in the simulation assumption, the PBCH acquisition time for PBCH-DMRS time index reading and PBCH reading will be used to derive the basic SI reading time.

## Simulation Assumptions

Table 1: Parameters for basic SI reading (PBCH acquisition)

| Parameter | Unit | Value |
| --- | --- | --- |
| Carrier frequency | GHz | 4GHz; 30GHz |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; 120 kHz; 240 kHz for 30GHz |
| Number of Tx antennas | - | 1 Assuming only 1Tx port is used |
| Number of Rx antennas | - | 2, 4 (uncorrelated,Without receiver beamforming) |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 20 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| RB Utilization | % | 0 |
| PBCH symbols within the SS block |  | PSS-PBCH-SSS-PBCH |
| Data and Control Power offset with respect to PSS and SSS | dB | 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | 0 |
| PSS and SSS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS RE positions within the PBCH resource | - | According to RAN1 agreements |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit |
| PBCH SNR | dB | -12 : 0 dB, at least including -6,-8 dB |
| Propagation Condition | - | EPA5, ETU70 for 4GHz TDL-C with desired RMS delay spread 100ns Note1 for 30GHz |
| UE speed and Doppler Shift | - | 3km/h speed with 83Hz Doppler at 30GHz |
| Detection Method |  | Based on one shot detection (No combination for different PBCHs,) |
| NOTE 1: The channel models of CDL and TDL are used for simplified link level evaluations, which are defined in TR 38.900. |  |  |

## DMRS Time Index Acquisition

For the purpose of determining cell identification delay requirements, the DMRS time index reading time should be based on the same simulation assumptions as PSS/SSS detection as captured in our paper [2].

Proposal #1: Align simulation assumptions for DMRS time index reading and PSS/SSS detection to define cell identification requirements

## NR-PBCH Decoding

The basic SI reading time is the number of SS-Blocks required to decode PBCH with 99% success rate. The PBCH decoding results are for single-shot PBCH decoding without any soft-combining.

The PBCH decoding time in number of SS-Blocks for 4GHz are captured in Table 3.

Table 3: PBCH decoding time for 4GHz

|  | SCS | Channel | SNR: -12 dB | SNR: -10 dB | SNR: -8 dB | SNR: -6 dB | SNR: -4dB | SNR: -2dB | SNR: 0dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx | 15KHz | EPA5 | >10 | 5 | 3 | 2 | 2 | 1 | 1 |
|  |  | ETU70 | >10 | 8 | 3 | 2 | 1 | 1 | 1 |
|  | 30KHz | EPA5 | >10 | 5 | 3 | 2 | 2 | 1 | 1 |
|  |  | ETU70 | >10 | 10 | 4 | 2 | 1 | 1 | 1 |
| 4Rx | 15KHz | EPA5 | 4 | 2 | 1 | 1 | 1 | 1 | 1 |
|  |  | ETU70 | 6 | 2 | 1 | 1 | 1 | 1 | 1 |
|  | 30KHz | EPA5 | 4 | 2 | 1 | 1 | 1 | 1 | 1 |
|  |  | ETU70 | 10 | 3 | 1 | 1 | 1 | 1 | 1 |

Observation #1: For baseline case of 2Rx antenna, the PBCH acquisition time at 4GHz is 4 SS-Blocks at -8dB SNR across all scenarios

Observation #2: For baseline case of 2Rx antenna, the PBCH acquisition time at 4GHz is 2 SS-Blocks at -6dB SNR

The PBCH decoding time in number of SS-Blocks for 30GHz in TDL-C channel are captured in Table 4.

Table 4: PBCH decoding time for 30GHz

|  | SCS | SNR: -12 dB | SNR: -10 dB | SNR: -8 dB | SNR: -6 dB | SNR: -4dB | SNR: -2dB | SNR: 0dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx | 120KHz | >10 | 6 | 3 | 2 | 1 | 1 | 1 |
|  | 240KHz | >10 | 8 | 3 | 2 | 1 | 1 | 1 |

Observation #3: At very low SNR, < -10dB, the acquisition time with 240 KHz SCS is larger than that with 120 KHz

Observation #4: At -8 dB SNR, for 30GHz the PBCH acquisition time is 3 SS-Blocks

Observation #5: At -6 dB SNR, for 30GHz the PBCH acquisition time is 2 SS-Blocks

# Conclusions

In this paper we present the PBCH Decoding delay for 4GHz and 30GHz and have the following observations.

Observation #1: For baseline case of 2Rx antenna, the PBCH acquisition time at 4GHz is 4 SS-Blocks at  -8dB SNR across all scenarios

Observation #2: For baseline case of 2Rx antenna, the PBCH acquisition time at 4GHz is 2 SS-Blocks at  -6dB SNR

Observation #3: At very low SNR, < -10dB, the acquisition time with 240 KHz SCS is larger than that with 120 KHz

Observation #4: At -8 dB SNR, for 30GHz the PBCH acquisition time is 3 SS-Blocks

Observation #5: At -6 dB SNR, for 30GHz the PBCH acquisition time is 2 SS-Blocks

In addition we propose that:

Proposal #1: Align simulation assumptions for DMRS time index reading and PSS/SSS detection to define cell identification requirements

It is suggested that these results be considered while defining requirements in RAN4.

# References

- R4-1709886, “Updated PBCH Simulation assumption”, Huawei, HiSilicon

- R4- 1712298, “Simulation results for Cell Detection in NR”, Intel Corporation