3GPP TSG-WG RAN4 Meeting #85	R4-1712539

# Reno, Nevada, USA 27th Nov – 1st Dec, 2017

Source: 	Nokia, Nokia Shanghai Bell, AT&T

Title: 					TP to TR 36.715-04-01: CA_4DL_2A-2A-46C_1UL_BCS0

Agenda Item: 			8.4.2

Document for:	Approval

# Introduction

This TP introduces 4DL/1UL CA configuration CA_2A-2A-46C in to the TR 36.715-04-01

**************************** Start of changes ********************************************

## 5.X	CA_2A-2A-46C_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-2A-46C | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 80 | 0 |
|  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

### 5.X.2	Co-existence studies

Table 5.X.2-1 summarizes frequency ranges where harmonics occur due to Band 2 and Band 46 CA with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 3700 | 3820 | 5550 | 5730 |
| 46 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 |

It can be seen from Table 5.X.2-1 that the second harmonic of band 66 falls in band 48.

### 5.X.3	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band 2 and Band 46 for 1UL/4DL CA_2A-2A-46C, the TIB,c and RIB,c values are shown in Table 5.X.3-1 and Table 5.X.3-2, respectively, as derived from CA_2A-46A.

Table 5.X.4-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-2A-46C | 2 | 0 |

Table 5.X.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-2A-46C | 2 | 0 |

### 5.X.4	REFSENS requirements

Reference sensitivity for CA_2A-2A-46C are specified in Table 5.X.4-1 for the uplink configuration specified in Table 7.3.1-2 and Table 7.3.1A-0eC of TS36.101. The exclusion zones for band 46 due to harmonics are specified in Table 7.3.1A-0eC of TS36.101.

Table 5.X.4-1: Reference sensitivity QPSK PREFSENS (CA with band 46)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-2A-46C | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |

**************************** End of changes ********************************************