3GPP TSG-RAN WG4 RAN4-85	R4-1712727

Reno, United States, 27 November - 1 December, 2017

Source:		Ericsson

Title:			TP to section 8.2 (Radiated Emission) in TS 38.113.

Agenda item:		9.6.2.1

Document for:	Approval

# Introduction

A text proposal to specify the radiated emission limits to be considered for BS 1C and 1H in TS 38.113 [1] is presented in this document.

# Discussion

During the meeting in Dubrovnik, an initial text of TS 38.113 [1] was approved. Considering this initial agreement and the skeleton approved for TS 38.113, we propose in this contribution a TP for TS 38.113 on Radiated Emission limits (Section 8.2).

Proposal 1: Agree on the TP to Section 8.2 in TS 38.113 (NR) on emission limits to be considered for BS 1C and 1H attached to this contribution.

# Conclusion

Based on the above discussion, it is proposed to agree on TP to Section 8.2 in TS 38.113 (NR) attached to this contribution.

# References

3GPP TS 38.113 V.0.0.2, Base Station (BS)  ElectroMagnetic Compatibility (EMC) (Release 15)

# TP to the draft TS 38.113

In this section, TP to Section 4.4 in TS 38.113 [1] is proposed for approval.

- ------------------------------ Proposed Text ------------------------------

## 8.2	Radiated emission

### 8.2.1	Radiated emission, BS

This test is applicable to BS type 1-C and BS type 1-H. This test shall be performed on a representative configuration of BS.

For BS type 1-O and BS type 2-O, the radiated emission is covered by radiated spurious emission requirement in TS 37.105 [2], conforming to the test requirement in TS 37.145-2 [4].

#### 8.2.1.1	Definition

This test assesses the ability of BS to limit unwanted emission from the enclosure port.

#### 8.2.1.2	Test method

<text to be added>

#### 8.2.1.3   Limits

The frequency boundary and reference bandwidths for the detailed transitions of the limits between the requirements for out of band emissions and spurious emissions are based on ITU-R Recommendations SM.329 [24] and SM.1539 [26].

The BS type 1-C and BS type 1-H shall meet the limits below:

Table 8.2.1.3-1: Limits for radiated emissions from BS

| Frequency range | Minimum requirement (e.r.p.)/Reference Bandwidth |
| --- | --- |
| 30 MHz f < 1000 MHz | -36 dBm/100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm/ 1 MHz |
| [FBW RF,low - 10 MHz < f < FBW RF,high + 10 MHz (Note)] | Not defined |
| NOTE: [For BS capable of multi-band operation, the frequency ranges relating to the RF bandwidths of all supported bands apply.] |  |

------------------------------ End of proposed text ------------------------------