3GPP TSG-RAN WG4 Meeting #85	R4-1712778

Reno, NV, USA, 27 November – 1 December 2017

Agenda Item:	7.2.3

Source:	Ericsson

Title:	SI acquisition time requirements for non-BL CE UE

Document for:	Discussion

# 1	Introduction

RAN#83 agreed with the way forward on the SI acquisition time for non-BL UE as follows [1]:

- RAN4 will consider shortening the SI acquisition time testing requirement for non-BL CE Mode B UE assuming 2Rx in Rel-14.

- RAN4 will consider how to clarify the SI acquisition time requirement for non-BL CE Mode B UE assuming 2Rx in Rel-14 in the core requirement.

- Companies are encouraged to propose the SI acquisition time for non-BL CE Mode B UE based on the practical simulation results in RAN4#84.

In RAN4#84bis, it was also agreed that [2]:

Proposal 2: If the test case design is needed, the SI acquisition delay can be defined with reduced value.

This contribution discusses the SI acquisition time requirements for Rel-14 non-BL CE UEs based on the agreement above.

# 2	Discussion

## 2.1	 CGI reading delay

CGI reading delay requirement includes the MIB and SIB1 acquisition times for the signalled physical cell ID. For eMTC, the CGI reading delay is specified only for CE Mode B and it is set to 5,120ms. Since the requirements are derived based on single receive antenna, we expect the MIB acquisition time and SIB1-BR acquisition time can be improved with 2 receive antenna assumption for non-BL UE in Rel-14.

Table 1 and Table 2 compare the MIB acquisition time and the SIB1-BR acquisition time between 1Rx UE and 2Rx UE. The comparison assumes SNR=-15dB, EPA1/ETU1 and 95% of decoding success rate. The reason we chose 95% was the CGI requirement is the combination of MIB and SIB1-BR acquisition and the requirement is 90% success rate. The detailed simulation parameters are given in [3].

Table 1	MIB acquisition time improvement for 1Rx UE and 2Rx UE.

|  | MIB acquisition time (1Rx UE, 95% success rate, SNR=-15dB) | MIB acquisition time (2Rx UE 95% success rate, SNR=-15dB) |
| --- | --- | --- |
| EPA1 | 760ms | 280ms |
| ETU1 | 760ms | 240ms |

Table 2	SIB1-BR acquisition time improvement for 1Rx UE and 2Rx UE.

|  | SIB1-BR acquisition time (1Rx UE, 95% success rate, SNR=-15dB) | SIB1-BR acquisition time (2Rx UE, 95% success rate, SNR=-15dB) |
| --- | --- | --- |
| EPA1 | 960ms | 240ms |
| ETU1 | 1,520ms | 320ms |

Table 3 shows the expected CGI reading delay reduction, and it is expected the delay is reduced by 70-75% from our simulation results. Considering the existing reading time 5,120ms for BL/CE UE, the CGI reading delay can be reduced to 1,600ms with two received antennas.

Proposal 1: CGI reading time delay can be reduced from 5,120ms to 1,600ms for non-BL UE.

Table 3	Expected CGI reading time reduction due to the enhanced MIB/SIB acquisition.

|  | CGI reading delay with 1Rx UE | CGI reading delay with 2Rx UE | CGI reading delay reduction |
| --- | --- | --- | --- |
| EPA1 | 760+960 = 1,720ms | 280+240 = 520ms | 70% |
| ETU1 | 760+1520 = 2,280ms | 240+320 = 560ms | 75% |

## 2.1.2	Requirements affected by SI acquisition

TS36.133 specifies TSI-EUTRA-M1-CEModeB for Cat-M1 CE Mode B or enhanced coverage as the time required for receiving all the relevant system information data, which include MIB and relevant SIBs (e.g., SIB1-BR and SIB2). In the current specification, it is set to 6,400ms in the test case. Since it could be improved with two received antennas and the network can distinguish between BL/CE UE (e.g., UE DL category M1 or M2) and non-BL CE UE (e.g. UE category combined with coverage enhancement capability signalling), we propose to introduce a new variable such as TSI-EUTRA-nonBL-CEModeB in the requirements for RRC re-establishment, Cell selection, or Handover, but to set shorter value in the test case.

Proposal 2: For the requirements of RRC re-establishment, Cell selection, or Handover, it is proposed to define a new term referring to the system information acquisition time TSI-EUTRA-nonBL-CEModeB for non-BL UE, and to set a shorter value in the corresponding test cases.

Regarding the actual time in the test case, when we apply the same scaling we have done for CGI delay requirements, i.e., reduction by 70%, TSI-EUTRA-nonBL-CEModeB = 1,920ms for non-BL CE Mode B UE.

Proposal 3: RAN4 set TSI-EUTRA-nonBL-CEModeB = 1,920ms for non-BL CE Mode B UE.

# 3	Conclusion

Proposal 1: CGI reading time delay can be reduced from 5,120ms to 1,600ms for non-BL UE.

Proposal 2: For the requirements of RRC re-establishment, Cell selection, or Handover, it is proposed to define a new term referring to the system information acquisition time TSI-EUTRA-nonBL-CEModeB for non-BL UE, and to set a shorter value in the corresponding test cases.

Proposal 3: RAN4 set TSI-EUTRA-nonBL-CEModeB = 1,920ms for non-BL CE Mode B UE.

# 4	Reference

R4-1706183, “Way forward on SI acquisition time requirements for non-BL/CE UE”, Ericsson.

R4-1710154, “Discussion on the SI acquisition requirement for R14 non-BL/CE UE”, Intel.

R4-1710796, “SI acquisition time for non-BL CE UE”, Ericsson.

# Appendix

Table 4	Simulation assumption for MIB/SIB1-BR acquisition time.

| Parameters | CE Mode B |
| --- | --- |
| System bandwidth | 10MHz |
| Transmit antennas | 2 |
| Propagation channel | EPA1, ETU1 |
| TBS for SIB1-BR | 208 bits |
| SIB1-BR repetition number | 16 |
| PBCH repetition | Enabled |
| UE receive antenna | 2 |
| UE frequency offset | 100 Hz |
| Evaluation criteria | 95% MIB decoding success rate at SNR=-15dB 95% SIB1-BR decoding success rate at SNR=-15dB |

Table 5	Simulation assumption for SIB2-BR acquisition time.

| Parameters | CE Mode B |
| --- | --- |
| System bandwidth | 10MHz |
| Transmit antennas | 2 |
| Propagation channel | EPA1, ETU1 |
| si-TBS | 208 bits |
| si-WindowLength-BR | 40ms |
| si-RepetitionPattern | everyRF |
| si-Periodicity | 160ms (rf16) |
| UE receive antenna | 2 |
| UE frequency offset | 100 Hz |
| Evaluation criteria | 95% SIB2-BR decoding success rate at SNR=-15dB |