3GPP TSG-RAN WG4 Meeting #85	R4-1713516

Reno, Nevada, USA, 27 Nov-1 Dec 2017

Source: 	Ericsson

Title:  	TP to TR 38.133v0.3.0 (2017-11): Further definitions of synchronous and asynchronous Dual connectivity in Rel-15 LTE-NR combinations

Agenda Item:	9.7.6.2

Document for:	Approval

# Background

An updated version of the TR 38.133 v0.3.0 (2017-11) has been circulated in [1]. We provide this TP based on this latest version of the TS.

In RAN4#84bis, we have agreed a way forward regarding synchronous and asynchronous requirements for LTE-NR dual connectivity [2]. In [1], the agreements related to the way forward has been documented in 38.133.

In our companion contribution, we have provided further discussions in [3], where we have provided further proposals on remaining requirements. We provided the updates in this TP for TS 36.133.

# Proposal

We propose to adopt the text in this contribution to TR 38.133.

# References

R4-1711985, TR 38.133 v0.3.0 (2017-11), Source: Intel, updated draft circulated in RAN4 mailing list on 13 November 2017.

R4-1711964, WF on synchronous and asynchronous definitions for LTE-NR DC in Rel-15, Ericsson, MediaTek

R4-17xxxxx, “Further discussions on synchronous and asynchronous Dual connectivity in Rel-15 LTE-NR combinations”, Ericsson

# Text proposal for TR 38.133

# <Start of Text proposal>

## 7.5	Maximum Transmission Timing Difference

### 7.5.1	Introduction

A UE shall be capable of handling a relative transmission timing difference between subframe timing boundary of PCell and slot timing boundaries of PSCell to be aggregated E-UTRA-NR dual connectivity.

### 7.5.2	Minimum Requirements for E-UTRA-NR Dual Connectivity

For inter-band LTE-NR dual connectivity, the UE shall be capable of handling a maximum uplink transmission timing difference between PCell and PSCell as shown in Table 7.5.2-1.

Table 7.5.2-1 Maximum uplink transmission timing difference requirement for asynchronous operation

| Sub-carrier spacing in LTE PCell (kHz) | UL Sub-carrier spacing for data in NR PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120Note1 | 62.5 |
| Note1 : For intra-band FDD-FDD LTE-NR dual connectivity, 120kHz is not applied. |  |  |

For inter-band TDD-TDD (PCell-PSCell) and inter-band TDD-FDD (PCell-PSCell or PSCell-PCell) combinations, the UE shall meet the requirements in Table 7.5.2-1 provided that the UE indicates that it is capable of asynchronous dual connectivity [TS 36.331].

For inter-band TDD-TDD (PCell-PSCell) and inter-band TDD-FDD (PCell-PSCell or PSCell-PCell) combinations, the UE shall meet the requirements in Table 7.5.2-2 provided that the UE indicates that it is capable of synchronous dual connectivity only [TS 36.331].

Table 7.5.2-2 Maximum uplink transmission timing difference requirement for synchronous operation in inter-band TDD-TDD and TDD-FDD combinations

| Sub-carrier spacing in LTE PCell (kHz) | UL Sub-carrier spacing for data in NR PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 35.21 |
| 15 | 30 |  |
| 15 | 60 |  |
| 15 | 120 |  |

For intra-band FDD-FDD LTE-NR dual connectivity with collocated deployment, the UE shall be capable of handling a maximum uplink transmission timing difference between PCell and PSCell as shown in Table 7.5.2-1 provided the UE indicates that it is capable of asynchronous dual connectivity [TS 38.331].

For intra-band TDD-TDD LTE-NR dual connectivity with collocated deployment, only synchronous and collocated operation is allowed, thus no uplink transmission timing difference is applicable.

## 7.6	Maximum Receive Timing Difference

### 7.6.1	Introduction

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of PCell and slot timing boundaries of PSCell to be aggregated for E-UTRA-NR dual connectivity.

### 7.6.2	Minimum Requirements for E-UTRA-NR Dual Connectivity

For inter-band LTE-NR dual connectivity, the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from PCell and slot timing of signal from PSCell at the UE receiver as shown in Table 7.6.2-1 below.

Table 7.6.2-1 Maximum receive timing difference requirement for asynchronous operation

| Sub-carrier spacing in LTE PCell (kHz) | DL Sub-carrier spacing in NR PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120 | 62.5 |
| Note1 : DL Sub-carrier spacing is min{SCSSS, SCSDATA}. Note2 : For intra-band FDD-FDD LTE-NR dual connectivity, 120kHz is not applied. |  |  |

For inter-band TDD-TDD (PCell-PSCell) and inter-band TDD-FDD (PCell-PSCell or PSCell-PCell) combinations, the UE shall meet the requirements in Table 7.6.2-1 provided that the UE indicates that it is capable of asynchronous dual connectivity [TS 36.331].

For inter-band TDD-TDD (PCell-PSCell) and inter-band TDD-FDD (PCell-PSCell or PSCell-PCell) combinations, the UE shall meet the requirements in Table 7.6.2-2 provided that the UE indicates that it is capable of synchronous dual connectivity only [TS 36.331].

Table 7.6.2-2 Maximum receive timing difference requirement for synchronous operation in inter-band TDD-TDD and TDD-FDD combinations

| Sub-carrier spacing in LTE PCell (kHz) | DL Sub-carrier spacing in NR PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 33 |
| 15 | 30 |  |
| 15 | 60 |  |
| 15 | 120 |  |
| Note1 : DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

For intra-band FDD-FDD LTE-NR dual connectivity with collocated deployment, the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from PCell and slot timing of signal from PSCell as shown in Table 7.6.2-1 provided the UE indicates that it is capable of asynchronous dual connectivity [TS 36.331].

For intra-band LTE-NR dual connectivity with collocated deployment, the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from PCell and slot timing of signal from PSCell as shown in Table 7.6.2-3 provided the UE indicates that it is only capable of synchronous dual connectivity [TS 36.331].

Table 7.6.2-3 Maximum receive timing difference requirement for synchronous operation in intra-band collocation scenario

| Sub-carrier spacing in LTE PCell (kHz) | DL Sub-carrier spacing in NR PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 4.76 |
| 15 | 30 | 2.38 |
| 15 | 60 | 1.19 |
| Note1 : DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

# <End of Text proposal>