3GPP TSG-RAN WG4 Meeting #85	R4-1713517

Reno, Nevada, USA, 27 Nov-1 Dec 2017

Source: 	Ericsson

Title:  	TP to TR 38.817-01 v0.2.0 (2017-11): Definition of synchronous and asynchronous Dual connectivity in Rel-15 LTE-NR combinations

Agenda Item:	9.7.6.2

Document for:	Approval

# Background

An updated version of the TR 38.817-01 has been circulated in [1]. We provide this TP based on this latest version of the TR.

In RAN4#84bis, we have agreed a way forward regarding synchronous and asynchronous requirements for LTE-NR dual connectivity [2]. In this TP, we provide texts to reflect this agreement such that the requirements are documented in the TR.

# Proposal

We propose to adopt the text in this contribution to TR 38.817-01.

# References

R4-17xxxxx, TR 38.817-01 v0.2.0 (2017-11): General aspects for UE RF for NR, NTT Docomo, updated draft circulated in RAN4 mailing list on 24 October 2017.

R4-1711964, WF on synchronous and asynchronous definitions for LTE-NR DC in Rel-15, Ericsson, MediaTek

# Text proposal for TR 38.817-01

# <Start of Text proposal>

# 4 General and common aspects

The main text of the document should start here, after the above clauses have been added.

The following styles and editing techniques are aimed to help in the formatting of the document using the 3GPP Template: 3GPP_70.dot, available from the 3GPP FTP site (ftp://ftp.3gpp.org/Information).

## << Texts not mentioned here from Section 4.1 to Section 4.6 >>

## 4.7	Definition of synchronous and asynchronous Dual connectivity in Rel-15 LTE-NR combinations

There are several issues related to deployment for LTE-NR dual connectivity, that should be taken into consideration while deciding synchronous and asynchronous requirements for LTE-NR dual connectivity.

- LTE-NR dual connectivity deployment should provide enough flexibility so that cells in different spectrum range can be aggregated together. For example, in a real network, a very wide area cell in below 1 GHz can be aggregated with a mmWave small cell, thus requiring that, the UE should be able to handle as large relative propagation delay difference as possible. This will provide significant flexibility to the operators in terms of deployment options and aggregation options.

- NR has several new features compared to LTE, such as different sub-carrier spacing, etc, which provides new opportunities compared to LTE networks. While interworking between LTE and NR is required to some extent, extending LTE DC requirements as they are to NR DC requirements may cause the loss of this above mentioned flexibility.

- LTE and NR will be implemented in the UE using significantly different design principle and requirements, thus, it will not be suitable to consider synchronization between LTE and NR network in most cases. Moreover, in an UE, preferably different RF chains and baseband processing will be used for LTE and NR chains. Else, if NR is implemented using many of LTE designs, NR will not be able to achieve its full potentials. Thus, we propose not to consider synchronous LTE-NR dual connectivity as a general case.

Thus, it is indeed important to use asynchronous operation as the baseline for LTE-NR dual connectivity. This will provide wide flexibility to the operators in terms of deployment and aggregation possibilities.

Moreover, SFN synchronization between LTE and NR cells are also not needed in LTE-NR dual connectivity operation. This is similar to Rel-12 LTE DC framework.

Following is agreed with respect to LTE-NR DC:

- For inter-band LTE-NR dual connectivity, all UE supports asynchronous DC between LTE and NR. Asynchronous here refers to subframe/slot timing boundary alignment.

- For inter-band LTE-NR dual connectivity, following MRTD and MTTD is defined as described in Table 4.7-1.

Table 4.7-1: Requirements for asynchronous requirements for inter-band LTE-NR dual connectivity

| LTE PCell SCS (kHz) | NR PSCell SCS (kHz) | Maximum receive timing difference, MRTD (µs) for asynchronous operation | Maximum transmission timing difference, MTTD (µs) for asynchronous operation |
| --- | --- | --- | --- |
| 15 | 15 | 500 | 500 |
| 15 | 30 | 250 | 250 |
| 15 | 60 | 125 | 125 |
| 15 | 120 | 62.5 | 62.5 |

- For Inter-band TDD-TDD (PCell-PSCell) and inter-band TDD-FDD (PCell-PSCell or PSCell-PCell) combinations,

  - Simultaneous RXTX in DC operation can be supported based on UE Capability.

- For Intra-band TDD-TDD case:

  - Synchronous DC operation as well as NR DL/UL resource assignment to avoid DL/UL interference with LTE TDD is needed for intra-band LTE-NR DC.

  - It is proposed to only consider collocation scenario for intra-band LTE-NR DC in Rel-15.

  - UE shall support the synchronous TDD-TDD LTE-NR intra-band DC provided the MRTD at the UE does not exceed values shown in Table 4.7-2.

  - Table 4.7-2: Requirements for synchronous requirements for intra-band TDD-TDD LTE-NR dual connectivity

| LTE PCell SCS (kHz) | NR PSCell SCS (kHz) | Maximum receive timing difference, MRTD (µs) for synchronous operation | Maximum receive timing difference, MRTD (µs) for asynchronous operation |
| --- | --- | --- | --- |
| 15 | 15 | TBD | NA |
| 15 | 30 | TBD | NA |
| 15 | 60 | TBD | NA |

- For FDD-FDD Intra-band LTE-NR DC,

  - It is proposed to only consider collocation scenario for intra-band LTE-NR DC in Rel-15.

  - If UE supports asynchronous dual connectivity assuming separate RF chains, following MRTD and MTTD is defined in this case as described in Table 4.7-3.

Table 4.7-3: Requirements for asynchronous requirements for intra-band FDD-FDD LTE-NR dual connectivity

| LTE PCell SCS (kHz) | NR PSCell SCS (kHz) | Maximum receive timing difference, MRTD (µs) for asynchronous operation | Maximum transmission timing difference, MTTD (µs) for asynchronous operation |
| --- | --- | --- | --- |
| 15 | 15 | 500 | 500 |
| 15 | 30 | 250 | 250 |
| 15 | 60 | 125 | 125 |

# <End of Text proposal>