3GPP TSG-RAN WG4 Meeting #85	R4-1713526

Reno, Nevada, USA, 27th November- 1st December 2017

Source: 	Ericsson

Title:  	TP to TR 38.104v0.4.0: Absolute levels for FR2 ACLR absolute levels for NR BS

Agenda Item:	9.5.2.5.2.2

Document for:	Approval

# Background

A new version of TS 38.104 has been approved in [1].

In the past in RAN4, extensive discussions have been done in RAN4 with respect to inband receiver blocking for FR2 NR BS. We have proposed a methodology which considers joint probability of wanted and interferer while considering the receiver blocking for FR2 NR BS. We presented our analysis in [2] and [3].

# Proposal

We propose to adopt the text in this contribution to TR 38.104.

# References

R4-1711971, TR 38.104 v0.4.0: Draft TS 38.104 v 0.4.0

R4-1706811, The importance for considering joint probability for receiver blocking in NR BS, Ericsson

R4-1708145, Consideration of joint probability for wanted and interference signals in mmWave NR BS receiver blocking investigation, Ericsson

# Text proposal for TR 38.104 v0.4.0

# <Start of Text proposal>

### 9.7.3	OTA Adjacent Channel Leakage Power Ratio (ACLR)

#### 9.7.3.1	General

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

The requirement applies during the transmitter ON period.

#### 9.7.3.2	Minimum requirement for BS type 1-O

Minimum  requirement  is defined based on basic limit.

The OTA ACLR limits  are specified as basic limits in Table 6.6.3.2-1 in subclause 6.6.3.2.

The OTA ACLR absolute limits are based on emissions scaling, where the emissions limits are defined as:

basic limit + 9 dB.

The OTA ACLR limit or the OTA ACLR absolute limit , whichever is less stringent, shall apply outside the Base Station RF Bandwidth or Radio Bandwidth.

For a multi-band RIB, the OTA ACLR requirement applies for the first adjacent channel inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 15MHz. The OTA ACLR requirement for the second adjacent channel applies inside any Inter RF Bandwidth gap with a gap size Wgap ≥ 20 MHz.

#### 9.7.3.3	Minimum requirement for BS type 2-O

Minimum  requirement for OTA ACLR limits  is specified in Table 9.7.3.3-1.

Minimum requirement for OTA ACLR absolute limits is specified in Table 9.7.3.3-2.

The OTA ACLR limit in Table 9.7.3.3-1 or the ACLR absolute limit   in Table 9.7.3.3-2 or spectral emission mask as defined in Section 9.7.4.3.2 shall apply, whichever is less stringent.

Table 9.7.3.3-1: BS type 2-O ACLR limits

| NR Channel bandwidth | BS adjacent channel centre frequency offset below the lower or above the upper Base Station RF bandwidth edge | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| [50, 100, 200, 400, …] | BWChannel | NR of same BW | NR of same BW with SCS that provides largest transmission bandwidth configuration (BWConfig) | 28 (Note 1) 26 (Note 2) |
| Note 1: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz Note 2: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz |  |  |  |  |

Table 9.7.3.3-2: BS type 2-O ACLR absolute limits

| Frequency range | BS maximum output power | ACLR absolute limit (Note 1) |
| --- | --- | --- |
| 24.24 GHz – 33.4 GHz | ≥ 35 dBm | -13 dBm/MHz |
|  | < 35 dBm | [-20 dBm/MHz] |
| 37 GHz – 52.6 GHz | ≥ 33 dBm | -13 dBm/MHz |
|  | < 33 dBm | [-20 dBm/MHz] |
| Note 1: In the measurement bandwidth, exclude 0 f < 10% of the total transmission bandwidth as specified in Section 9.7.4.3.2, where f is defined as Frequency offset from “edge of transmission”. |  |  |

Editor’s note: The draft requirement for BS type 2-O is a carrier centric spectrum emissions mask and relies on the present levels of spurious emission limits documented in 9.7.5. The type of emission mask and the frequency range it covers may need to be revised in case other emission limits are changed.

# <End of Text proposal>