3GPP TSG RAN WG4 Meeting #85                         R4-1713672

Reno, USA, Nov 27th-Dec 1st, 2017

Source: 	NEC

Title:		      Proposal on NR BS EVM Requirement for FR1

Agenda Item:    9.5.2.4.1.1

Document for:   Approval

- Introduction

In the last RAN4 meeting, a WF [1] on simulation assumption for NR BS EVM requirements was approved. In this contribution, we provide our simulation results on NR BS EVM performance of frequency range 1 (FR1), and make a proposal.

- Discussion

CRS have been used for channel estimation in LTE DL. In NR, UE-specific DMRS will be used to estimate the DL channels. The NR DL channel estimation performance compared with LTE would be different due to the different reference signals. So, it’s necessary to evaluate NR DL link performance to check if the EVM values defined for LTE BS can be reused for NR BS.

2.1 Simulation assumptions

For FR1, RAN1 hasn't finalized the final DMRS patterns yet. In this contribution, we evaluate the EVM impacts together with two selected DMRS density patterns [2] illustrated in Figure 1. The EVM values can be obtained from Table 1 below as per the WF mentioned. In addition, the link level simulation assumptions can be found in Table 2 below.

Figure 1, DMRS pattern assumptions

Table 1, EVM for NR

| Modulation scheme for NR-PDSCH | Required EVM [%] measured over all allocated resource blocks |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |

Table 2, Link level simulation assumption

| Carrier Frequency | 4GHz |
| --- | --- |
| Channel Model | PedA |
| Bandwidth | 10MHz |
| Subcarrier Space | 15KHz |
| Coding Scheme | LDPC |
| Code Rate | QPSK 1/5 16QAM 2/5 64QAM 2/3 256QAM 5/6, |
| HARQ | 0 |

  - Simulation results

2.2.1 Discussion on DMRS patterns

In order to study the difference of DMRS pattern 1 and pattern 2, we simulate the throughput performance of each DMRS pattern with the perfect channel estimation under 256 QAM modulations.  Figure 2 shows the throughput performance of MMSE and PERFECT channel estimation when DMRS pattern 1 is used. The throughput loss of MMSE channel estimation is caused by the channel estimation accuracy since the DMRS overheads are same. Figure 3 shows the DMRS pattern 2 results. It can be observed that, when compared with the perfect channel estimation, the throughput performance of the DMRS pattern 2 is slightly better than that of the DMRS pattern 1 due to the difference of DMRS density.

Another difference between DMRS pattern 1 and pattern 2 is the overhead of DMRS symbols. The overheads of DMRS pattern1 and DMRS pattern2 are different and they occupy 6and12 REs per RB (12x14 RE), respectively. So the final max throughput should obey the following rule: DMRS pattern1> DMRS pattern2 when the channel estimation performance is good enough.

Figure 2 Channel estimate of DMRS pattern1

Figure 3 Channel estimate of DMRS pattern2

2.2.2 EVM simulation results

In the EVM simulation, throughput with target EVM and ideal case (EVM 0) are compared. They are shown in Figure 4 and Figure 5. Figure 4 shows the EVM impacts on throughput performance for NR DMRS Pattern1. It can be observed that with EVM or not, the throughput difference is negligible. For DMRS pattern 2 in figure 5, similar performance can be observed. To further check the difference, we list the throughput loss at 95% max throughput over different modulation schemes and DMRS patterns in Table 3. As the simulation results show, the throughput losses are under 5% for both NR DMRS pattern 1 and pattern 2. So the LTE EVM requirements are tight enough for NR DMRS pattern 1 and 2. So, we propose,

Proposal 1: Reuse LTE EVM requirements for FR1 NR BS.

Table 3 Throughput loss compared with EVM 0

| Modulation [EVM %] | QPSK[17.5] | 16QAM[12.5] | 64QAM[8] | 256QAM[3.5] |
| --- | --- | --- | --- | --- |
| NR DMRS pattern 1 | 0.08% | 0.05% | 0.18% | 0.06% |
| NR DMRS pattern 2 | 0.02% | 0.06% | 0.06% | 0.03% |

EVM 0&17.5

EVM 0&12.5

EVM 0&8

EVM 0&3.5

Figure 4 EVM impacts on throughput performance, NR DMRS Pattern1

EVM 0&17.5

EVM 0&12.5

EVM 0&8

EVM 0&3.5

Figure 5 EVM impacts on throughput performance, NR DMRS Pattern2

- Conclusion

In this contribution, we provide our initial simulation results on BS EVM requirements for FR1. From the simulation results, we propose that,

Proposal 1: Reuse LTE EVM requirements for FR1 NR BS.

- Reference

[1] R4-1711777 ‘WF on simulation assumption for NR BS EVM requirement’, Huawei, HiSilicon

[2] R4-1711162 ‘Simulation Results of DM-RS patterns on EVM for FR1’, Ericsson

[3] RAN4#84bis Main Session Chairman Notes