3GPP TSG-WG RAN4 Meeting #85	R4-171xxxx

# Reno, Nevada, USA 27th Nov – 1st Dec, 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on wideband operation

Agenda item:		9.3.1.2

Document for:		Approval

# 1	Introduction

In this paper, we discuss the difference of intra-band CA and wideband operation and propose what the baseline RF requirement should be for each operation.

# 2	Discussion

There has been a long discussion what wideband operation is specified in RAN4 RF requirement but no consensus has been reached due to no common understanding on the difference between wideband operation and intra-band carrier aggregation. The agreed way-forward [2] has sort out the scenarios to some extent but there has been no agreed what spectrum utilization is expected for wideband operation and what UE RF requirement applies.

In this contribution, we propose the RAN4 way-forward aligned with the latest discussion from RAN1 and RAN1 perspective.

First, we discuss the LTE intra-band contiguous carrier aggregation illustrated in Figure 1. Each component carrier for BS and UE are identical. In LTE, we assume a nominal channel spacing to derive the RF requirement for intra-band contiguous CA and there is a guard band among component carriers.

Figure 1 An example of carrier aggregation scenario

Second, we consider an approach indicated in RAN1 LS [1] where intra-band contiguous CCs can be used within a wideband CC without guard band.

Figure 2 An example of Wide operation scenario

Several contributions pointed out both options are needed and should be specified, although the priority of the second operation was not the same among companies. It is also noted that there is no clear understanding yet what RF requirement is specified for each case and if it is the same or not.

We understand that a CA based RF requirement needs to be developed for the scenario depicted in Figure 1. For this case, the framework to specify RF requirement can be based on the intra-band contiguous CA from E-UTRA UE specifications, in particular for the frequency range 1(FR1).

On the other hand, for the wideband operation depicted in Figure 2, the coexistence requirement (such as ACLR and ACS) should be still based on the wideband CC as the splitting UE component carrier was just to ease the UE implementation but not to change the RF requirement for coexistence. The emission and blocking requirement should be based on the wideband CC requirement. The UE should conform to the single wideband CC requirement even if the aggregations of CCs within the BS carrier is made. In other words, the wideband operation is introduced just to segment the single channel bandwidth into multiple PRB for the sake of parallel baseband processing.

Observation 1: Wideband operation is used only to split the single channel bandwidth into multiple PRB segments for parallel processing; RF scenario is essentially the same as the single carrier.

This

The following table shows the baseline assumptions for wideband operation and carrier aggregation.

Table 1 Overview of wideband operation and carrier aggregation

|  | Wideband operation | Carrier aggregation (intra-band contiguous) |
| --- | --- | --- |
| multiple CCs for BS | No | Yes |
| multiple CCs for UE | Yes | Yes |
| PRB grid | contiguous | non-contiguous |
| Guard band | zero | non-zero |
| channel raster for BS | one raster point | multiple |
| channel raster for UE | multiple | multiple |
| Channel spacing | minimum | nominal |
| BS Transceiver requirement | single carrier | CA based |
| UE Transceiver requirement | single carrier based | CA based |
| UE Spectrum Utilization | The same as wideband CC | Per CC based |

# 3	Conclusion

We have discussed the RF requirements for wideband operation and carrier aggregation. The following proposal are derived.

Observation 1: Wideband operation is used only to split the single channel bandwidth into multiple PRB segments for parallel processing; RF scenario is essentially the same as the single carrier.

Proposal 1: UE RF requirement for wideband operation shall be based on the single carrier requirement.

Proposal 2: UE RF requirement for carrier aggregation shall be based on the E-UTRA CA requirement as a starting point.

# 4	References

- R1-1706615		LS on Wideband Operating Options

- R4-1711819	WF on wideband operation, Intel Corporation, LGE