3GPP TSG-RAN WG4 #85	R4-1714205

Reno, Nevada, USA, 27th November – 1 December 2017

Source: 	Ericsson

Title:  	TP on relevance with band 46 and band 48 for Rel-15 TR 36.790: LAA/eLAA for the “CBRS” 3.5GHz band in the United States

Agenda Item:	8.11.1

Document for:	Approval

# Introduction

In RAN#75, a new work item was agreed for licence assisted access (LAA) and enhanced LAA (eLAA) operation for the CBRS 3.5GHz band in US [1]. A new TR is agreed as TR 36.790 and a skeleton is agreed  in [2].

In this contribution, we propose a text for this TR which includes texts related to scope of the report.

# Text proposal

The following text proposal is intended for TR 36.790.

<<<<< start of TP <<<<<<

## 4.2	Relevance with LAA/eLAA band 46 and TDD band 48.

Band 49 BSs and UEs operate in the same frequency range as Band 48 but is adopting the same interlaced transmissions as Band 46.

Channel access procedure for band 49 shall be similar to band 46. This is applicable for both UL and DL.

With reference to UE requirements:

- the ON/OFF mask and transmission templates specified for FS3 operations in Band 46 shall be reused for FS3 operations in the CBRS band

- A dedicated NS value and associated A-MPR table has to be specified for interlaced transmissions of FS3

- In-band emissions requirements shall be specified as done for Band 46, in band emission mask for Band 49 10 MHz channel needs to be defined

- ACLR and ACS shall be same as band 48

- Band 49 ACS requirements should be verified similarly to Band 46 for Band 49, but without the relaxation allowed for 20 MHz in Band 46 to be on par with Band 48 requirements.

In-band blocking in Band 49 shall be defined for an unwanted interfering signal falling into the UE receive band or into the first 60 MHz below or above the UE receive band, similarly as Band 46.

- out-of-band blocking for inter-band carrier aggregation with band 49 shall be specified as for Band 46

- Wideband-intermodulation requirements for band combinations with Band 49 shall be specified similarly as Band 46 but with wideband interferer bandwidth equal to the wanted signal bandwidth

With reference to BS requirements:

- ACLR, ACS and blocking shall be the same as band 48

- same fixed reference channels (FRCs) for reference sensitivity, dynamic range and in-channel selectivity as Band 46 shall be reused

<<<<< end of TP <<<<<<

# Conclusion

We propose to adopt the above mentioned text proposal for TR 36.790.

# References

- RP- 170804, “New WI: LAA/eLAA for the CBRS 3.5GHz band in the United States”, AT&T, RAN#75, March 2017.

- R4-1709823, “Template skeleton for TR 36.790: LAA/eLAA for the “CBRS” 3.5GHz band in the United States”, Ericsson.