TSG-RAN Working Group 4 (Radio) meeting #85	R4-1714238

Reno, USA, 27th November- 1st December 2017

Source:	Ericsson

Title:	TP for TR 37.843: CATR Test Method Procedure for EVM

Agenda item:	8.27.4.1

Document for:	Approval

# Introduction

This contribution presents a test procedure for EVM measurement in a Compact Antenna Test Range which maybe is a method for conformance testing in [2].

# Discussion

Modulation quality is defined by the difference between the measured carrier signal and a reference signal. Modulation quality can be expressed as Error Vector Magnitude (EVM) for E-UTRA.

The EVM requirement however is important only between the BS and its intended UE. Hence the EVM is only of importance in directions where the BS intends to communicate with the UE’s. It is therefore in a set of specific directions from the AAS BS which is of interest rather than the average or total EVM over the entire sphere.

The range of directions which the AAS BS meets the EVM requirement is declared by the manufacturer as an EVM directions range.

Figure 1: Example of OTA coverage range and 5 conformance points

[Start of Text Proposal]

## 10.2 	Measurement uncertainty for directional requirements

For the derivation of the measurement uncertainty for direction requirements, the approach already defined in the TR 37.842 [4] shall be reused as much as possible.

### 10.2.1	CATR Test Method for EVM

10.2.2.1	Measurement Procedure

# Figure 1: CATR measurement system setup for EIRP

## 10.2.2.2	Test method limitations and scope

The maximum size of the DUT is a chamber restriction that would affect the quality of the quiet zone. For larger DUT sizes larger size chambers should be considered such that the uncertainty of the quiet zone is considered.

## 10.2.2.3	Procedure

Stage 1 – Calibration:

[Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

1)	Path loss calibration C→A:

a)	Measure SGH (or other calibrated reference antenna) reflection coefficient separately at the antenna's connector with a network analyser (or equivalent measurement equipment) to obtain ΓSGH.

b)	Measure cable loss from point C to input of SGH, call this LC↔SGH which is the equivalent of 20log|S21| from the use of a network analyser.

c)	Calculate the combined total path loss from C→A by using the following expression:

-	LSGHcal = LC,SGH + 10log(1 - |ΓSGH|2) - GSGH;

-	where10log1-ΓSGH2  10log(1 - |ΓSGH|2)  is the compensation for SGH connector return loss, GSGH  is the known gain of the reference SGH.

2)	Connect SGH and C↔A cable.

3)	To remove polarization(s) mismatch between range antenna (labelled as feeder antenna in diagram) and SGH use positions to position the SGH in the beam peak direction of range antenna.

4)	Measure path loss C→B with network analyzer LC→B = 20log|S21|.

5)	Calculate the test path loss compensation factor.  This is the total path loss between A↔B using the results from step 1c and 4. L = LSGHcal LSGHcal  - LC→B.

# Stage 2 - Measurement:

1)	Align DUT with beam peak direction of range antenna.

2)	Configure TX branch and carrier at a time according to the manufacturer's declared rated output power.

3)	Set the AAS BS to transmit the test signal according to E-TM3.1 at 5 MHz bandwidth configuration.

4)	Measure EVM of each carrier arriving at the measurement equipment (such as a spectrum analyzer or equivalent instrument) denoted in figure 2.

5)	Repeat steps 3-4 for all conformance test beam direction pairs as described in 3GPP TS 37.145-2 [2], subclause 6.2.

6)	Repeat steps 3-5 for E-TM3.2, E-TM3.3 and E-TM2.

## 10.2.2.4	Procedure

The EVM of each E-UTRA carrier for different modulation schemes on PDSCH shall be less than the limits defined in 3GPP TS 37.105 [1], subclause 9.x.x.x.

[End of Text Proposal]

# Conclusion

This contribution presents:

- An option to combine EVM and output power measurement

- No requirement for calibration procedure is in square brackets [] and needs further discussions if test ranges if measurement equipment fulfil the dynamic range for the test.

- Further investigation of the dynamic range of EVM measurement equipment may be necessary.

# References

[1]	TS 37.105, Active Antenna System (AAS) Base Station (BS) transmission and reception

[2] 	TS 37.145-2, Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing

[3] 	TS 37.842, Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)