3GPP TSG-RAN WG4 #86 Meeting                                R4-1801734

Athens, Greece, 26 February to 2nd of March, 2018

Agenda item:	7.3.1.3

Source: 	ZTE

Title: 	Minimum guardband of SS-PBCH block with SCS 240 kHz

Document for:	Approval

Introduction

RAN4#AH-1801 meeting made the following agreements and in RAN4 chairman’s notes it’s stated FFS for the minimum guardband of SS/PBCH block with SCS 240 kHz in February meeting as below [1]. In our contribution [5] we also provided our views on the minimum guardband of SS/PBCH block with SCS 240 kHz for FR2 and proposed the same minimum guardband values as Intel proposed below. This contribution basically has the same minimum guardband values proposal as [5] and further discusses the guardband size issue when the SS/PBCH block has extra power boosting than the data block.

Agreements:

Do not introduce UE capability to support SU according to wide channel bandwidth for intra-band CA in the Rel-15 scope.

Define minimum guardbands for 240 kHz SCS from the UE RF RX and BS Tx perspective only. Further revise the values in case the 240 kHz for data transmission are introduced in the future. FFS for minimum guardbands

=> FFS for the minimum guardband in Feb meeting. Proposed the minimum guardband from Intel

Table 1. Minimum guardband for 240 kHz SCS, kHz

| SCS (kHz) | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- |
| 240 | 3800 | 7720 | 15560 |

Minimum guardband of SS/PBCH block with SCS 240 kHz

In RAN4#85 meeting the maximum transmission bandwidth configuration NRB for SCSs 60 kHz and 120 kHz combination for FR2 were decided as shown in Table 1 below and have been specified in [4].

Table 1: Maximum transmission bandwidth configuration NRB for FR2

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 32 | 66 | 132 | 264 |

The minimum guardband for SCSs 60 kHz and 120 kHz were decided as shown in Table 2 and have been specified in [4]. The minimum guardbands have been calculated using the following equation: (CHBW * 1000 (kHz) - NRB * SCS x 12) / 2 - SCS/2, where NRB is from Table 1.

Table 2: Minimum Guardband [kHz] for FR2

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N.A |
| 120 | 1900 | 2420 | 4900 | 9860 |

In the frequency domain, a SS/PBCH block consists of 240 contiguous subcarriers with the subcarriers numbered in increasing order from 0 to 239 within the SS/PBCH block [3]. To derive the minimum guardband for SS/PBCH block with SCS 240 kHz, in this contribution we assume that this could be done by regarding SS/PBCH block as data block with single numerology SCS 240 kHz in the channel BW. As RAN4 did before for data channel with SCSs 60 kHz and 120 kHz for NR spectrum utilization, we did some PSD simulations for SCS 240 kHz to find the maximum NRB value for each channel BW to meet the unwanted emission requirements [4], and the results are summarized in Table 3. QPSK modulation is used in the simulation since a UE shall assume SS/PBCH block are QPSK modulated [3]. 50 MHz channel BW is too narrow for the 20-RB long SS/PBCH block.

Table 3: NRB for SCS 240 kHz

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 240 | N.A | 32 | 64 | 128 |

The minimum guardbands for SCS 240 kHz could be calculated using the same equation which is used to calculate the minimum guardband for other SCSs such as 60 kHz and 120 kHz in [4]. The results are summarized in Table 4 below.

Minimum guardband = (CHBW * 1000 (kHz) - NRB * SCS * 12) / 2 - SCS/2,

where NRB is from Table 3.

Table 4: Minimum Guardband [kHz] for SCS 240 kHz

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 240 | N.A | 3800 | 7720 | 15560 |

Proposal 1: When a SS/PBCH block with SCS 240 kHz is placed at the edge of a channel bandwidth and has the same TX power/kHz as the data block, the minimum guardband [kHz] will be:

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 240 | N.A | 3800 | 7720 | 15560 |

The TX power that the network is used for SSB transmission is included in SIB1 as below [2]. To improve UE receiving performance of SS/PBCH block, it’s possible that the network uses extra high TX power/kHz for SSB transmission than data blocks.

-- TX power that the NW used for SSB transmission. The UE uses it to estimate the RA preamble TX power.

-- (see 38.213, section 7.4)

ss-PBCH-BlockPower					INTEGER (-60..50),

When the SS/PBCH block has extra power boosting it will have more unwanted emission outside of the channel BW than the case without extra power boosting. How this will be handled in RAN4 should be further discussed and decided. In our opinion there are two options to deal with this issue:

- Option 1:  Adding extra guardband to the values in the minimum guardband table. FFS on extra guardband sizes for different levels of extra power boosting on SS/PBCH block;

- Option 2:  No extra guardband will be considered when the extra power boosting of SS/PBCH is ≤ X dB. Adding extra guardband when the extra power boosting of SS/PBCH is > X dB. FFS on X and extra guardband sizes for different levels of extra power boosting on SS/PBCH block.

Proposal 2: RAN4 should discuss and decide how to handle the guardband for SS/PBCH block when the block is placed at the edge of a channel bandwidth and has extra high power boosting compare to the data block. Here are two options could be considered:

- Option 1: Adding extra guardband to the values in the minimum guardband table in TS38.104. FFS on extra guardband sizes for different levels of extra power boosting on SS/PBCH block;

- Option 2: No extra guardband will be added when the extra power boosting of SS/PBCH is ≤ X dB. Adding extra guardband in TS38.104 when the extra power boosting of SS/PBCH is > X dB. FFS on X and extra guardband sizes for different levels of extra power boosting on SS/PBCH block.

Conclusions

Based on the discussions in this contribution, we have the following proposals:

Proposal 1: When a SS/PBCH block with SCS 240 kHz is placed at the edge of a channel bandwidth and has the same TX power/kHz as the data block, the minimum guardband [kHz] will be:

| SCS [kHz] | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 240 | N.A | 3800 | 7720 | 15560 |

Proposal 2: RAN4 should discuss and decide how to handle the guardband for SS/PBCH block when the block is placed at the edge of a channel bandwidth and has extra high power boosting compare to the data block. Here are two options could be considered:

- Option 1: Adding extra guardband to the values in the minimum guardband table in TS38.104. FFS on extra guardband sizes for different levels of extra power boosting on SS/PBCH block;

- Option 2: No extra guardband will be added when the extra power boosting of SS/PBCH is ≤ X dB. Adding extra guardband in TS38.104 when the extra power boosting of SS/PBCH is > X dB. FFS on X and extra guardband sizes for different levels of extra power boosting on SS/PBCH block.

# References

- RAN4#AH-1801 Main Session Chairman note_EOM, January 2018.

- RP-172570 draft TS38.331 RRC protocol, December 2017.

- RP-172284 draft TS 38.211 physical channels and modulation, December 2017.

- R4-1800265 TS 38.104 Combined updates (NSA) from RAN4 AH-1801, February 2018.

- R4-1800260 On minimum guardband of SS-PBCH block with SCS 240 kHz for FR2, ZTE, January 2018.