3GPP TSG-RAN WG4 Meeting #86	R4-1801768
Athens, Greece, 26 Feb - 02 Mar 2018

Agenda item:	7.9.4.2

Source:	Intel Corporation

Title:	Discussion on Propagation Channel models for demodulation

Document for:	Discussion

# Introduction

In RAN4-AH-1801 baseband only test methodology was agreed up on for demodulation performance testing in FR2.

- Proposal 1. Adopt “pure baseband” testing methodology for demodulation performance testing in FR2. - Assumes max rank 2 with cross polarized transmission - Rank 2 spatial MIMO would not be included - As a starting point an isotropic UE antenna pattern is assumed - Proposal 2. Methodology for RRM testing in FR2 is further discussed during the SI.

In this contribution we present our views on propagation channel model for defining UE performance requirements.

# Discussion

For the purpose of demodulation performance requirements the fading propagation channel models available are the channel models defined in TS 36.101 used in LTE and channel models defined in TR 38.900. The LTE fading channel models were designed to primarily support 15KHz SCS, with fixed PDP and not be suitable for FR2. The NR channel models – TDL and CDL are well suited for demodulation requirements for NR for all frequency ranges and SCS.

In the table below we compare the TDL and CDL propagation models for discussion on suitability for demodulation tests.

|  | NR TDL Channel Model | NR CDL Channel Model | Comments |
| --- | --- | --- | --- |
| Tx/Rx beamfoming impact | Model does not account the Tx/Rx beamforming. | Suitable for explicit modelling of Tx Beamforming and antenna geometry. Rx beamforming impact cannot be modelled. | Baseband only test methodology isolates the UE antenna impact. Baseband only test setup assumes beamlock and omni directional antenna at UE. Under practical conditions Tx/Rx beamforming will be applied and effective number of taps (and delay spread) might be reduced. Both models may need to be adjusted to be more close to the practical propagation conditions. |
| MIMO correlation modelling | Suitable for modelling configurable Tx/Rx antenna correlation | Tx/Rx antenna correlation is based on antenna geometry and spacing between elements. Effectively model will allow modelling of low Rx correlation only. | TDL channels model antenna correlation and would be the only option to consider baseband verification with different Rx correlation levels. |
| Power delay profile | PDP with multiple taps and low power for paths at larger delays. | PDP with multiple taps and low power for clusters at larger delays | Large number of taps may cause unnecessary TE complexity. For both models need to apply some filtering or thresholding to simplify model by reducing number low power paths/ clusters. |
| Delay spread | Suitable for setting a desired delay spread based on test conditions, SCS | Suitable for setting a desired delay spread based on test conditions, SCS | Both TDL and CDL models are well suited |

Based on the discussion above, TDL channel models seem better suited for demodulation requirements. Further modifications to the channel models are FFS.

Proposal#1: Define demodulation requirements with TDL channel models

Modifications to Channel Models

In order to capture the effect of beam-forming and antenna pattern, it might be necessary to modify the channel models. Also, with the effective number of taps reduced, the Doppler spread needs to be modelled as Doppler shift. These might require additional calibration of channel models.

Proposal#2: Further study the modifications necessary to channel models to capture effect of beamforming and antenna pattern

The TDL channel models (TDL-A/B/C) are defined with 23 paths.

Table 1: PDP of TDL-A

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0.0000 | -13.4 | Rayleigh |
| 2 | 0.3819 | 0 | Rayleigh |
| 3 | 0.4025 | -2.2 | Rayleigh |
| 4 | 0.5868 | -4 | Rayleigh |
| 5 | 0.4610 | -6 | Rayleigh |
| 6 | 0.5375 | -8.2 | Rayleigh |
| 7 | 0.6708 | -9.9 | Rayleigh |
| 8 | 0.5750 | -10.5 | Rayleigh |
| 9 | 0.7618 | -7.5 | Rayleigh |
| 10 | 1.5375 | -15.9 | Rayleigh |
| 11 | 1.8978 | -6.6 | Rayleigh |
| 12 | 2.2242 | -16.7 | Rayleigh |
| 13 | 2.1718 | -12.4 | Rayleigh |
| 14 | 2.4942 | -15.2 | Rayleigh |
| 15 | 2.5119 | -10.8 | Rayleigh |
| 16 | 3.0582 | -11.3 | Rayleigh |
| 17 | 4.0810 | -12.7 | Rayleigh |
| 18 | 4.4579 | -16.2 | Rayleigh |
| 19 | 4.5695 | -18.3 | Rayleigh |
| 20 | 4.7966 | -18.9 | Rayleigh |
| 21 | 5.0066 | -16.6 | Rayleigh |
| 22 | 5.3043 | -19.9 | Rayleigh |
| 23 | 9.6586 | -29.7 | Rayleigh |

The PDP of TDL-A is captured in Table 1. The highlighted rows show paths with average power < -15dB. Eliminating paths with low power could simplify the channel model, reducing the number of paths from 23 to 14 in this particular case. While eliminating the low power paths might simplify the channel model significantly, normalized path delays need to be re-calculated in order to achieve the same desired delay spread properties.

In [2] a thresholding method is proposed for CDL channel models.

Proposal#3: Further study the modifications necessary to simplify existing TDL channel models

LOS Channel Models

Under practical conditions with Tx/Rx beamforming, the N-LOS channel effectively transforms into a LOS channel. In 38.900 LOS models are defined for TDL and CDL and are well suited to be used for demodulation test cases. Also, with the assumption that spatial MIMO is not considered in the test methodology, the LOS channel models would be able to model the propagation conditions well.

The TDL LOS channel model - TDL-D/E are captured in Table 2 and Table 3.

Table 2: PDP for TDL-D

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -13.5 | Rayleigh |
| 2 | 0.035 | -18.8 | Rayleigh |
| 3 | 0.612 | -21 | Rayleigh |
| 4 | 1.363 | -22.8 | Rayleigh |
| 5 | 1.405 | -17.9 | Rayleigh |
| 6 | 1.804 | -20.1 | Rayleigh |
| 7 | 2.596 | -21.9 | Rayleigh |
| 8 | 1.775 | -22.9 | Rayleigh |
| 9 | 4.042 | -27.8 | Rayleigh |
| 10 | 7.937 | -23.6 | Rayleigh |
| 11 | 9.424 | -24.8 | Rayleigh |
| 12 | 9.708 | -30.0 | Rayleigh |
| 13 | 12.525 | -27.7 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 13.3 dB and a mean power of 0dB. |  |  |  |

Table 3:PDP for TDL-E

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.03 | LOS path |
|  | 0 | -22.03 | Rayleigh |
| 2 | 0.5133 | -15.8 | Rayleigh |
| 3 | 0.5440 | -18.1 | Rayleigh |
| 4 | 0.5630 | -19.8 | Rayleigh |
| 5 | 0.5440 | -22.9 | Rayleigh |
| 6 | 0.7112 | -22.4 | Rayleigh |
| 7 | 1.9092 | -18.6 | Rayleigh |
| 8 | 1.9293 | -20.8 | Rayleigh |
| 9 | 1.9589 | -22.6 | Rayleigh |
| 10 | 2.6426 | -22.3 | Rayleigh |
| 11 | 3.7136 | -25.6 | Rayleigh |
| 12 | 5.4524 | -20.2 | Rayleigh |
| 13 | 12.0034 | -29.8 | Rayleigh |
| 14 | 20.6519 | -29.2 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 22 dB and a mean power of 0dB. |  |  |  |

The LOS models can be further simplified to eliminate the low power paths – below average power of -20 or -25dB and reduce the complexity. Also, the normalized delays would need to be re-calibrated if the number of paths are reduced.

Proposal#4: Define UE performance test cases with LOS TDL channel models

# Conclusions

In this contribution we present our views on propagation channel models for UE demodulation requirements. Based on the discussion, we have the following proposals:

Proposal#1: Define demodulation requirements with TDL channel models

Proposal#2: Further study the modifications necessary to channel models to capture effect of beamforming and antenna pattern

Proposal#3: Further study the modifications necessary to simplify existing TDL channel models

Proposal#4: Define UE performance test cases with LOS TDL channel models

# References

- R4-1801288, WF on performance testing for FR2

- R4-1706514, On propagation model parameters for RRM and demodulation