3GPP TSG-RAN WG4#86 Meeting	R4-1801814

Athens, Greece, 26 February – 2 March 2018

Agenda item:	7.9.4.1.1

Title: 	Updated PSS SSS detection results for NR link level simulation in FR2

Source: 	Intel Corporation

Document for:	Discussion

- Introduction

In RAN4#84 Meeting, simulation assumptions for PSS/SSS detection in NR were agreed in [1]. Based on these assumptions, in RAN4 AH 1801 Meeting, we presented a set of PSS/SSS detection simulation results for mmWave bands under various channel conditions [2]. In this contribution, we present additional PSS/SSS detection simulation results for mmWave bands addressed in [1].

# Simulation Assumptions

The simulation assumptions used to determine the detection delay are based on [1] and are also provided below in Table 1 to Table 4.

Table 1: Cell-specific Parameters for PSS/SSS Detection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| Carrier frequency |  | 4GHz; 30GHz | The same frequency as for Cell 1 is used |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; 120 kHz; 240 kHz for 30GHz | The same values as for Cell 1 are used |
| RB Utilization | % | 100 | 100 |
| Power boosting for PSS and SSS | dB | 0 | 0 |
| Power boosting for Data and Control | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 (baseline) FFS: 4 | 1(baseline) FFS: 4 |
| SS burst set periodicity | ms | 5 | 5 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Relative Delay of 1st Path (asynchronous) | ms | 0 | 1.5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Es/Noc for K=1 | dB | 6.4 | 1.25; FSS: 0.25; -0.75 (SINR: -6dB; FFS: -8dB, -7dB) |
| Es/Noc for K=4 | dB | 2, 6,4, -2 | 2, 1, 1, 1 (SINR: -2.12, -5.97, -4.45, -1.12); -1, 5, -3, 2 dB (SINR: -5.12, -1.97, -8.45, -0.12) |
| Number of Tx antennas | - | 1 | 1 |
| PSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| SSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| Propagation Condition | - | AWGN, EPA5, ETU30 TDL-A/B/C/D with 100ns for 4 GHz and 30ns for 30 GHz Note1 CDL-A/B/C/D with 100ns for 4 GHz and 30ns for 30 GHz Note1 (secondary priority) |  |
| Noc Model | - | AWGN noise |  |
| NOTE 1: The channel models of CDL and TDL are used for simplified link level evaluations, which are defined in TR 38.901. |  |  |  |

Table 2: UE-specific Parameters for Cell Detection

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 by the UE | Yes |
| False detect threshold | Required as in a real UE implementation |
| UE having apriority knowledge of system being synchronous or asynchronous | No |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence IDs. |
| CP length detection | Not simulated |
| Receive antennas | 2, 4 (uncorrelated (except for CDL model) Option 1: With receiver beamforming Option 2: Without receiver beamforming) |

Table 3: Example Cell ID Combinations that can be simulated

| Case # | Cell 1 (Interferer Cell) |  | Cell 2 (Desired Cell) |  | Scenario |
| --- | --- | --- | --- | --- | --- |
| 1 | psc1 | ssc1 | psc2 | ssc2 | Synchronous |
| 2 | psc1 | ssc1 | psc1 | ssc3 | Synchronous |

Table 4: Example PSS, SSS indices that can be simulated simulations

| PSS Indices |  | SSS Indices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Label | Code index | Label | Code index (m0, m1) | Cell ID | (,) |
| psc1 | 43 | ssc1 | (20,0) | 337 | (112, 1) |
| psc2 | 86 | ssc2 | (40, 0) | 674 | (224, 2) |
|  |  | ssc3 | (35,0) | 673 | (224, 1) |

In the following, we present PSS/SSS detection simulation results for mmWave bands under various channel conditions based on simulation assumptions in Table 1 and the case number (Case#1 and Case#2) in Table 3.

# Simulation Results

Figure 1 to Figure 4 show the CDF curves for joint PSS/SSS detection delay in terms of number of SS blocks (SSB) for Case#1 and Case#2 of mmWave bands in AWGN, CDL and TDL channels with 120KHz and 240KHz SCS, respectively. It is also assumed that: (1) UE is aware of the correct serving cell ID; (2) the number of transmitted SS block within an SS burst set is one (i.e. K=1); (3) SINR is -6dB without RF impairments; and (4) two Rx antennas with ideal Rx beamforming and without beam sweeping process.

Fig. 1 CDF curve for joint PSS/SSS detection delay in terms of number of SS blocks for Case#1 and 120KHz SCS.

Fig. 2 CDF curve for joint PSS/SSS detection delay in terms of number of SS blocks for Case#1 and 240KHz SCS.

Fig. 3 CDF curve for joint PSS/SSS detection delay in terms of number of SS blocks for Case#2 and 120KHz SCS.

Fig. 4 CDF curve for joint PSS/SSS detection delay in terms of number of SS blocks for Case#2 and 240KHz SCS.

Observation: For AWGN, and CDL and TDL channels of 3km/h UE velocity and 30ns delay scaling, in most of the simulation cases, two SS blocks are required to achieve 90% PSS/SSS detection rate at SINR=-6dB with ideal Rx beamforming, and without beam sweeping process or RF impairments.

# Conclusions

In this contribution, we present PSS/SSS detection simulation results for mmWave bands under various channel conditions based on simulation assumptions in [1].

Observation: For AWGN, and CDL and TDL channels of 3km/h UE velocity and 30ns delay scaling, in most of the simulation cases, two SS blocks are required to achieve 90% PSS/SSS detection rate at SINR=-6dB with ideal Rx beamforming, and without beam sweeping process or RF impairments.

# References

[1] R4-1708698, “Updated link level simulation assumption of cell detection in NR”, Huawei, HiSilicon, NTT DOCOMO, INC., August 2017.

[2] R4-1800174, “PSS SSS detection results for NR link level simulation results in FR2”, Intel, January 2018.