3GPP TR 36.715-00-01 V0.1.0 (2018-02)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

LTE Advanced xDL/1UL Carrier Aggregation with x>5 (Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners’ Publications Offices.

Keywords

LTE-Radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles – Sophia Antipolis

Valbonne – FRANCE

Tel. : +33 4 92 94 42 00 Fax : +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	6

5	xDLs/1UL Carrier Aggregation: General part	6

5.1	UE RF specific	6

5.2	RRM specific	6

6	6DLs/1ULs Inter-Band Carrier Aggregation: Specific Band Combination Part	6

7	 7DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part	7

Annex A: Change history	7

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for LTE Advanced 1ULs and xDLs Carrier Aggregation with x>5 under Rel-15 time frame. The purpose is to gather the relevant background information and studies in order to address 1ULs and xDLs Carrier Aggregation requirements.

Table 1-1 Bandwidth combinations

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| 2A-46D-48A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6 A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| 2A-46C-48C-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| 2A-46A-48D-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 |  |  |  | Yes |  | Yes |  |  |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| 2A-46E-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 46 | See CA_44E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| 2A-46D-48C-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 140 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| 2A-46C-48D-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 140 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| 2A-48E-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| 46C-48E-66A | - | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 140 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| 2A-46C-48E | - | 2 |  |  | Yes | Yes | Yes | Yes | 140 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| 46E-48C-66A | - | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 140 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| 2A-46E-48C | - | 2 |  |  | Yes | Yes | Yes | Yes | 140 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[4]		RP-172839, “New WID on LTE Advanced inter-band CA Rel-15 for xDL/2UL with x>5”, Qualcomm Incorporated

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

Mixed Intra-band and inter-band carrier aggregation: Carrier aggregation of component carriers in same operating band and different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Background

The present document is a technical report for LTE Advanced 1UL and xDL Carrier Aggregation under Rel-15 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: this part covers BS and UE specific which is band combination independent.

- 	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs

# 5	xDLs/1UL Carrier Aggregation: General part

## 5.1	UE RF specific

## 5.2	RRM specific

# 6	6DLs/1ULs Inter-Band Carrier Aggregation: Specific Band Combination Part

# 7		7DLs/2ULs Inter-Band Carrier Aggregation: Specific Band Combination Part

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2018-02 | 3GPP RAN4#86 | R4-1802467 |  |  | TR36.715-00-01 Skeleton for xDL/1UL CA with x>5 | N/A | 0.0.1 |