TSG-RAN Working Group 4 (Radio) meeting #86	R4-1802499

Athens, Greece, 26th February – 2nd March 2018

Source:	Ericsson

Title:	TP to TS 38.141-2 subclause 7.3 OTA reference sensitivity level

Agenda item:	7.7.3

Document for:	Approval

# Introduction

This contribution presents a text proposal for OTA reference sensitivity level in subclause 7.3 of TS 38.141-2 and we would like to encourage feedback from other companies on the text proposal.

# Discussion

The OTA sensitivity test in subclause 7.2 of TS 37.145-2[4] is based on work done in TR 37.842 [2] and would be able to use in 38.141-2.

# Conclusion

This contribution presents a text proposal for OTA reference sensitivity level for FR2 in sub-clause 7.3 in TS 38.141-2.

The OTA sensitivity test procedure, presented in this contribution, is applicable for, SAC (Shielded Anechoic Chamber) and CATR (Compact Antenna Test Range) type of chambers.

# References

[1]	3GPP TS 38.104 V15.0.0 (2017-12)

[2]	3GPP TR 37.842 V13.2.0 (2017-03)

[3]	3GPP TR 37.843 V15.0.0 (2017-12)

[4]	3GPP TS 37.145-2 V14.3.0 (2017-12)

[5]	3GPP TS 38.141-1 V0.0.112 (2017-11065)

[Start of Text Proposal]

## 7.3	OTA reference sensitivity level

### 7.3.1	Definition and applicability

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level for a declared OTA REFSENS RoAoA. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received at the RIB at which a reference performance requirement shall be met for a specified reference measurement channel.

### 7.3.2	Minimum Requirement

For BS type 2-O the minimum requirement is in 3GPP TS 38.104 [1], subclause 10.3.3.

### 7.3.3	Test Purpose

The test purpose is to verify that the BS can meet the BER or throughput requirement for a specified measurement channel at the EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

### 7.3.4	Method of test

#### 7.3.4.1	Initial conditions

Test environment:

- Normal; see 3GPP TS 38.141-1 [4], clause B.2.

RF positions to be tested:

- B for single carrier in single-band operation, see subclause 4.9.x.

- BRFBW for multi carrier in single-band operation, see subclause 4.9.x.

- BRFBW_T'RFBW and B'RFBW_TRFBW  for multi carrier in multi-band operation, see subclause 4.9.x.

Directions to be tested:

-	Receiver target reference direction (see table 4.6-x, [D10.9])

#### 7.3.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in Annex x.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Ensure the polarisation is accounted for such that all the power from the test antenna is captured by the BS under test.

5)	Configure the beam peak direction of the BS according to sensitivity RoAoA for the appropriate beam identifier.

6)	Set the BS to transmit the beam(s) of the same operational band as the sensitivity RoAoA being tested according to the appropriate test configuration in clause 5.

7)	Start the signal generator for the wanted signal to transmit:

-	The test signal as specified in subclause 7.3.5.4

8)	Set the test signal mean power so the calibrated radiated power at the BS Antenna Array coordinate system reference point is as specified in subclause 7.3.5.

9)	Measure:

-	Throughput according to annex x, “General rules for statistical testing” for all supported polarizations.

For multi-band capable BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

### 7.3.5	Test Requirements

#### 7.3.5.1	General

The EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA. The test requirement is calculated from the declared value offset by the EISREFSENS Test Tolerance specified in subclause 4.1.

#### 7.3.5.4	Test Requirements

For each measured carrier, the throughput measured in step 7 or subclause 7.3.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in clause A.1 with parameters specified in table 7.3.5.4-1.

Table 7.3.5.4-1: EISREFSENS level

| NR Channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference measurement channel | EISREFSENS Level (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 30.0 GHz | 30.0 GHz < f ≤ 40.0 GHz | 40.0 GHz < f ≤ 52.6 GHz |
| 50 | 60 | G- FR2-A1-4 in clause A.1 in [1] | Declared minimum EISREFSENS + TBD | Declared minimum EISREFSENS + TBD | Declared minimum EISREFSENS + TBD |
| 100,200 | 60 | G- FR2-A1-1 in clause A.1 in [1] | Declared minimum EISREFSENS + TBD | Declared minimum EISREFSENS + TBD | Declared minimum EISREFSENS + TBD |
| 50 | 120 | G- FR2-A1-5 in clause A.1 in [1] | Declared minimum EISREFSENS + TBD | Declared minimum EISREFSENS + TBD | Declared minimum EISREFSENS + TBD |
| 100,200,400 | 120 | G- FR2-A1-2 in clause A.1 in [1] | Declared minimum EISREFSENS + TBD | Declared minimum EISREFSENS + TBD | Declared minimum EISREFSENS + TBD |

## [End of Text Proposal]