3GPP TSG-RAN WG4 Meeting AH-1801	R4-1802742

San Diego, California, USA, 22 – 26 January 2018

Agenda Item:	7.6.3.7

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Simulation Results for FRC for BS receiver in-channel sensitivity requirements

Document for:	Discussion

# 1.	Introduction

Fixed Reference Channels (FRC) for BS receiver requirements and simulation assumptions were discussed in RAN4#84Bis, and the way forward was agreed in [1]. Further discussions have been held via RAN4 reflector, and some simulation parameters had been revised accordingly [2].

This contribution provides the simulation results for FRC for BS receiver in-channel sensitivity requirements using QPSK, according to the agreed simulation parameters in [1] as well as those revised by email discussions [2].

# 2.	Discussion

The simulation results for FRC without FDM (i.e. the 6REs not used for DMRS in each DMRS symbol are not used for PUSCH transmission) for BS receiver in-channel sensitivity requirements using QPSK are summarized in Table 1 below. Note that practical channel estimation is used but no other imperfection is applied in the simulation.

Table 1: Simulation results for in-channel sensitivity requirements without FDM using QPSK

|  | Channel Bandwidth (MHz) | SCS (kHz) | TBS (bits) | LDPC Code Rate | Number of PRBs | 95% Throughput SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| G-FR1-A1-7 | 5 | 15 | 1320 | 1/3 | 15 | -0.8 |
| G-FR1-A1-8 | 5 | 30 | 528 | 1/3 | 6 | -0.6 |
| G-FR1-A1-9 | 10 | 60 | 528 | 1/3 | 6 | -0.6 |
| G-FR2-A1-4 | 50 | 60 | 2856 | 1/3 | 33 | -1 |
| G-FR2-A1-5 | 50 | 120 | 1416 | 1/3 | 16 | -0.8 |

# 3.	Conclusion

This contribution has provided the simulation results for FRC for BS receiver in-channel sensitivity requirements using QPSK.

# In-channels

[1]	R4-1801031, “WF on NR BS RF FRCs”, Ericsson, Huawei, ZTE, CATT, Nokia, Nokia Shanghai Bell.

[2]	R4-1802735, “TP to TR 38.817-02: Simulation Assumptions for NR BS RF FRCs (Annex B)”, Nokia, Nokia Shanghai Bell, Ericsson.