3GPP TSG-RAN WG4 #86	R4- 1802759

Athens, Greece, EU February 26th – March 2nd 2018

Source:	Ericsson

Title:	TP for draft CR 37.843: CATR Test Method Measurement Uncertainty for EVM

Agenda item:	6.27.3.1.2

Document for:	Approval

# Introduction

This contribution presents the measurement uncertainty framework and assessment for EVM measurement in a Compact Antenna Test Range which maybe is a method for conformance testing in [2].

# Discussion

Modulation quality is defined by the difference between the measured carrier signal and a reference signal. Modulation quality can be expressed as Error Vector Magnitude (EVM) for E-UTRA.

The EVM requirement however is important only between the BS and its intended UE. Hence the EVM is only of importance in directions where the BS intends to communicate with the UE’s. It is therefore in a set of specific directions from the AAS BS which is of interest rather than the average or total EVM over the entire sphere.

The range of directions which the AAS BS meets the EVM requirement is declared by the manufacturer as an EVM directions range.

Figure 1: Example of OTA coverage range and 5 conformance points

To reflect those points, the following is a proposed measurement uncertainty for a compact antenna test range for measuring OTA EVM.

[Start of Text Proposal]

## 10.2 	Measurement uncertainty for directional requirements

For the derivation of the measurement uncertainty for direction requirements, the approach already defined in the TR 37.842 [4] shall be reused as much as possible.

10.2.2.5	Uncertainty budget format

# Table 10.22.5-1: CATR uncertainty budget format for EVM measurement

| UID | Description of uncertainty contribution | Details in annex |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Misalignment DUT & pointing error | B2-1 |
| 2 | RF power measurement equipment | E |
| 3 | Standing wave between DUT and test range antenna | B2-3 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | B2-4 |
| 5 | QZ ripple DUT | B2-5 |
| 19 | Miscellaneous uncertainty | B2-14 |

10.2.2.6	Uncertainty assessment

| ACLR uncertainty budget |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty Source | Uncertainty value f ≦ 3GHz | Uncertainty value 3 GHz < f ≦ 4.2 GHz | Uncertainty value 4.2 GHz < f ≦ 6 GHz | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3 GHz < f ≦ 4.2 GHz | Standard uncertainty ui [dB] 4.2 GHz < f ≦ 6 GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |
| 1 | Misalignment DUT & pointing error | 0 | 0 | 0 | Exp. normal | 2 | 1 | 0 | 0 | 0 |
| 2 | RF power measurement equipment | 0.14 | 0.26 | 0.26 | Gaussian | 1 | 1 | 0.14 | 0.26 | 0.26 |
| 3 | Standing wave between DUT and test range antenna | 0.21 | 0.21 | 0.21 | U-shaped | √2 | 1 | 0.15 | 0.15 | 0.15 |
| 4 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.0012 | 0.0012 | 0.0012 | Normal | 1 | 1 | 0.0012 | 0.0012 | 0.0012 |
| 5 | QZ ripple DUT | 0.0928 | 0.0928 | 0.0928 | Normal | 1 | 1 | 0.0928 | 0.0928 | 0.0928 |
| 19 | Miscellaneous uncertainty |  |  |  |  |  | 1 |  |  |  |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  |  |  |  |  |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  |  |  |  |  |

[End of Text Proposal]

# Conclusion

This contribution presented a text proposal for measurement uncertainty relating to EVM requirement for CATR test method.

# References

[1]	TS 37.105, Active Antenna System (AAS) Base Station (BS) transmission and reception

[2] 	TS 37.145-2, Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing

[3] 	TS 37.842, Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)