3GPP TSG-RAN WG4 Meeting #86	R4-1802893

Athens, GR, 26th Feb. – 2nd March, 2018

Source:	NTT DOCOMO, INC.

Title:	Indoor anechoic chamber test method procedure for OBUE

Agenda item:	6.27.3.1.4

Document for:	Approval

- Introduction

This contribution presents OBUE test procedure for an indoor anechoic chamber test method for conformance testing.

- OBUE conformance requirement

Using TRP as a figure of merit to provide equivalent OBUE performance and protection to that of the existing conducted requirement, the conducted requirements can be adopted by translating the test procedure and requirement from TS 37.145-1, sub-clause 6.6.5 into the spatial domain.

In order that the requirement is measurable, the total power OBUE may be approximated as the sum of the EIRP at a number of discrete directions on the sphere around the DUT as follows:

, where EIRPOBUE is the filtered mean power within the OBUE region and p1 and p2 denotes two orthogonal polarizations.

The number of discrete points required for conformance of the OBUE OTA being defined as the TRP requirement is subject to the TRP sampling grid discussion.

- Indoor anechoic chamber test method for OBUE

In this section, we discuss and propose the OBUE test method for an indoor anechoic chamber based on TR.37.842.

The method measures the OBUE in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.3.2.1-1.

The maximum size of the DUT is a chamber restriction that would affect the quality of the quiet zone. For larger DUT sizes larger size chambers should be considered such that the uncertainty of the quiet zone is taken into account.

The test consists of two stages: the calibration and the measurement. The test procedure is as follows.

# Figure 10.3.2.1-1: Indoor Anechoic Chamber measurement system setup for OBUE

# Figure 10.3.2.3-1: Indoor Anechoic Chamber calibration system setup for OBUE

# Stage 1 - Calibration: [Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

1)	Connect the reference antenna and the receiving antenna to the measurement RF out port and RF in port of the network analyzer, respectively, as shown in figure 10.2.2.3-1.

2)	Install the reference antenna with its beam peak direction and the height of its phase centre aligned with the receiving antenna.

3)	Set the centre frequency of the network analyzer to the carrier centre frequency of the tested signal for OBUE measurement of AAS BS and measure LFEIRP, E→D , which is equivalent to 20log|S21| (dB) obtained by the network analyzer:

-	LFEIRP, E→D: Pathloss between E and D in figure 10.2.2.3-1.

4)	Measure the cable loss, LFEIRP, E→F between the reference antenna connector and the network analyzer connector:

-	LFEIRP, E→F: Cable loss between E and F in figure 10.2.2.3-1.

5)	Calculate the calibration value between A and D with the following formula:

-	LEIRP_cal, A→D = LFEIRP, E→D  + GREF_ANT_EIRP, A→F -LFEIRP, E→F.

-	LEIRP_cal, A→D:  Calibration value between  A and D in figure 10.2.2.3-1.

-	GREF_ANT_EIRP, A→F: Antenna gain of the reference antenna.

# Stage 2 - Measurement:

6)	Uninstall the reference antenna and install the AAS BS with the manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with the testing system.

7)	Set the AAS BS to generate the tested beam with the beam peak direction intended to be the same as the testing direction.

8)	Rotate the AAS BS to make the testing direction aligned with the direction of the receiving antenna.

9)	Set the AAS BS to transmit the test signal at the maximum power according to E-TM1.1.

10)	Measure the mean OBUE power for each carrier arriving at the measurement equipment connector, denoted by PR_AAS_OBUE, D.

-	PR_AAS_OBUE, D: Measured mean OBUE power for each carrier at the measurement equipment connector at D in figure 10.2.2.1-1.

11)	Calculate the EIRP with the following formula:

# EIRPOBUE = PR_AAS_OBUE, D+ LEIRP_cal, A→D

12)	Repeat the above steps 10)~11) at a number of discrete directions around the sphere.

13)	Calculate TRPs from EIRPOBUE following equation

, where EIRPOBUE is the filtered OBUE power and p1 and p2 denotes two orthogonal polarizations.

- Conclusion

This contribution presents:

- An option to combine OBUE and output power measurement

- Further investigation of the dynamic range of OBUE measurement equipment may be necessary.