3GPP TSG-RAN WG4 RAN4#86	R4-1803563

Athens, Greece, February 26th – March 2nd, 2018

Title:	Draft Reply LS on Baseband Processing Capabilities

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN4

To:	RAN2, RAN2

Contact Person:

#### Name:	Valentin Gheorghiu

E-mail Address:	vgheorgh [AT] qti.[DOT]qualcomm [DOT] com

# 1	Overall description

RAN4 would like to thank RAN2 for the LS on Baseband processing capabilities in R2-1801631.

RAN4 confirms that RAN2 agreements on BPC are aligned with RAN4 views:

- Keep supportedBW-PerCC as per CC capability. (Value range to wait for RAN4)

- Keep modulationOrder as per CC capability

- Separate supported modulation for DL and UL.

Below are the replies from RAN4 to the RAN2 questions.

- RAN2 understands that SCS support is separate for UL and DL and is dependent on the operating RF band, so does RAN4/1 view that the baseband processing capabilities in the BPC table depend on SCS used?

Reply: Yes. baseband processing capabilities in the BPC table depend on the used SCS. RAN4 agreed in RAN4#86 the following as captured in the chairman minutes: “from RAN4 perspective UE shall be able to signal the supported SCS per CC for each band combinations.”

Q1.1: Does RAN4 see a need to signal the SCS capability per CC in the BPC?

Reply: Yes.

Q1.2: Does RAN4 see a need to signal the SCS capability separately for UL and DL in the BPC?

Reply: Yes.

- What should be value ranges for supportedBW-PerCC which is reported per CC in BPC?

Reply: Full range of channel bandwidths should be supported, it can be found in the latest version of 38.101-1 for FR1 and 38.101-2 for FR2.

- How does RAN4 intend to define the carrier bandwidth and the number of aggregated intra-band carriers? Is that supposed to be done by the bandwidth class (as in LTE) or by other means?

Reply: RAN4 intends to define the carrier bandwidth and the number of aggregated intra-band carriers using bandwidth class as in LTE. RAN4 will further provide details of CA BW Class after discussion is concluded.

- What is the relationship between the carrier bandwidth of aggregated carriers and the maximum channel bandwidth supported by the UE?

Reply: The aggregated channel BW for multiple carriers supported by the UE in CA case does not directly depend on the maximum CBW per one carrier supported by the UE.

# 2	Actions

ACTION to RAN2:

RAN4 respectfully asks RAN2 to take into considerations the above replies

# 3	Date of Next RAN4 Meetings:

TSG-RAN WG4 Meeting #86bis 	16th – 20th April 2018      Melbourne, Australia

TSG-RAN WG4 Meeting #87	21st – 25th May 2018		Busan, Korea