3GPP TGS-RAN WG4 Meeting #86-Bis	R4-1804124
Melbourne, Australia, April 16 – April 20, 2018

Agenda item:	7.4.8.1

Source:	Intel Corporation

Title:	On power class for FWA devices in FR2

Document for:	Approval

# 1	Introduction

FR2 power class agreements have evolved over the last months. The parameters needed to derive the peak EIRP for handheld devices were agreed in [1] and a range for the peak EIRP requirement was approved in [2]. However, no consensus has been found to finalize the requirement [3-5]. With a plan to target Rel-15 timeframe, during the AH-1801 meeting a new power class UE type for fixed wireless access (FWA) devices was introduced [6]. Lastly, in RAN4 #86, the parameters table for FWA peak EIRP calculation was discussed and approved [7].

This paper summarizes the power class agreements for FWA devices, presents an example value for peak EIRP requirement and proposes how to define and divide this UE type for approval.

# 2	Discussion

## 2.1	Background

The deadline for Rel-15 is fast approaching and power class definition in FR2 needs to be finalized. Thus far, discussions have focused on the requirements for handheld UE types and a range of peak EIRP values based on data reported by seven companies was approved [2]. While the plan was to narrow the range and finalize the requirement in the following meeting, no consensus has been found amongst the presented proposals in the last two meetings [3-5].

Agreements from RAN4 #85 [2]:

- The handheld UE peak EIRP range is defined as follows and will be captured in TS38.101-2 in this meeting

  - For 28 GHz: [21.2-25.2] dBm

  - For 39 GHz: [19.4-23.7] dBm

- Companies are encouraged to provide additional analysis with the intention to finalize the UE peak EIRP requirement

A new power class UE type for fixed wireless access (FWA) devices was approved during AH-1801 and its requirements also target the Rel-15 scope [6]. The agreements for this UE type stipulate assessing impact on UE requirements and aligning on evaluation parameters needed to derive the minimum peak EIRP requirement.

Work plan for FWA [6]:

To align on the evaluation parameters for the new UE type, a table was approved in RAN4 #86 [7]. During this meeting, it was also brought up that both use case and deployment scenarios should be clarified to assess how to apply the UE feature list to these devices [8].

## 2.2	Defining FR2 Power Class

Discussions on how to narrow and finalize the peak EIRP range for handheld devices are still on-going. While this debate continues, the issue of a higher peak EIRP required by operators remains. If consensus is found and the handheld peak EIRP value is lower than expected, a higher peak EIRP can be obtained from other UE types.

Observation 1: Having separate UE type requirements may help bridge the gap in the expected minimum peak EIRP requirements.

If we think about the most popular use cases, defining new power class UE types is a logical and necessary path forward. Fixed wireless access, with its large coverage area and relatively low latency, is expected to have significant growth in 5G through mm-wave frequencies. Larger portable devices (i.e. tablets) are popular form-factors with over 1 billion users worldwide [9]. The FCC has even adopted a general framework for additional UE types across 28 and 39 GHz based on higher transmit power [10]. Considering all of this, defining requirements for new UE types helps lay the ground work needed for their projected growth. This also brings up an important discussion point, what is the best way to separate and define UE types in power class. Some relevant differences between UE types to consider, include form-factor size, use case, and environment while device is operating.

Observation 2: The differences found in UE types that will support FR2 will be reflected in their achievable minimum peak EIRP values. A RAN4 discussion is needed to find the best way to divide power class based on these differences.

Based on current and anticipated use cases, it makes sense to have two types: portable and fixed wireless access (FWA). The portable type can then be subcategorized depending on size or form-factor as handheld or larger (tablets). For the fixed wireless access, we can make a distinction based on the platform they are mounted on. The FCC defines higher power devices as transportable stations and focuses the definition of these devices as stationary while operating [10]. While this excludes a potential use case of devices fixed inside vehicles, we can expand the FWA term and further divide as fixed-stationary (including customer premises equipment) and fixed-on-moving platform.

Proposal 1: Discuss having two main UE types for FR2 power class: portable and fixed. Consider further defining portable devices based on their form-factor as handheld and larger. Categorize fixed devices depending on whether they are fixed on a stationary platform or a moving platform.

Given the amount of time remaining for Rel-15 and the fact that fixed-on-moving platform devices will likely require further studies, we will focus the rest of the paper on FWA devices on a stationary platform. The example values used to derive peak EIRP in the upcoming section will be for this type.

## 2.3	Power class requirements for FWA

It is important to assess the potential impact including a new UE type will have on our requirements. In last month’s RAN4 #86 meeting, a table detailing relevant architecture parameters was approved [7]. The content of that table is captured in Table 1 in this paper. We will use the data found in Table 1 to provide a preliminary minimum peak EIRP for FWA devices on a stationary platform.

Table 1. FWA parameters for minimum peak EIRP evaluation

| Parameter | Unit | Freq. range 24.25-29.5 GHz | Freq. range 37.0-40.0 GHz | Comments |
| --- | --- | --- | --- | --- |
| Pout per element | dBm | 14.0 | 14.0 |  |
| # of antennas in array |  | 16 | 16 | Previously 4 for handheld |
| Total conducted power per polarization | dBm | 26.0 | 26.0 | Previously 20 dBm |
| Avg. antenna element gain | dBi | 4.0 | 4.0 |  |
| Antenna roll-off loss vs frequency | dB | -2.0 | -2.5 |  |
| Realized antenna array gain | dBi | 14.0 | 13.5 |  |
| Polarization gain | dB | 2.80 | 2.80 |  |
| Mismatch and transmission line loss including load pull | dB | -2.50 | -3.00 |  |
| Beam forming loss (phase shifter and amplitude error) | dB | -0.50 | -0.50 |  |
| Finite beam table | dB | -0.25 | -0.25 |  |
| Beam forming loss (one beam table fits all) | dB | -0.25 | -0.25 |  |
| Form-factor integration losses | dB | -3.00 | -4.00 |  |
| Total implementation loss (worst-case) | dB | -6.50 | -8.00 |  |
| Peak EIRP (Minimum) | dBm | 36.30 | 34.30 |  |

The FWA example in Table 1 is for a different use case than handheld devices. The higher power expected from these devices is possible in great part to their larger form-factor. This enables different architecture implementations with more antenna elements in the array (larger gain) and allows for greater separation between sensitive RF components and high dielectric constant/lossy materials and reflectors that may otherwise affect radiation. The larger size also makes the overall integration easier and reduces the implementation losses. These differences are captured in the number of antennas, total conducted power and total implementation loss parameters. Compared to our handheld numbers, the number of antennas increased from 4 to 16, thus increasing the total conducted power by 6 dB. The overall implementation loss parameter has been reduced from the handheld number, highlighting the integration challenges of smaller form-factors.

Factoring all these changes in the link budget yields a minimum peak EIRP of 36.30 dBm for 28 GHz, and 34.30 dBm for 39 GHz. These numbers correspond to a minimum peak EIRP increase of over 15 dB compared to our numbers for handheld devices (20.2 dBm for 28 GHz, 18.4 dBm for 39 GHz).

Observation 3: The example minimum peak EIRP calculations for FWA devices on a stationary platform yield 36.30 dBm for 28GHz, and 34.30 dBm for 39GHz. This represents an increase of over 15 dB from our handheld device values.

Proposal 2: Define the minimum peak EIRP requirement for FWA devices on a stationary platform as 36.30 dBm for 28 GHz, and 34.30 dBm for 39 GHz.

We need to be mindful of the potential impact the new FWA UE type may have on RAN4 requirements. There will certainly be implications on testability and spherical coverage studies, as measurements must now take into account the fixed nature of the device, use case and typical surrounding materials or environment. From a co-existence perspective, no additional work is needed as previous studies for +55dBm CPE have been concluded with no issues reported [11]. However, the impact on UE requirements we have covered so far is not comprehensive. Additional topics are not precluded from future discussions, especially depending on the particular use case of the FWA device. This is why we need to agree on how to approach the definition and classification of FWA devices. It is also important to note that there may be required features for handheld devices that can be made optional for FWA devices. These need to be discussed, agreed and documented.

Observation 4: Companies are encouraged to include additional topics of interest that may impact FWA requirements for discussion in upcoming meetings.

Proposal 3: We should revisit the required handheld device features and determine which ones can be defined as optional capabilities for FWA devices.

# 3	Conclusions

In this paper we detailed our views on how to address introducing new power class UE types. Focusing on FWA devices, we discussed a parameters table to be use as an alignment tool, along with preliminary results for peak EIRP. The following observations and proposals have been made:

Observation 1: Having separate device type requirements can help bridge the gap in the expected minimum requirements peak EIRP.

Observation 2: The differences found in UE types that will support FR2 will be reflected in their achievable minimum peak EIRP values. A RAN4 discussion is needed to find the best way to divide power class based on these differences.

Proposal 1: Discuss having two main UE types for FR2 power class: portable and fixed. Consider further defining portable devices based on their form-factor as handheld and larger. Categorize fixed devices depending on whether they are fixed on a stationary platform or a moving platform.

Observation 3: The example minimum peak EIRP calculations for FWA devices on a stationary platform yield 36.30 dBm for 28GHz, and 34.30 dBm for 39GHz. This represents an increase of over 15 dB from our handheld device values.

Proposal 2: Define the minimum peak EIRP requirement for FWA devices on a stationary platform as 36.30 dBm for 28 GHz, and 34.30 dBm for 39 GHz.

Observation 4: Companies are encouraged to include additional topics of interest that may impact FWA requirements for discussion in upcoming meetings.

Proposal 3: We should revisit the required handheld device features and determine which ones can be defined as optional capabilities for FWA devices.

# 4	References

- R4-1706935, “WF on power class framework for mmWave,” Intel Corporation, Huawei, LGE, 3GPP RAN4 AH NR#2, June 2017

- R4-1714445, “WF on pi/2 BPSK spectrum shaping and power class in FR2,” Intel Corporation, Apple, Qualcomm, IITH, LGE, Nokia, Huawei, Motorola Mobility, 3GPP RAN4 #85, November 2017

- R4-1801106, “WF peak EIRP requirement for mmW,” Qualcomm,  RAN4 AH-1801, January 2018

- R4-1803372, “WF on output power requirements on FR2,” Qualcomm, Samsung, Verizon, AT&T, T-Mobile USA, NXP, DISH, Orange, ZTE, US Cellular, C Spire, Ericsson, Telstra, KT, Nokia, Vodafone, Mavenir, Telecom Italia, Reliance Jio, IITH, IITM, CEWiT, Tejas Networks, 3GPP RAN4 #86, February 2018

- R4-1803460, “WF on FR2 peak EIRP for handheld UE,” Intel, Xiaomi, OPPO, vivo, MediaTek, LGE, Apple, Qorvo, 3GPP RAN4 #86, February 2018

- R4-1801191, “WF on Fixed wireless access use case for FR2,” Intel Corporation, Verizon, January 2018

- R4-1801787, “On FR2 power class for FWA devices,” Intel Corporation, RAN4 #86, February 2018

- R4-1802402, “Handling on minimum requirements for CPE devices,” Samsung, RAN4 #86, February 2018

- Statista: Forecast number of tablet users worldwide 2013-2020, https://www.statista.com/statistics/377977/tablet-users-worldwide-forecast/

- FCC 16-89

- R4-1711861, “Summary of simulation results on Coexistence Studies for 55dBm CPE,” Ericsson, Qualcomm, Samsung, Verizon, Nokia, RAN4 #84bis, October 2017