3GPP TSG-RAN WG4 Meeting #86bis		R4-1804159
Melbourne, Australia, April 16 – 20, 2018

Agenda item:	7.4.8.6

Source:	Intel Corporation

Title:	Open loop TPC for FR2

Document for:	Discussion

# Introduction

In lasting meeting, we presented our paper [1] to show our views on OL TPC for FR2. With some updates, we resubmit our paper in this meeting for further discussion.

# Discussion

During the last meeting, we recognized that beam correspondence is a necessary assumption in open loop power control from the network point of view. Whereas during closed loop the network can issue corrections via TPC commands, during open loop the network relies on the UE’s ability perform three main actions: 1) to measure the DL signal, 2) to select the corresponding beam, and 3) to set the output power.  Thus, it makes sense to define a requirement on beam correspondence in conjunction with a requirement on open loop power control. It was agreed in the chairman note [2] that

- The requirement on UE beam correspondence can be defined in conjunction with the requirement on open loop power control.

- An open loop power control requirement is needed to quantify the UE’s ability to select the UL beam corresponding to the DL signal (beam correspondence) and its ability to correctly set its output power to compensate the overall coupling loss (open loop TPC).

Although practical UE implementations optimize the selection of particular beamforming coefficients, we conceptually denote the UE’s spatial response in terms of a number of discrete beams and a radiated power value associated with each beam. This is the basis of the spherical coverage discussions, and Figure 1 below provides a conceptual illustration.

Figure 1: Conceptual illustration of UE beams

Using this illustration we can represent the open loop power control procedures using four scenarios, as shown in Figure 2 below.

a) b) 
c) d)

Figure 2: Illustration of OL TPC with four scenarios

In Scenarios 1 and 2 the UE remains in the same position (denoted as Position 1) while the propagation coefficient (which is the inverse of the path loss) varies.  In response to such conditions the UE is expected to select a single beam and to adjust its output power to compensate the path loss and beam gain such that the total received power at the base station approaches the target value of P0-nominal.

In Scenarios 3 and 4 the UE position changes to Position 2 so that the UE now selects a different set of beamforming coefficients to transmit to the gNB.  As in Scenarios 1 and 2, the propagation coefficient is varied to ensure that the UE can adjust its output power to compensate the path loss and beam gain such that the total received power at the base station approaches the target value of P0-nominal.

We note that in each of these scenarios, the UE locates the DL beam, selects beamforming coefficients based on this received beam, and adjusts its output power to compensate the total coupling loss.

A conceptual illustration of a metric to evaluate this requirement is shown in Figure 3.

a) b)

Figure 3: OL TPC metric

There are limits to this metric:  the minimum UE output power as well as the test equipment sensitivity determine the lower bound, while the maximum UE output power in any given direction determines the upper bound.  Since this test is not intended to quantify either of these extreme points, it is possible to define the test case parameters such that optimal operating points are considered.  For example, test equipment can determine the DL power level for which the UE transmits nearly minimum power as one test point and select another DL power level several dB lower for another test point.  Multiple test points and multiple UE positions can be defined to improve the metric’s value, although a trade-off with total test time needs to also be considered.

Evaluation metrics for OL TPC can be defined as:

Option 1: Define the OL TPC absolute tolerance requirement as the average deviation (across all test points) of the total power received by the gNB from the expected P0-nominal (Figure 3a)

Option 2: Define the OL TPC and beam correspondence relative tolerance requirement as the average difference (across all test points which share the same UE position) between the EIRP delta for scenarios with the same position and different propagation coefficient and these propagation coefficients’ difference

Proposal 1: The metric defined for the open loop power control requirement can be defined as Option 1 and Option 2; other options are not precluded.

A procedure for this test can be described as follows:

- Place the DUT inside the test chamber in an orientation relative to the test equipment measurement antenna corresponding to POS1

- Transmit a DL signal to the UE with power level PDL,1 such that the propagation coefficient (inverse of the path loss) is A1

- Measure the UE output power

- Repeat steps 1 through 2 by sweeping all test positions from POS1 to POSN and propagation coefficients A1 to AN such that all test points are covered; the test positions and propagation coefficients may be randomized to ensure the randomization of the initial conditions of each test

- The propagation coefficients are chosen such that PDL,1 does not exceed the DL power level used in the minimum output power test, while PDL,N is not less than the DL power level used in the spherical coverage test; alternatively, PDL,N can be defined as a fixed delta relative to PDL,1, such as 4 or 6 dB. In other words, UE transmit power should be within its dynamic range.

- Calculate metrics associated with Options 1 from the complete set of test results

- Calculate metrics associated with Option 2 from a subset of test results which share the same DUT position relative to the test equipment measurement antenna

Proposal 2: The open loop power control requirement metric can be measured using the test procedure described above.

# Conclusions

In this contribution we provided a proposal for defining an open loop power control test for FR2 UEs.  We have made the following proposals:

Proposal 1: The metric defined for the open loop power control requirement can be defined as Option 1 and Option 2; other options are not precluded.

Proposal 2: The open loop power control requirement metric can be measured using the test procedure described above.

# References

- R4-1801803, “Open loop TPC for FR2”, Intel Corporation, 3GPP RAN4 #86, Feb, 2018.

- Chairman note, 3GPP RAN4 #86, Feb, 2018.