3GPP TSG-WG RAN4 Meeting #86bis	R4-180xxxx

# Melbourne, Australia 16th – 20th April, 2018

Source: 	Nokia, Nokia Shanghai Bell, Verizon

Title: 					TP to TR 36.715-04-01: CA_4DL_2A-13A-48A-66A_1UL_BCS0

Agenda Item: 			6.4.2

Document for:	Approval

# Introduction

This TP introduces 4DL/1UL CA configuration CA_2A-13A-48A-66A in to the TR 36.715-04-01.

**************************** Start of changes ********************************************

## 5.X	CA_2A-13A-48A-66A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-13A-48A-66A | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  | 13 |  |  | Yes | Yes |  |  |  |  |
|  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.X.2	Co-existence studies

Table 5.X.2-1 summarizes frequency ranges where harmonics occur due to Band 2, Band 13, Band 48 and Band 66 CA with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 3700 | 3820 | 5550 | 5730 | - | - |
| 13 | 777 | 787 | 1554 | 1574 | 2331 | 2361 | 3108 | 3148 |
| 48 | 3550 | 3700 | - | - | - | - | - | - |
| 66 | 1710 | 1780 | 3420 | 3560 | 5130 | 5340 | - | - |

It can be seen from Table 5.X.2-1 that the 2nd harmonics of band 2 and band 66 UL hit band 48 DL.

### 5.X.3	∆TIB,c and ∆RIB,c values

For 4DL/1UL CA_2A-13A-48A-66A, the TIB,c and RIB,c values are shown in Table 5.X.3-1 and Table 5.X.3-2, respectively, as derived from the maximum relaxation value of CA_2A-13A-48A, CA_2A-13A-66A, CA_2A-48A-66A and CA_13A-48A-66A.

Table 5.X.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2-13-48-66 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |

Table 5.X.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2-13-48-66 | 2 | 0.3 |
|  | 13 | 0.5 |
|  | 48 | 0 |
|  | 66 | 0.3 |

### 5.X.4	REFSENS exceptions

Reference sensitivity exceptions due to the harmonic issue for CA_2A-13A-48A-66A are specified in Table 5.X.4-1 for the uplink configurations specified in Table 7.3.1-2 of TS36.101.

Table 5.X.4-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-13A-48A-66A9,10 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 13 |  |  | -97 | -94 |  |  |  |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 |  |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |
| CA_2A-13A-48A-66A11 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 13 |  |  | -97 | -94 |  |  |  |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 |  |
| NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2A-13A-48A-66A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

Table 5.X.4-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_2A-13A-48A-66A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |

**************************** End of changes ********************************************