3GPP TSG-RAN WG4 Meeting #86bis	R4-1805405

Melbourne, Vic, Australia, 16th – 20th April 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to draft CR TR 37.843 - CATR set up for OTA ACLR

Agenda item:	6.26.3.1.4

Document for:	Approval

# 1	Introduction

During the last RAN4#86 meeting, the WF on OTA TR structure for Section 10 [1] was approved, which outlined the skeleton for capturing OTA directional and TRP conformance requirements.

Based on the discussion in [2], we contribute a TP to TR 37.843, which is a test method for OTA ACLR measurements in a CATR chamber. Constructive feedback is encouraged.

# 2	Discussion

In [2], we discussed and presented a test procedure for OTA ACLR measurements of OTA AAS BS and NR BS type 1-O. The procedure serves a framework for further optimization, which leads to further reduction in measurement time.

# 3	Conclusions

It is proposed to agree on the TP to TR 37.843.

# 4	References

- R4-1803388, WF on TR structure for Section 10, Ericsson

- R4-1805394, CATR set up for ACLR of OTA AAS BS and NR BS type 1-O, Nokia, Nokia Shanghai Bell

# 5	Text proposal

TR 37.843 V15.0.0

--------------Start of text proposal-------------

# 10.3.1		Measurement uncertainty for TRP requirements

Based on the definition of the TRP measurement, the measurement uncertainty of the TRP requirement relies on the measurement uncertainty derivation for the directional requirements, with consideration of the additional uncertainty contributors, depending on the OTA test range and sampling grid.

#### 10.3.1.2	OTA ACLR

##### 10.3.1.2.1	Compact antenna test range

##### 10.3.1.2.1.1		Description

Refer TR 37.842.

##### 10.3.1.2.1.2	Test method limitation and scope

Refer to TR 37.842.

##### 10.3.1.2.1.3	Procedure

Stage 1 - Calibration:

Refer to TR 37.842 for EIRP measurements. Adjacent channels of wanted channels under measurement should also be included in the calibration stage.

# Stage 2 - Measurement:

- Set up OTA AAS BS/NR BS type 1-O (DUT) in place of SGH from the calibration stage.

2)    Configure OTA AAS BS/NR BS type 1-O output power according to the manufacturer’s declared rate power.

3)	 Set the DUT to transmit a single beam according to the test signal according to E-TM1.1 at 5 MHz bandwidth configuration.

- Set the spectrum analyser centre frequency start frequency (fstart) and stop frequency (fstop) to assigned carrier frequency, (FBW_RF,low – FACLR,low ) and (FBW_RF,high – FACLR,high), respectively.

- Set the number of trace points in a sweep by the spectrum analyser to ≥2×spanBWchannel, where span= fstop-fstart.

- Configure the sweep time of the spectrum analyser to less than the transmission duration; for non-continuous transmission, employ the spectrum analyser gated sweep).

- Select the spectrum analyser power detection mode to power averaging.

- Select a suitable spherical sampling grid (e.g., spherical equal area sampling grid, etc.) and determine the total number of spherical points T = FFS.

9)  Set the DUT position to the first test point from the set of T points in the sampling grid and align with the direction of range antenna.

10) Measurements should be made with the measurement antenna oriented in both orthogonal polarizations. If the measurement antenna supports single polarization, then it should be oriented in one polarization at a time.

11) Record two orthogonal polarized mean power levels (Pmeas,p1, Pmeas,p2) of assigned and adjacent carrier frequencies, and DUT angular position in list L.

12)	 Move the DUT to the next test point and repeat Steps 10-11 until all T spherical points are completed.

13) For each Pmeas sample in L, calculate and record EIRP in L, where EIRP = Pmeas + LA→B  dB.

14) Calculate TRP (using the formula for the sampling grid selected in Step 8 using the EIRP samples in L for the assigned carrier frequency and adjacent carrier frequencies.

--------------end of text proposal-------------