3GPPRAN4#86-Bis	R4-1805430

Melbourne, Australia April 16th- 20th, 2018

Title: 	PSCell Configuration Timeline

Source: 	Qualcomm Incorporated

Agenda item:	7.9.10.3 - PSCell addition/release/change and SCell (de)activation [NR_newRAT-Core]

Document for:	Discussion

# Introduction

In this contribution, we provide our view on the PSCell addition timeline. First, we propose to change the time of the end of PSCell addition timeline till PRACH rather than CQI reporting, since this is more in line with the it is being defined in LTE. Also, this brings the definition more in line with handover definition. Secondly, we provide our view of the PSCell addition timeline.

# Discussion

In the currently agreed WF on PSCell addition [1], as part of PSCell configuration time we are re-using the SCell activation time, which includes time to report back CQI. However, this definition is inconsistent with the definition for PSCell configuration where the end is UE transmission of PRACH pre-amble. We propose to the change the Tconfig_PSCell definition to only include the time to PRACH pre-amble transmission. This would also harmonize the definition of PSCell configuration with that of handover.

Proposal 1: The configuration time for PSCell addition should not include time for CQI reporting. The configuration time should be

Tconfig_PSCell = TRRC_delay + Tprocessing + Tloops + Tsearch + TPSCell_ DU

Where TRRC_delay: is the RRC procedure delay

Tprocessing: is the SW processing time needed by UE.

Tsearch: is the time for PSS/SSS detection, which is 0 for known cell. And [TBD] ms for unknown cell provided that the signal quality is sufficient for successful cell detection on the first attempt

Tloops: time, for AGC and loops for time refinement

TPSCell_DU:  delay uncertainty in acquiring the first available PRACH occasion in the PSCell

The processing time for PSCell add should be same as the processing time the UE takes during handover and we propose the same value to be used here.

Proposal 2: Tprocessing can be up to 20ms.

For the PSCell addition the UE will need at least 3 SMTC duration for loops, which includes 2 samples for AGC and 1 for time refinement.

Proposal 3: The time for AGC and loop refinement can be up to 3 SMTC duration.

Finally, similar to handover, the search time would be needed in the case of unknown or blind cell add. In FR1, the search will take 1SMTC duration, whereas in the FR2 where we need to take beams in account, the search will take (N) SMTC, where N is the number of Rx beams. Note, that since the PSCell is unknown, the UE will assume a minimum default periodicity of 20ms for SMTC.

Proposal 4: In the case of unknown cell, Tsearch can be up to 2*max (20ms, 1 SMTC) for FR1 and (N+1) *max (20ms, 1 SMTC) for FR2.

# Conclusions

Proposal 1: The configuration time for PSCell addition should not include time for CQI reporting. The configuration time should be

Tconfig_PSCell = TRRC_delay + Tprocessing + Tloops + Tsearch + TPSCell_ DU

Where TRRC_delay: is the RRC procedure delay

Tprocessing: is the SW processing time needed by UE.

Tsearch: is the time for PSS/SSS detection, which is 0 for known cell. And [TBD] ms for unknown cell provided that the signal quality is sufficient for successful cell detection on the first attempt

Tloops: time, for AGC and loops for time refinement

TPSCell_DU:  delay uncertainty in acquiring the first available PRACH occasion in the PSCell

Proposal 2: Tprocessing can be up to 20ms.

Proposal 3: The time for AGC and loop refinement can be up to 3 SMTC duration.

Proposal 4: In the case of unknown cell, Tsearch can be up to 1*max (20ms, SMTC) for FR1 and N *max (20ms, SMTC) for FR2.

# References

[1] R4-1803505, Way forward on PSCell addition delay and SCell activation delay, RAN4#86, Athens, Greece