3GPP TSG-RAN4 #87 	R4-1806218

# Busan, KR, May 21 – 25, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0004 | rev | 0 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MPR applicability for NR intra-slot multiple transmissions type FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-02-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Due to multiple possible intra-slot transmissions type TDM multiplexed the MPR to be aplied in the 6.2.4 subclause for Pcmax needs clarification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a paragraph that clarifies the MPR rule to be applied for the entire slot transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPR value to be applied per slot is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ---------------------------------------Start of Change 1-------------------------------

### UE maximum output power reduction

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE Power Class 3, the allowed maximum power reduction (MPR) is defined in Table 6.2.2-1.

Table 6.2.2-1 Maximum power reduction (MPR) for power class [2] and 3

| Modulation | MPR (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 0.5 | 0 |
| DFT-s-OFDM QPSK | ≤ 1 | 0 |
| DFT-s-OFDM 16 QAM | ≤ 2 | ≤ 1 |
| DFT-s-OFDM 64 QAM | ≤ 2.5 |  |
| DFT-s-OFDM 256 QAM | 4.5 |  |
| CP-OFDM QPSK | ≤ 3 | ≤ 1.5 |
| CP-OFDM 16 QAM | ≤ 3 | ≤ 2 |
| CP-OFDM 64 QAM | ≤ 3.5 |  |
| CP-OFDM 256 QAM | ≤ 6.5 |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

LCRB,Max is the maximum number of RB for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1. RBStart,Low = LCRB/2 rounded down to next integer with floor at 1

RBStart,High = LCRB,Max – RBStart,Low – LCRB

Where Inner RB allocation range is specified as follows: Inner RB allocation are LCRB/2 away from each edge of the maximum RB allocation for all LCRB ≤ LCRB,Max/2 rounded up to the next integer.

RBStart,Inner : valid RBStart values for Inner RB allocations

For LCRB ≤ LCRB,Max/2 rounded up to the next integer, RBStart,Low ≤ RBStart,Inner ≤ RBStart,High

Where Outer RB allocation range is all allocations which are not Inner RB allocation.

When multiple transmissions are scheduled and time multiplexed in a slot, the MPR is evaluated for each type and the maximum value over the transmissions is then applicable for the entire slot.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.5 apply.

### ---------------------------------------End of Change 1-------------------------------